# An Introduction to Mechatronics: LabVIEW

Charles Pezeshki and Dale McDonald 
School of Mechanical and Materials Engineering
Washington State University

## Instructor’s Portion

### Summary

Any introduction to mechatronics must introduce the basic tools needed to effectively interact with electrical/mechanical/computer/control systems.  The underlying logical thought process, which is necessary for successful implementation of a useful mechatronic system should be introduced and reinforced.

This laboratory exercise introduces the student to the LabVIEW environment with particular focus upon loops and other program structures.  Parts 1 and 2 ask the student to create a simple VI that controls the emission of light from a light emitting diode.  The tasks are accomplished using case structures, sequence structures, while loops, and for loops.  The relative advantages/disadvantages of each structure are explored in the context of the given objective.

### Uses

This laboratory has applications in virtually every discipline where the use of LabVIEW is applicable, including, mechanical engineering and electrical engineering.

### Equipment List

- Computer running Windows, Macintosh, Linux, Sun, or HP-UX

- National Instruments LabVIEW 6.1 (PN: 776670-03)

- National Instruments PCI-6024E Data Acquisition Board (PN: 777743-01)

- National Instruments Sheilded Cable (PN: 182419-02)

- National Instruments BNC-2110 I/O connector block (PN: 777643-01)

- Light Emitting Diode

### Setup

#### Before the Day of the Lab

- Install LabVIEW (see the LabVIEW Release Notes for your version of LabVIEW).

- Install your PCI-6024E board (see the 6023E/6024E/6025E User Manual).

- Cable the PCI-6024E data acquisition board to the BNC-2110 I/O connector block.

- Configure the PCI-6024E board (See the NI-DAQ Release Notes for your version of NI-DAQ).

- Solder the LED terminals to two pieces of wire (22 gauge for example).

- Place the negative wire/terminal into digital ground on the connector block.

- Place the positive wire/terminal into digital channel zero on the connector block.

### References

- Beyon, Jeffrey Y. (2001), LabVIEW Programming, Data Acquisition, and Analysis, Prentice-Hall Inc., Upper Saddle River, NJ.

## Student’s Portion

### Introduction

The objective of this laboratory is to introduce and explore the LabVIEW environment.  Your goal is to create a simple VI that manipulates the LED attached to the connector box.  However, your primary focus should be to understand how to apply LabVIEW, a graphical programming language, in a mechatronic setting.

You will first build a VI that controls the LED using a case structure and a while loop in conjunction with the sequence structure.  Secondly, you will build a VI that accomplishes a similar objective, this time using a for loop in place of the while loop.

Objective

- Gain familiarity with the LabVIEW environment

- Explore LabVIEW program structures.  Understand the advantages/disadvantages of each for a given application.

### Theory

This laboratory is strictly an introduction to LabVIEW.

### Pre-Lab Preparation

- Consult LabVIEW Programming, Data Acquisition, and Analysis by Jeffrey Beyon.  Pay particular attention to the chapter on LabVIEW program structures (chapter 4).

### Workstation Details

- Computer with National Instruments LabVIEW software.

- National Instruments DAQ board (inside computer).

- National Instruments Shielded cable

- National Instruments Connector box

- Light emitting diode

### Lab Procedure

- Launch the Measurement and Automation Explorer

- Create a DIO virtual channel (consult the MAX help files if necessary).

- Launch LabVIEW

- Part 1

- Place a Boolean control in the front panel.  This control will be the power button for your VI.

- Place a case structure in the block diagram.  Wire the Boolean control to the selector terminal of the case structure.

- Place a while loop inside the case structure.

- Within the while loop, place two sequence frames.

- Within the two sequence frames place the Write to Digital Port VI and DAQ Channel Name Constant.  These are located in the Functions>>Data Acquisition>>Digital I/O and Functions>>Data Acquisition subpalettes respectively.

- In sequence frame zero, wire the DAQ Channel Name Constant to the appropriate terminal on the Write to Digital Port VI.  Select the wiring tool and right click above the “pattern” terminal on the Write to Digital Port VI. Select create constant and place a value of 1 inside this constant.  Right click above the “device” terminal on the Write to Digital Port VI. Select create constant and place a value of 1 inside this constant (verify through MAX that your device is device 1).  Finally, in the Functions>>Time and Dialog subpalette, select the Wait VI and wire a constant to its input terminal.

- Repeat this process in sequence frame 1, changing only the constant value of 1 connected to the pattern terminal to a value of 0.

- Access Functions>>comparison and place the Less? VI within the while loop, outside the sequence frames.  Wire the iteration terminal to the top-left terminal of the Less? VI. Right click on the bottom-left terminal of the Less? VI and select create control.  Wire the output of the Less? VI to the conditional terminal.  Place a digital indicator in the block diagram and monitor the current iteration index from the iteration terminal.

- Return to the Front panel.  Label the Boolean control “power”, label the digital control “Desired Pulses”, and label the digital indicator “Pulses Observed”.

- Part 2

- Place a Boolean control in the front panel.  This control will be the power button for your VI.

- Place a second Boolean control in the front panel.  This control will allow the user to toggle between a set and a variable number of pulses to the L.E.D.

- Place a case structure in the block diagram.  Wire the Boolean power button control to the selector terminal of the case structure.

- Place a second case structure inside the first.  Wire the Boolean pulse toggle switch control to the selector terminal of the inner case structure.

- Place for loops inside the ‘true’ and ‘false’ frames of the inner case structure.  Right click on the count terminal in the ‘false’ frame.  Select create constant and give this constant a value of one.  Right click on the count terminal in the ‘true’ frame, and select create control.

- Return to the front panel.  Label the power button and the pulse toggle switch appropriately.  Be sure that your label for the toggle switch corresponds to the correct case frame within the diagram window.  Label the digital control “Desired number of pulses”.

### Lab Report

Execute the VIs that you created in parts one and two.

Using the VI you created in part one and with the power “on” select 5 iterations.  Count the pulses from the L.E.D.  You will observe one more pulse than you (the user) desired.  Why is this?  How could you fix this problem so that the user can input a desired number of pulses and observe that the program creates that number of pulses (in the block diagram, add one to the iteration)?  Implement this change to your program.  Did this fix the problem?  Turn the “power” to your instrument off.  Run the Program.  Do you observe any pulses?  Why is this and how is it important?

Using the VI you created in part two and with the power “on” select 5 iterations.  Execute your program.  Did you observe the correct number of pulses?  Why is this and why is the for loop structure preferable to the while loop structure for this exercise?