# An Introduction to Mechatronics:

# Stepping Motor Control

Charles Pezeshki and Dale McDonald 
School of Mechanical and Materials Engineering
Washington State University

## Instructor’s Portion

### Summary

Once the students have been introduced to the LabVIEW environment they are prepared to create a basic mechatronic system.  Critical thinking skills must be developed, and to that end the students will be assimilating hardware (mechanical system), PC/software (computer system), and electrical components for successful completion of the laboratory.

This laboratory exercise may be the first opportunity for the students to create a mechatronic system.  The students will use LabVIEW to create a VI that controls the position, velocity, and acceleration of a standard stepping motor.  This exercise is applicable to many disciplines as an introduction to mechatronics that allows the student the opportunity to control a widely used piece of industrial equipment.

### Uses

This laboratory has applications in virtually every engineering discipline, particularly in manufacturing engineering and mechanical engineering.

### Equipment List

- Computer running Windows, Macintosh, Linux, Sun, or HP-UX

- National Instruments LabVIEW 6.1 (PN: 776670-03)

- Parker Hannifin CX57-** Stepping Motor

- Parker Hannifin CX Indexer/Drive

### Setup

#### Before the Day of the Lab

- Install LabVIEW (see the LabVIEW Release Notes for your version of LabVIEW).

- Install necessary stepping motor drive.  We used the Parker Hannifin CX Indexer/Drive with stepping motor CX57-**.

- Verify operational status of your stepping motor drive system (refer to reference below)

### References

- Parker Hannifin Corp. Compumotor CX Indexer/Drive User Guide

- Beyon, Jeffrey Y. (2001), LabVIEW Programming, Data Acquisition, and Analysis, Prentice-Hall Inc., Upper Saddle River, NJ.

- Acarnley, Paul P. (1984), Stepping Motors: a guide to modern theory and practice, Peter Peregrinus Ltd., London, U.K.

## Student’s Portion

### Introduction

The objective of this laboratory is to combine mechanical, electrical and computer systems to build a simple mechatronic system.  You will create a virtual instrument that controls a stepping motor, which is a commonly used piece of industrial hardware.

Using LabVIEW, with a stepping motor and appropriate drives, you will control the position, velocity, and acceleration of a stepping motor.  Your objective is to explore the limits (steps/rev, speed, acceleration) of the stepping motor and document what you observe.  Of equal importance in this laboratory is the understanding of how you may use LabVIEW to issue commands (string and numeric) that control the behavior of the stepping motor.

For a long program, you may expect that a text file has been created containing the necessary commands to accomplish your particular objective.  The final portion of this laboratory involves the creation of a text file containing appropriate commands and the creation of a LabVIEW virtual instrument that reads and executes these commands.

Use the help files provided by LabVIEW and pay attention to the subpalette names within the controls and functions palettes.  You will be expected to locate the VIs needed for this exercise.

Objective

- Create and interact with a mechatronic system.

- Design a VI that is useful/applicable in a wide variety of industrial settings

- Further understand the capabilities of LabVIEW for this type of system.

### Theory

Stepping motors typically come in two varieties, variable reluctance and permanent magnet.  In the variable reluctance type, precise positioning of the shaft is achieved by energizing the rotor windings in the desired sequence, causing the poles of the stator to align accordingly.  In the permanent magnet type, applying current to the windings causes one set poles of the rotor to become a north pole and the other a south pole.

The number and size (degrees/radians) of each step depends upon the number of poles and teeth of that particular stepping motor.  A process called micro-stepping however can produce very fine steps.  Refer to Stepping Motors: a guide to modern theory and practice by Paul Acarnley for further discussion of the principles of operation of stepping motors.

### Pre-Lab Preparation

- Review the reference material stated previously to further understand how a stepping motor works.

- Review the user guide for you particular motor/drive system.

- Consult LabVIEW Programming, Data Acquisition, and Analysis by Jeffrey Beyon and review appropriate commands.

### Workstation Details

- Computer with National Instruments LabVIEW software.

- CX Indexer/Drive.

- CX57-** Stepping motor.

### Lab Procedure

Part 1:  Assembling the Components

- Place three string controls in the front panel of your new VI.  Label these controls as “Distance (microsteps)”, “Velocity (revs/sec)”, and “Acceleration (revs/sec2)”.

- Place a Boolean control in the front panel.  Label this control “Reverse Direction”.  We will use this control shortly.

- Place a numeric control in the front panel.  Label this control “Command Iterations”.  We will use this control shortly.

- Place a for loop inside the block diagram.  Wire the “Command Iterations” control to the count terminal of the for loop.

- Place a Concatenate Strings function within the for loop.  Adjust the size of this icon to allow for two input strings.  Place a string constant in the block diagram near the top input of the Concatenate Strings function.  Place a “V” in this string constant and wire it to the top input terminal Concatenate Strings.  Wire your velocity string control to the bottom input terminal of Concatenate Strings.

- Repeat this process for the acceleration and distance inputs to the stepper motor.  Replace the “V” with “A” or “D” as appropriate.

- Place a string constant in the for loop.  Inside this constant enter the command LD3, which disables the clockwise and counterclockwise end of travel limits.

- Place another string constant in the for loop.  Inside this constant enter the command MN, which sets the positioning mode preset values.

- Place another string constant in the for loop.  Inside this constant enter the command MPI, which sets the positioning mode to incremental.

- Place a case structure inside the for loop.  Within the “true” frame place a string constant and place an “H” within  the string constant.  Leave the “false” frame empty.  The “H” will change the direction of motion of the stepper motor.

- Place a another string constant in the for loop.  Place a “G” within  the string constant.  The “G” command is the execute or “GO” command.

- Place  two end of line constants within the for loop.

- Place a Serial Port Write VI within the for loop.

- Finally, place a Concatenate Strings function within the diagram window.  Adjust its size so that it may accept sixteen input strings.

Part 2:  Wiring the Components

Wire the components as shown in the attached diagram window.  The CX drive expects commands followed by a carriage return or an end of line character.  Note how this step is accomplished using LabVIEW.

Be sure to wire the commands in the order shown.  The stepping motor expects these commands in a certain order.  Refer to the “CX Indexer/Drive User Guide” for more information on specific commands and sequences.

Part 3:  Reading/Executing from a text file.

1. Create a text file containing similar commands to those used in Parts 1 and 2 of this laboratory.  Save this file (with one command on each line) as a text file.

2. In the front panel of your virtual instrument, place a file path control that allows you to select the text file containing your commands.

3. In the block diagram, place a case structure over all existing elements.

4. In the “false” case, place a Read Characters From File VI.  Wire the path control created in step 2 to the file path terminal of the Read Characters From File VI.

5. Place a Serial Port Write VI in the “false” case, and wire the character string terminal of the Read Characters From File VI to the string to write terminal of the Serial Port Write VI.

6. Place a Boolean control in the front panel.  Use this control to toggle between creating code via LabVIEW and reading a preexisting file that contains the desired code.

### Lab Report

Include a section that details the exact steps that you carried out to complete this laboratory.  Did you have any problems?  Did you use any LabVIEW functions that were new to you?  To verify that your code is correct and in the proper format, try outputting to a text file and double checking the results.

Investigate the limits of the stepping motor.  What was the maximum velocity and acceleration (i.e. did you notice any “chatter” or “buzzing”).  Feel free to customize your VI to add more features/commands.