# An Introduction to Mechatronics:

# Servo Motor Control

Charles Pezeshki and Dale McDonald 
School of Mechanical and Materials Engineering
Washington State University

## Instructor’s Portion

### Summary

Once the students have been introduced to the LabVIEW environment they are prepared to create a basic mechatronic system.  Critical thinking skills must be developed, and to that end the students will be assimilating hardware (mechanical system), PC/software (computer system), and electrical components for successful completion of the laboratory.

This laboratory exercise may be the first opportunity for the students to create a mechatronic system.  The students will use LabVIEW to create a VI that controls the motion of a simple servo motor.  This exercise is applicable to many disciplines as an introduction to mechatronics that allows the student the opportunity to control a widely used piece of industrial equipment.

### Uses

This laboratory has applications in virtually every engineering discipline, including manufacturing engineering and mechanical engineering.

### Equipment List

- Computer running Windows, Macintosh, Linux, Sun, or HP-UX

- National Instruments LabVIEW 6.1 (PN: 776670-03)

- National Instruments PCI-6024E Data Acquisition Board (PN: 777743-01)

- National Instruments Sheilded Cable (PN: 182419-02)

- National Instruments BNC-2110 I/O connector block (PN: 777643-01)

- Servo motor ( 5V Tower Hobbies STD TS-53)

### Setup

#### Before the Day of the Lab

- Install LabVIEW (see the LabVIEW Release Notes for your version of LabVIEW).

- Verify operational status of your servo motor.

### References

- Beyon, Jeffrey Y. (2001), LabVIEW Programming, Data Acquisition, and Analysis, Prentice-Hall Inc., Upper Saddle River, NJ.

- Miller, Richard W., (1977), Servomechanisms: devices and fundamentals, Reston Publishing Company, Reston, Virginia.

## Student’s Portion

### Introduction

The objective of this laboratory is to combine mechanical, electrical and computer systems to build a simple mechatronic system.  You will create a virtual instrument that controls a servo motor, which is a commonly used piece of industrial hardware.

Using LabVIEW you will control the motion of a servo motor.  Your objective is to explore the limits of the servo motor and document what you observe.  Of equal importance in this laboratory is the understanding of how you may use LabVIEW to control the servo, and observe that many ways may exist to accomplish the above task (dependent upon hardware and monetary investment).

Use the help files provided by LabVIEW and pay attention to the subpalette names within the controls and functions palettes.  You will be expected to locate the VIs needed for this exercise.

Objective

- Create and interact with a mechatronic system.

- Design a VI that is useful/applicable in a wide variety of industrial settings

- Further understand the capabilities of LabVIEW for this type of system.

### Theory

A servo motor generally consists of a motor, control circuitry, gears, an output shaft, and the case.  Of particular interest is the potentiometer, a variable resistor, which is used by the control circuitry to position the output shaft in the proper position.  Three wires are used to power and control the servo – a (usually) white control wire, a red wire for power (5V), and a black wire for ground.

The position and movement of the output shaft is dictated by a process called pulse coded modulation.  Basically pulse coded modulation refers to the process of sending a pulses of a given length that correspond to angular positions of the output shaft.  Refer to Servomechanisms: devices and fundamentals by Richard Miller and the documentation of your servo motor for further details.

### Pre-Lab Preparation

- Review the reference material state previously to further understand how a servo motor phsically works.

- Review the user guide for your particular motor.

- Consult the LabVIEW Programming, Data Acquisition, and Analysis by Jeffrey Beyon and review appropriate commands.

### Workstation Details

- Computer with National Instruments LabVIEW software.

- National Instruments DAQ board (inside computer).

- National Instruments Shielded cable

- National Instruments Connector box

- Servo motor

### Lab Procedure

Part 1:  Assembling the Components

- Place a Boolean control in the front panel, to act as a power switch for your servo motor VI.

- Place a numeric control (a dial perhaps) in the front panel of your virtual instrument.  This dial will be used to communicate with the servo motor (pulse coded modulation).

- Place a waveform chart in the front panel.  This chart will be used to observe the pulses that are being sent to the servo motor.

- In the block diagram place a while loop (all elements used in the block diagram are placed within this loop).

- Place a Square Wave VI within the while loop.

- Right click over the frequency and amplitude terminals on the Square Wave VI and select “create constant” (you may also choose to do the same with the sampling info).  The frequency is dependent upon the motor; we used a constant of 200 Hz and set the amplitude to 1.  Wire the frequency, amplitude, and if necessary the sampling information to the appropriate terminals on the Square Wave VI.  Also wire the numeric control created in step 2 to the duty cycle terminal of the Square Wave VI.

- Place a Write to Digital Port VI in the block diagram.  Place a DAQ channel name constant in the block diagram as well.  Wire the signal out terminal on the Square Wave VI to the pattern terminal on the Write to Digital Port VI.

- In the block diagram, wire the square waveform indicator to the line connecting the Square Wave VI and Write to Digital Port VI.  Wire the power (Boolean) control to the conditional terminal of the while loop.

- Wire the DAQ channel name constant to the digital channel terminal on the Write to Digital Port VI.  Verify that the defaults used by the Write to Digital Port VI are correct (device, port width, etc.).

- Execute the program and document the motion of the servo motor.  Experiment with the frequency and the duty cycle and observe how this effects the movement and positioning of the output shaft of the servo motor.

### Lab Report

Include a section that details the exact steps that you carried out to complete this laboratory.  Did you have any problems?  Did you use any LabVIEW functions that were new to you?  To verify that your code is correct and in the proper format, try outputting to a text file and double checking the results.

How does a change in frequency or a change in the duty cycle relate to pulse coded modulation.  Do you observe chatter?  Does the output shaft return to position after it is manually displaced?

This illustrative method of servo motor control may not produce the easiest or most reliable servo control.  Explore the LabVIEW help to determine how a more reliable method of pulse coded modulation may be achieved (pulse train?). What additional hardware or other modifications would you need to use these other methods (use counter etc.)?