# An Introduction to Mechatronics:

# Proximity Warning System

Charles Pezeshki and Dale McDonald 
School of Mechanical and Materials Engineering
Washington State University

## Instructor’s Portion

### Summary

Once the students have been introduced to the LabVIEW environment they are prepared to create a basic mechatronic system.  Critical thinking skills must be developed, and to that end the students will be assimilating hardware (mechanical system), PC/software (computer system), and electrical (optical system) components for successful completion of the laboratory.

This laboratory exercise is the first opportunity for the students to create a mechatronic system.  The students will use a photodiode as the backbone of a VI that acts as a proximity warning system.  Each student must design a VI that interacts with the photodiode, an L.E.D., and a speaker to sense the position of an object and output visual alarm and a warning message.  This exercise is applicable to many disciplines, particularly in manufacturing where photodiodes are inexpensive yet effective tools for sensing position, speed, etc.

### Uses

This laboratory has applications in virtually every engineering discipline, particularly in manufacturing engineering, mechanical engineering, and electrical engineering.

### Equipment List

- Computer running Windows, Macintosh, Linux, Sun, or HP-UX

- National Instruments LabVIEW 6.1 (PN: 776670-03)

- National Instruments PCI-6024E Data Acquisition Board (PN: 777743-01)

- National Instruments Sheilded Cable (PN: 182419-02)

- BNC-2110 I/O connector block (PN: 777643-01)

- Photodiode

- Light Emitting Diode

- Microphone

- Speaker

### Setup

#### Before the Day of the Lab

- Install LabVIEW (see the LabVIEW Release Notes for your version of LabVIEW).

- Install your PCI-6024E board (see the 6023E/6024E/6025E User Manual).

- Cable the PCI-6024E data acquisition board to the BNC-2110 I/O connector block.

- Configure the PCI-6024E board (See the NI-DAQ Release Notes for your version of NI-DAQ).

- Solder the LED terminals to two pieces of wire (22 gauge for example).

- Place the negative wire/terminal into digital ground on the connector block.

- Place the positive wire/terminal into digital channel zero on the connector block.

- Connect the photodiode to an appropriate analog input channel on the BNC-2110 I/O connector block.

### References

- Beyon, Jeffrey Y. (2001), LabVIEW Programming, Data Acquisition, and Analysis, Prentice-Hall Inc., Upper Saddle River, NJ.

- Seippel, Robert G. (1981), Optoelectronics, Reston Publishing Company Inc., Reston, Virginia.

## Student’s Portion

### Introduction

The objective of this laboratory is to combine software, hardware, and the computer system to build a simple mechatronic system that will act as an industrial proximity warning system.  The primary hardware you will use is a photodiode, widely used in industry for the ability to measure speed and position in an accurate and inexpensive manner.

Using LabVIEW, a photodiode, a microphone, speakers, and an LED, design a proximity warning system for use in any industrial application where the possibility of bodily harm from machinery may exist.  This warning system must have, at a minimum, the ability to determine when an object (fingers, hand, etc.) has crossed into an unsafe area.  The proximity warning system must sound an alarm and set off a visual alarm (LED) for as long as the foreign object remains in the region of danger.  When the object has cleared the alarms must shut off, and the program should run normally.

Objective

- Create and interact with a mechatronic system.

- Design a VI that is useful/applicable in a wide variety of industrial settings

- Further understand the capabilities of LabVIEW for this type of system.

### Theory

A photodiode will be used to detect the presence of an object in an “unsafe” area, where the possibility of damage/danger to humans and/or equipment is a possibility.  Simply speaking, a photodiode (solar cell) is a semiconductor device that absorbs photons of light creating a current flow and a difference of potential between the two terminals connected to the load.  This photovoltaic effect will be used to our advantage to activate a warning and shut down any dangerous processes.  For further discussion of the photovoltaic effect and photodiodes, please refer to Optoelectronics by Robert Seippel.

### Pre-Lab Preparation

- Review the reference listed previously to further understand the physical processes taking place.

- Consult LabVIEW Programming, Data Acquisition, and Analysis by Jeffrey Beyon and review appropriate commands.

### Workstation Details

- Computer with National Instruments LabVIEW software.

- National Instruments DAQ board (inside computer).

- National Instruments Shielded cable

- National Instruments Connector box

- Photodiode (solar cell)

- Light emitting diode

- Microphone

- Speaker

### Lab Procedure

Part 1

- Launch the Measurement and Automation Explorer.

- Create a DIO virtual channel (consult the MAX help files if necessary) for use with the light emitting diode.

- Create an analog output virtual channel (consult the MAX help files if necessary) for use with a second light emitting diode.

- Create an analog input channel for use with the photodiode.

- Using the sound recorder, record a brief warning message.  Save in .wav format.

- Launch LabVIEW.

- Part 2

- Place a Boolean control in the front panel.  This control will be the power button for your VI.

- Place a while loop in the block diagram.  Attach the Boolean control from your power button to the conditional terminal of the while loop.

- Place the AI Sample Channel VI in the block diagram, and specify the analog input channel that your photodiode is connected to.  Wire the sample terminal of this VI to the top-left terminal of a Greater than function.  Wire a numeric control to the bottom-left terminal of the Greater Than function (the photodiode produces a specific current/voltage in ambient light, if this current/voltage falls below this user defined value the VI can output an appropriate alarm).

- Place a case structure inside the while loop, to the right of the Greater Than function.  Wire the output of the Greater Than function to the selector terminal of the case structure.

- If the above condition is true, the “unsafe area” has not been breached and our process may continue.

- We now add that process that executes when there are no objects present in the “unsafe area”.  Our process is the generation of a sine wave that outputs to the analog connected LED.

- Place a Sine Wave VI within the “true” frame of the case structure. Place digital controls for the frequency and amplitude of this sine wave in the front panel.

- Locate the AO Update Channel VI and place it in the “true” frame of the case structure as well.  Wire the “signal out” terminal of the Sine Wave VI to the “input” terminal of the AO Update Channel VI.  Specify the appropriate analog output channel.

- Now specify the warnings (message and light) that are executed if the above condition is false meaning something has passed into the “unsafe area”.

- Place a Write to Digital Port VI inside the false frame of the case structure.  Specify a pattern of 1 and the correct channel for your digital LED.

- In the front panel place a file path control.

- Locate the Sound Play Wave File VI and place it in the false frame of the case structure.

- Connect the file path control to the wave file path terminal on the Sound Play Wave File VI.

- Test and troubleshoot your VI.

### Lab Report

Include a section that details the exact steps that you carried out to complete this laboratory.  Did you have any problems?  Did you use any LabVIEW functions that were new to you?

Investigate the uses of photodiodes/solar cells in industry.  Discuss the photovoltaic effect and how it applied in this exercise.  Investigate other applications/disciplines where the photovoltaic effect plays a major role.