# An Introduction to Mechatronics:

# Fourier Series and the FFT

Charles Pezeshki and Dale McDonald 
School of Mechanical and Materials Engineering
Washington State University

## Instructor’s Portion

### Summary

Once the students have been introduced to the LabVIEW environment they are prepared to use LabVIEW to investigate common engineering problems and undertake the associated analysis.  This laboratory provides an introduction to the Fourier series and the Fast Fourier Transform.

LabVIEW will be used to illustrate the creation of a Fourier series approximation to a square wave.  The Fast Fourier Transform will be used to visualize the spectral content of the square wave approximation, and the connection between the fundamental frequency/harmonics to the Fourier series coefficients will be exhibited.

### Uses

This laboratory has applications in virtually every mathematics and engineering discipline, including electrical engineering and mechanical engineering.

### Equipment List

- Computer running Windows, Macintosh, Linux, Sun, or HP-UX

- National Instruments LabVIEW 6.1 (PN: 776670-03)

### Setup

#### Before the Day of the Lab

- Install LabVIEW (see the LabVIEW Release Notes for your version of LabVIEW).

### References

- Beyon, Jeffrey Y. (2001), LabVIEW Programming, Data Acquisition, and Analysis, Prentice-Hall Inc., Upper Saddle River, NJ.

- Wheeler, Anthony J. and Ganji, Ahmad R. (1996), Introduction to Engineering Experimentation, Prentice-Hall Inc. Englewood Cliffs, NJ.

## Student’s Portion

### Introduction

The objective of this laboratory is to understand the underlying principles of the Fourier series approximation and the Fast Fourier Transform.  A certain amount of confusion may exist upon a student’s first introduction to these concepts and this laboratory is meant to clearly explain the two concepts.  Further knowledge of LabVIEW will be gained as we take advantage of built in Fast Fourier Transform VI’s.

Use the help files provided by LabVIEW and pay attention to the subpalette names within the controls and functions palettes.  You will be expected to locate the VIs needed for this exercise.

Objective

- Understand the concepts of the Fourier series approximation.

- Understand the concept of a Fast Fourier Transform.

- Design a VI assembles the appropriate Fourier coefficients and create a four term approximation to a square wave of period pi.

- Use a Fast Fourier Transform VI to observe the spectral content of the signal and relate this to the Fourier coefficients.

### Theory

A Fourier series consists of the sum of a constant and a series of sine and cosine terms.  Any periodic function can be represented by this series and in this exercise we are able to approximate a function whose derivative does not exist at certain points within the domain with a function that is differentiable (of all orders) throughout the domain.

The Fast Fourier Transform (a generalization of the Fourier series) is an economical computation of the Discrete Fourier Transform.  The Fast Fourier Transform is used to decompose a time-based signal into the frequency domain.  We then can observe the spectral content that dominates the signal.

For a thorough discussion of the Fourier series and the Fast Fourier Transform, please consult Introduction to Engineering Experimentation by Anthony Wheeler and Ahmad Ganji.

### Pre-Lab Preparation

- Review the reference material stated previously to understand the underlying theory of the fourier series and the Fast Fourier Transform.

- Consult the LabVIEW Programming, Data Acquisition, and Analysis by Jeffrey Beyon and review appropriate commands.

### Workstation Details

- Computer with National Instruments LabVIEW software.

### Lab Procedure

Part 1:  Fourier coefficients of a square wave T = 1/pi

- We chose to approximate a square wave with frequency pi and amplitude 1 (of course any periodic function could be used).

- The previously mentioned reference was consulted and the first four terms of the Fourier series was calculated (the coefficients are given in this exercise, but this calculation may be assigned as part of the laboratory exercise.

- Coefficients –

  - A0 = 0 (no offset)

  - A1 = 1,        0 = pi

  - A2 = 1/3,   30  9.4250

  - A3 = 1/5,   50  15.7080

  - A4 = 1/7,   70  21.9911

Part 2:  Assembling the Fourier series

- Place a Boolean control in the front panel to act as a power button for your VI.

- Place eight digital controls in the front panel (four pairs).  Within each pair, one control corresponds to frequency of a sine wave and the other corresponds to an amplitude (the An coefficients) of a sine wave.

- Place a waveform chart in the front panel and label it as “Four term Fourier series approximation to a square wave”.

- Place a waveform graph in the front panel.  Label this graph as “FFT of the square wave approximation”.

- In the block diagram, place four Sine Waveform VIs.  To each VI connect one frequency control and one amplitude control to the appropriate terminal.

- Place three Add Waveform VIs in the block diagram.  Wire the outputs of the first two sine waveforms into one, wire the last two sine waveforms into a second, and add the resultant together using the third.

- Place a Magnitude-Phase FFT VI in the block diagram.  Wire the combined sine waveforms into the time signal terminal on the Magnitude-Phase FFT VI.  Wire the magnitude terminal of the Magnitude-Phase FFT VI to the waveform graph.

- Wire the time signal from the final Add Waveform VI to the waveform chart.

- Place a Wait VI in the block diagram, and select create constant.  Place a value of 100 in that constant (100 ms).

- Place a while loop in the block diagram such that all VIs and other elements are within this while loop.  Wire the power control to the conditional terminal of the while loop.

### Lab Report

Include a section that details the exact steps that you carried out to complete this laboratory.  Did you have any problems?  Did you use any LabVIEW functions that were new to you?

Adjust the x and y scales on the waveform chart (remember the frequency of the square wave is pi).  Does the approximation resemble a square wave?  What happens if you remove terms of the series?  What do you think will happen if you add terms to the series?  Consult any calculus or engineering mathematics text and state the definition of the fundamental frequency.  What are first/second/third/etc. harmonics?

Adjust the y scale on the waveform graph so that the mapping is logarithmic and the limits are .01 – 1.0.  What are the primary spectral components of your time based signal?  How does the FFT relate to the Fourier series (coefficients/harmonics)?