# Natural Circulation Analysis

David A. Botelho, José L. H. Faccini, and Milton Soares
Reactors Division
Nuclear Engineering Institute – IEN/CNEN

Av. Brigadeiro Trompowsky, S/N CP 68550 CEP 21945-970, Rio de Janeiro, RJ, Brazil

## Instructor’s Portion

### Summary

Natural convection is a mechanism of heat transport accomplished by mass transport driven by the gravity field.

The students operate a small-scale thermal-hydraulic natural circulation experiment used to validate analytic design methods and compare experimental data to analytic results, thus evaluating the steady performance of a similar full-scale system.  The temperature, mass flow rate, and void fraction signals from the sensors are collected using LabVIEW and a National data acquisition board.

Similar systems are employed for decay heat removal of advanced nuclear reactors [5].  Being passive mechanisms, they provide additional safety in the operation of these important electricity-generating stations.

There are many examples of natural convection in nature, as the rising of the hot gas of a flame or the smoke exhaust movement from a furnace.  These movements result from the lower density of the hot fluid compared to the density of the surrounding air.  Another example is a water heating system for houses and industry buildings.  In them a certain volume of the water is placed into cells surrounded by a solar heat absorbing material, located at a lower level.  The hotter fluid is allowed to flow trough pipes into a storage drum located at a higher level.  The driving force is the density difference of the flowing down colder water from the storage drum and the rising up hotter water that leaves the heat absorbing cells.

### Uses

This laboratory has application in nuclear engineering but can also be used to help design solar heating system for homes and industrial buildings, a branch area of mechanical engineering.

The main disciplines are fluid flow and heat transfer in fluids (liquid water and water/steam mixtures).

Some possible variations on this experiment.

- The introductory experiment deals with liquid water single-phase flow, where only fluid temperature measurements, employing thermocouples, are executed to infer the total hydraulic resistance of the circuit and the natural convection mass flow rate.  The experiment can also be completed performing the mass flow rate measurements and comparing it to the analytically calculated mass flow.

- Two-phase flow experiments can also be performed in which the steam fraction (void fraction) measurements are made and used to infer the total hydraulic resistance and the mass flow rate which is compared to the mass flow rate measurements.  The steam bubbles velocity measurements and the void fraction knowledge are also used to calculate the steam quality and important two-phase similarity numbers defined in the references [6] and [7].

### Equipment List

- Pentium PC Computer running Windows NT/98/95 with floppy disk drive.

- PCI 6033E DAQ Board, p/n 777516-01.

- PC-AO-2DC, Analog and Current Output, p/n 777092-01

- Type SH100100 Shielded Cable Assembly, p/n 182853-02.

- Type NB1 Cable, p/n 180524-20.

- LabVIEW Full Development System for Windows NT/98/95, p/n. 77667-03.

- Digital Thermometer for thermocouple type K (ECB).

- Websites

- National Instruments - www.ni.com

- ECB - www.ecbequip.com.br

### Setup

A scaled drawing of the natural convection circuit (CCN) is shown in the Fig. 1.  This system comprises the heater vessel (in red color) including the upper and lower heater plena, the core barrel, the downcomer, and heating rod bundle, and the hot and the cold piping connecting the heater vessel to a heat exchanger.  The heater section has 52 electrically heated rods bundle with external diameter of 9.5x10-3m.  The active length is 0.366m.  The circuit operates near atmospheric pressure.

A heat exchanger (cooler, in blue color) with 7 vertical stainless steel straight tubes with 0.6m height and 0.0314m internal diameter, is located 0.944m above the active heating section.  The primary fluid flows inside the tubes.  The flow area of the heat exchanger tubes is about 0.935 times the flow area of the heater bundle channels.  A vertical hot leg pipe (in brown color) connects the upper plenum of the heater vessel to the entrance (upper) plenum of the cooler, and a cold leg pipe (in blue color) links the outlet (lower) plenum of the cooler to the inlet downcomer of the heater vessel.  A vertical pipe (in green color) connects the cold leg pipe of the circuit to the expansion drum (also in green color), which may be open to the exterior atmosphere.  The expansion drum is about 2.17m from the lower part of the heater section.  Thus, the pressure at half-height of the heater is about 0.12 bar.  The expansion drum is provided to absorb the circuit fluid volume changes.  A safety valve is installed to protect it against over-pressure.

Twelve (12) thermocouples to monitor transient temperatures are placed at convenient locations.  A continuous electric current control and power source feeds the electric resistors of the heater.  An electronic control and data acquisition unit coupled to a Pentium PC, controls, monitors, and store all transient data obtained from a given test.

#### Before the Day of the Lab

- Check the heat exchanger secondary side flowrate and temperature.

- Check the ambient air temperature digital thermometer.

- Turn on / turn off all devices.

- Check the installation/configuration of data acquisition & control system.

- Launch LabVIEW application already installed and perform a sample experiment.

On the Day of the Lab

- Power up the data acquisition & control system: turn on power control unit, temperature data acquisition unit and the computer; wait until the system warm up (± 20 minutes).

- Launch the LabVIEW application.

- Run a previous temperature data acquisition to verify if all thermocouples measurements show same values (initial condition).

Figure1.  The Natural Convection Circuit.

### References

[1] Bird, R.B., Stewart, W.E., and Lightfoot, E.N.  Transport Phenomena, John Wiley & Sons, Inc., New York., 1960.

[2] Kay, J. M., and Nedderman, R. M.,  An Introduction to Fluid Mechanics and Heat Transfer, Cambridge University Press, 1974.

[3] Wallis, G. B., One-dimensional Two-phase Flow, McGraw-Hill, New York, 1969.

[4]  Shieh, A. S., Ramsom, V.H., Krishnamurty, R.,  RELAP5/MOD3 Code Manual, Volume 6: Validation of Numerical Techniques.  NUREG/CR-5535; EGG-2596. USNRC, Washington, DC, 1994.

[5]  Vijuk, R.P. and Bruschi, H.,  AP600 offers a simpler way to greater safety, operability, and maintainability.  Nuclear Engineering International p. 22-28, Nov., 1988.

[6]  Reyes Jr., J.N. and Hochreiter, L.,  Scaling analysis for the OSU AP600 test facility (APEX). Nucl. Eng. Des. 186, p.53-109, 1998.

[7]  Botelho, D.A., Faccini, J.L.H., De Sampaio, P.A.B., and Moreira, M.L., Natural Circulation in a Loop Similar to a Passive Nuclear Reactor Cooling System.  Presented in the Latin American Section of the American Nuclear Society (ANS) Symposium 2002. Rio de Janeiro, June, 17-20, 2002.

## Student’s Portion

### Introduction

- The first experiment deals with the single-phase operation of the circuit.  The proper heater power need to be evaluated and set.  It is also needed to estimate and set the mass flow rate on the secondary side of the heat exchanger.  The students need to be acquainted to the data acquisition system of the experiment and with the locations of the various thermocouples in the experiment.  They will use the plots of the hot leg thermocouple (T4) and of the cold leg thermocouple (T8) the operating power and other geometric and thermodynamic data to calculate the total hydraulic resistance and the natural circulation steady-state mass flow rate of the experiment.  The students will be instructed to also perform the mass flow rate measurements and to compare it to the analytically calculated mass flow.

- More advanced two-phase experiments will be performed in which the steam fraction (void fraction) analytic estimates and measurements will be made to infer the total hydraulic resistance and the mass flow rate which is compared to the mass flow rate measurements, and evaluate other important parameters for the performance of similar prototypic full scale systems, as the phase-change number, the Froude number, and the Drift-Flux number, among others.

### Objective

To learn experimental measurement techniques for temperature, mass flow rate, and void fraction, and the analytic methods employed to represent single and two-phase natural circulation flow.

### Theory

#### Natural circulation one-dimensional flow

Natural convection through a closed loop is usually denominated as natural circulation.  The most common analytical representations of natural circulation employ the fluid flow and the energy transport equations.  There are several very good textbooks that derive them from the general principles of mass, force (or momentum) and total energy conservation.  Two almost complete and excellent are the references [1] and [2].  There are others that treat, specifically, the so called one-dimensional flow as [3].  These are flows through pipes or channels with length much greater than diameter, in which it is developed and maintained, along the axial length, a radial velocity and temperature distribution or profile.  These profiles may be parabolic for laminar flows in cylinders.  For turbulent flows the profiles are almost flat in radial locations away from the wall.  In the developed flows it is usual to represent the fluid velocity, the pressure, the temperature, and the thermal properties by radial average values.  The radial average values may vary with axial position and time.  Here we present the one-dimensional momentum conservation equation derived from the reference [4].

Mass conservation equation:

A∂ ρ∂ t+∂ m∂ s = 0                                                                              (1)

Momentum conservation equation:

∂ m∂ t+ u ∂ m∂ s+ m ∂ u∂ s +A∂ p∂ s +A ρ g cosθ+PhΓw=0                    (2)

Constitutive relation for friction:

Ph Γw= fw2 Dh  u  m = fw2 Dh ρ A  m  m

Conservation of internal energy:

ρ cpA ∂ T∂ t + u ∂ T∂ s −k A∂2 T∂ s2 = Pa qw'' + u Ph Γw                   (3)

Boussinesq approximation for the liquid density:

ρ =ρ0 1−β0 Ts − T0                                                                    (4)

By u, p, ρ, and T, are denoted the liquid velocity, the pressure, the density, and the fluid temperature, respectively.  The transverse flow area is represented by A, s is the line coordinate, and t is the time. The gravity acceleration field isg, θ is the angle of a component axis with the vertical, ρ0 is the reference liquid density, β is the reference liquid volumetric thermal expansion coefficient, and T0 is the reference temperature.  The friction factor is fw, the hydraulic perimeter is Ph, the hydraulic diameter is Dh, and the  mass flow rate is m=ρ u A.  The liquid specific heat at constant pressure is cp and the liquid thermal conductivity is k.  The heated perimeter is Pa and qw'' is the heat flux from a heating wall to the fluid.

It is assumed a steady-state condition (thus ∂ m∂ t=0) a uniform fluid properties in the hotter branch (heater and heat exchanger upper half and hot leg) and in the cold branch (heater and heat exchanger lower half and cold leg).  It is assumed that the flow is incompressible (thus ∂ m∂ s=0).  In the uniform area sections it is also neglected the momentum transport by convection (thus  m ∂ u∂ s =0).  The effect of the area changing locations in the momentum balance is incorporated as the denominated form loss coefficient dissipation, being added to the friction factor.  From the incompressible flow approximation, ∂ m∂ s=0, the mass flow rate is uniform on all circuit cross sections (m=uniform).  From the mass conservation equation it also follows that ∂ ρ∂ t=0, the condition of a steady density with time.  The momentum balance is written as:

A∂ p∂ s +A ρ g cosθ + fDh + Kk δs − sk  m  m 2 ρ A=0

It is employed the usual Einstein summation notation for repeated indices, Kk are local form loss coefficients, and δ(s−sk) is the usual Dirac function representation.

This simplified momentum is integrated from the heater (section 1) to the heat exchanger (section 2) in the hot leg path, and from the heat exchanger (section 2) to the heater (section 1) in the cold leg path, to obtain

ρ g cosθ d s+ fDh+Kk δk(s−sk)  m  m 2 ρ A2d s = 0                                (5)

According to the Einstein summation notation and the Dirac function definition, the last term of the Eq. (5) is

Kk δs − sk  m  m 2 ρ Ads = kKk  m  m 2 ρk Ak

The angle values are  cosθ = 1 in the hot leg and  cosθ = −1 in the cold leg.  The temperatures are  T = T2 in the hot leg and  T = T1 in the cold leg.  It is assumed that the temperature increases linearly with distance from  T = T1 to  T = T2 in the heater (with length  LH) and decreases linearly from  T = T2 to  T = T1 in the cooler (with length  LC).  The vertical distance between the cooler and the heater centers is  LCH.  The densities are thus calculated from this temperature distribution.

From the substitution of these angle values, the density according to the Eq. (4) and the given temperature distribution, the body force term of the Eq. (5) is

ρ g cosθ d s = ρ0  1 +β0 T0  g cosθ d s − ρ0 β0 Ts g cosθ d s

The first closed integral of the RHS of the last equation is zero for the given angle distribution.  Thus

ρ g cosθ d s = − ρ0 β0 Ts g cosθ d s  = −ρ0 β0 g Δ T LH2 + T2 LCH − LH2 − LC2 + Δ T LC2 − T1 LCH − LC2 − LH2

where  Δ T = T2 − T1.  Thus,

ρ g cosθ d s = −ρ0 β0 g Δ T  LH2 + LCH − LH2 − LC2 + LC2

Therefore, after the substitution of the angle values for positive velocities in the direction 1-2 in the hot leg it results

−ρ0 β0 g LCH T2 − T1 + j f Δ sjDh+Kj m2 2 ρj Aj2 = 0                           (6)

where

K = k Kk

in the length interval Δ sj.

The summation term in the Eq. (6) is used to define the total hydraulic resistance of the circuit as:

HR = j f Δ sDh+Kj 1 Aj2                                                                (7)

The circuit driven head is defined as the opposite of the other term.  Thus

DH = ρ0 β0 g LCH T2 − T1                                                                 (8)

The Eq. (6) and the Eq. (7) are employed to formally define an average density as

ρ = HRj f Δ sDh+Kj 1 ρj Aj2                                                                 (9)

With the HR, DH, and the average density definitions, the Eq. (6) is then equivalent to:

HR m2 = 2 ρ DH                                                                             (10)

By the substitution of the driving head DH from the Eq. (8) into the Eq.(10) the mass flow rate m can be obtained as

m = 2 ρ ρ0 β0 g LCH T2 − T1HR                                                            (11)

Given the thermal power applied to the heater and measuring the temperatures in the cold and in the hot legs, the mass flow rate mcan also be calculated from the steady state heat balance

Q = m cp T2 − T1                                                                            (12)

where Q is the actual heating power transferred to the liquid water and cp  is the average specific heat capacity in the heater.  The Eq. (12) can also be obtained from the steady state condition applied to the energy conservation Eq. (3) neglecting the axial heat diffusion in the fluid and the heat generated by the fluid friction.

The driving head DH is taken from the Eq. (8) and the mass flow rate m from the Eq. (12) are substituted into the Eq. (10) and solved for the total hydraulic resistance HR

HR = 2 ρ ρ0 β0 g LCH T2 − T13Q/cp 2                                                       (13)

Similarity

The concept of similarity between two different scaled hydraulic systems is important to test the full scale system in a reduced scale and thus obtain data important to verify its performance.  Thus analytic design models can be validated in the small scale experiment.  Similar systems are defined as systems that satisfy the same equations.  A corollary of this definition is that similar systems must have the same non-dimensional hydraulic numbers.  One of these numbers is the overall friction number that is defined as the hydraulic resistance times the square of the heater cross-sectional area of each system.  As the heater contains N heated cylindrical rods of diameter, d , in a square array with a space, e , between centers, its equivalent cross section flow area is

AH = N e2 − π d24                                                                      (14)

and the corresponding friction number is

NFric = HR AH 2                                                                           (15)

#### b) Two-phase flow

Mass conservation equation:

A∂ ρ∂ t+∂ m∂ s = 0                                                                            (1b)

Momentum conservation equation:

∂ m∂ t+ u ∂ m∂ s+ m ∂ u∂ s +∂∂ sAαg ρfρg1−α ρugj2+A∂ p∂ s +A ρ g cosθ+PhΓw=0   (2b)

Constitutive relation for friction:

Ph Γw= fw2 Dh  u  m = fw2 Dh ρ A  m  m

Conservation of internal energy:

∂ A ρ e∂ t+∂ m h∂ s+∂∂ sAαg ρf ρg ρ hfg ugj−u A ∂ p∂ s = Pa qw,,+ u Ph Γw  (3b)

Liquid water and steam mixture density:

ρ = α ρg +  1 − α  ρf = ρf − αs Δ ρ                          (4b)

Other thermodynamic state equations:

h = x hgp +  1 − x  hfp

e = h − p/ρ

T=Tsatp

Mixture momentum flux:

j = α ug +  1 − α  uf

By u, p, ρ, h, e, and T, are denoted the mixture velocity, the pressure, the mixture density, the mixture enthalpy, the mixture internal, and the fluid saturation temperature, respectively.  The thermodynamic enthalpies for the liquid and steam phases are hfand hg.  The densities for the liquid and steam phases are ρf and ρg.  The velocities for the liquid and steam phases are uf and ug.  The steam velocity relative to the mixture momentum flux, j, is the drift velocity, ugj = ug − j.  The transverse flow area is represented by A, s is the line coordinate, and t is the time. The gravity acceleration field isg and θ is the angle of a component axis with the vertical.  The friction factor is fw, the hydraulic perimeter is Ph, the hydraulic diameter is Dh, and the  mass flow rate is m=ρ u A.  The liquid specific heat at constant pressure is cp and the liquid thermal conductivity is k.  The heated perimeter is Pa and qw'' is the heat flux from a heating wall to the fluid.  To complete the symbol definitions it is well to explain that the steam quality is x, the void fraction (steam fraction) is α, Δ ρ = ρf − ρg is the density difference, and the heat of vaporization is hfg = hg− hf.

It is assumed a steady-state condition (thus ∂ m∂ t=0) a uniform fluid properties in the hotter branch (heater and heat exchanger upper half and hot leg) and in the cold branch (heater and heat exchanger lower half and cold leg).  It is assumed that the flow is incompressible (thus ∂ m∂ s=0).  In the uniform area sections it is also neglected all momentum transport by convection (thus  m ∂ u∂ s =0 and ∂∂ sAαg ρfρg1−α ρugj2=0).  The effect of the area changing locations in the momentum balance is incorporated as the denominated form loss coefficient dissipation, being added to the friction factor.  From the incompressible flow approximation, ∂ m∂ s=0, the mass flow rate is uniform on all circuit cross sections (m=uniform).  From the mass conservation equation it also follows that ∂ ρ∂ t=0, the condition of a steady density with time.  The momentum balance is written as:

A∂ p∂ s +A ρ g cosθ + fDh + Kk δs − sk  m  m 2 ρ A=0

It is employed the usual Einstein summation notation for repeated indices, Kk are local form loss coefficients, and δ(s−sk) is the usual Dirac function representation.

This simplified momentum is integrated from the heater (section 1) to the heat exchanger (section 2) in the hot leg path, and from the heat exchanger (section 2) to the heater (section 1) in the cold leg path, to obtain

ρ g cosθ d s+ fDh+Kk δk(s−sk)  m  m 2 ρ A2d s = 0                              (5b)

According to the Einstein summation notation and the Dirac function definition, the last term of the Eq. (5b) is

Kk δs − sk  m  m 2 ρ Ads = kKk  m  m 2 ρk Ak

The angle values are  cosθ = 1 in the hot leg and  cosθ = −1 in the cold leg.  The void fraction are  α = α2 in the hot leg and  α = α1 in the cold leg.  It is assumed that the void fraction increases linearly with distance from  α = α1 to  α = α2 in the heater (with length  LH) and decreases linearly from  α = α2 to  α = α1 in the cooler (with length  LC).  The vertical distance between the cooler and the heater centers is  LCH.  The densities are thus calculated from this void fraction distribution.

From the substitution of these angle values, the density according to the Eq. (4b) and the given t void fraction distribution, the body force term of the Eq. (5b) is

ρ g cosθ d s = ρf g cosθ d s − Δ ρ αs g cosθ d s

The first closed integral of the RHS of the last equation is zero for the given angle distribution.  Thus

ρ g cosθ d s = − Δ ρ αs g cosθ d s  = −Δ ρ g Δ α LH2 + α2 LCH − LH2 − LC2 + Δ α LC2 − α1 LCH − LC2 − LH2

where  Δ α = α2 − α1.  Thus,

ρ g cosθ d s = −Δ ρ g Δ α  LH2 + LCH − LH2 − LC2 + LC2

Therefore, after the substitution of the angle values for positive velocities in the direction 1-2 in the hot leg it results

−Δ ρ g LCH  α2 − α1  + j f Δ sjDh+Kj m2 2 ρj Aj2 = 0                         (6b)

where

K = k Kk

in the length interval Δ sj.

The summation term in the Eq. (6b) is used to define the total hydraulic resistance of the circuit as:

HR = j f Δ sDh+Kj 1 Aj2                                                               (7b)

The circuit driven head is defined as the opposite of the other term.  Thus

DH = Δ ρ g LCH  α2 − α1                                                                 (8b)

The Eq. (6b) and the Eq. (7b) are employed to formally define an average density as

ρ = HRj f Δ sDh+Kj 1 ρj Aj2                                                                (9b)

With the HR, DH, and the average density definitions, the Eq. (6b) is then equivalent to:

HR m2 = 2 ρ DH                                                                         (10b)

By the substitution of the driving head DH from the Eq. (8b) into the Eq.(10b) the mass flow rate m can be obtained as

m = 2 ρ Δ ρ g LCH  α2 − α1 HR                                                        (11b)

Given the thermal power applied to the heater and measuring the void fractions in the cold and in the hot legs, the mass flow rate mcan also be calculated from the steady state heat balance

Q = m h2 − h1                                                                             (12b)

where Q is the actual heating power transferred to the fluid in the heater.  The Eq. (12b) can also be obtained from the steady state condition applied to the energy conservation Eq. (3b) neglecting the convection of the drift velocity term, ∂∂ sAαg ρf ρg ρ hfg ugj = 0, the heat transported by the pressure divergent term −u A ∂ p∂ s = 0, and the heat generated by the fluid friction, u Ph Γw = 0.

The driving head DH is taken from the Eq. (8b) and the mass flow rate m from the Eq. (12b) are substituted into the Eq. (10b) and solved for the total hydraulic resistance HR

HR = 2 ρ Δ ρ g LCH  α2 − α1 Q/h2 − h1 2                                                       (13b)

### Pre-Lab Preparation

- Read through the theory section from this experiment.

- Refer to [1], [2] textbooks and to class notes for any questions.

- Bring one virus-free blank floppy disk to lab.

- Bring blank paper sheets to take notes during the experiment.

### Workstation Details

- Pentium PC computer with National Instruments LabVIEW software.

- National Instruments DAQ boards (inside the computer).

- National Instruments cables and connectors (outside the computer).

### Lab Procedure

The experiment is divided in two parts. Part 1 is to be completed prior Part 2.

#### Setup (for both parts)

- Check if the following systems are turned on: power control unit, temperature data acquisition unit and the computer.

- Launch LabVIEW.

- Open ccn.vi located in CCN.llb in the C:\CCN.

- Set the power as shown in Table 2 (1290 Watts for part 1) and Table 2b (1425 Watts for part 2).

- Run CCN.vi.

- A window will open and you must give a name to a file where the data will be saved.

- Press Save button to start the experiment.

Part 1 (Single-phase experiment)

Your overall goal is to obtain the hot leg and the cold leg fluid temperatures from the thermocouples T4 and T8 graphs as a function of time until the difference reach a steady value .  From them:

- Estimate the overall friction coefficient of the circuit as explained in the Instructor’s Portion.

- Calculate the stationary mass flow rate as defined in the Instructor’s Portion.

- Given the overall friction coefficient of the full scale similar circuit of the advanced power reactor (prototype) the geometric data of the heating channels of both, the full size prototype and the reduced scale experiment, calculate the overall friction number, thus demonstrating the hydraulic similarity of the systems as defined in the Instructor’s Portion.

Example:

The geometric data of the CCN and of the full size prototype similar to the CCN are given in the Table 1.

TABLE 1.  Geometric Data of the Systems

|  | Prototype | CCN |
| --- | --- | --- |
| LH (m) | 3.66 | 0.366 |
| LC (m) | 6.0 | 0.6 |
| LCH (m) | 9.44 | 0.944 |
| N ( #) | 38280 | 52 |
| e (mm) | 12.6 | 12.7 |
| d (mm) | 9.144 | 9.525 |

The thermodynamic and operation data of the CCN are given in the Table 2.

TABLE 2.  Thermodynamic and Operation Data of the CCN

|  | CCN |
| --- | --- |
| Density, : (Kg/m3) | 958.5 |
| Thermal Expansion, : (0C-1) | 6.87 x 10-4 |
| Specific Heat, cp: (J/Kg) | 4213.0 |
| Applied Power (Watts) | 1290 |
| Percent Power Loss (%) | 15 |
| Liquid Power, Q (Watts) | 1096.5 |
| Temperature Difference (T2 – T1) (0C) | Read from Graph |

The temperature readings from the hot leg thermocouple T4 and from the cold leg thermocouple T8 are plotted in Fig. 1 and Fig. 2, respectively, as a function of time.  These data are plotted together in the Fig. 3.

Figure 1.  Hot Leg Water Temperature.

Figure 2.  Cold Leg Water Temperature.

Figure 3.  Hot Leg and Cold Leg Water Temperatures.

From the Fig. 3 graphs the temperature difference between the hot and the cold legs seem to reach a steady difference close to 100C.  As the expansion coefficient of the water is small, the average and the reference densities are close to the value given in the Table 2.  Thus, the numerical values from the Table 1 and Table 2 are substituted into the Eq. (13) to calculate the total hydraulic resistance HR of the circuit

1.

HR = 2 958,5Kg/m32 6.87×10−4 0C−1 9.81 m/s2 0.944 m 10 0C3 1096.5 W/ 4213 J/Kg0C 2

HR = 1.7257×108 m−4

This HR value and the other needed data are substituted into the Eq. (11) to obtain the natural circulation mass flow rate in the circuit

2.

m = 2 958,5Kg/m32 6.87×10−4 0C−1 9.81 m/s2 0.944 m 10 0C1.7257×108 m−4

m = 2.61×10−2 Kg/s

The hydraulic similarity for two-phase operation is demonstrated next.  The hydraulic resistances of the full-scale prototype similar to CCN, calculated in anterior study [7] are given in the Table 3.

TABLE 3. Hydraulic Resistances of the Prototype.

|  | Prototype |
| --- | --- |
| Pressure (MPa) | 7.446 |
| Saturated Liquid Density, f: (Kg/m3) | 732.09 |
| Saturated Steam Density, g: (Kg/m3) | 39.16 |
| Density Difference, : (Kg/m3) | 692.93 |
| Cold Leg Hydraulic Resistance (m-4) | 130.1 |
| Downcomer Hydraulic Resistance (m-4) | 1.3 |
| Lower Plenum Hydraulic Resistance (m-4) | 0.39 |
| Heater Hydraulic Resistance (m-4) | 1.96 |
| Upper Plenum Hydraulic Resistance (m-4) | 0.95 |
| Hot Leg Hydraulic Resistance (m-4) | 178.32 |
| Cooler Hydraulic Resistance (m-4) | 2.12 |
| Total Hydraulic Resistance (m-4) | 315.14 |

From the Table 1 data, the cross-sectional area of the heater of each system, AH and the Friction Number NFric are

3.

For the Prototype:

AH = 38280 12.62 − π 9.14424×10−6 m2 = 3.562 m2

NFric = 315.14 3.562 2 = 3998

For the CCN:

AH = 52 12.72 − π 9.52524×10−6 m2 = 4.682×10−3 m2

NFric = 1.7257×108 4.682×10−32 = 3783

The hydraulic similarity data is summarized in the Table 4.

TABLE 4.  Hydraulic Similarity of the Systems

|  | Prototype | CCN |
| --- | --- | --- |
| HR (m-4) | 315.14 | 1.7257x108 |
| AH (m2) | 3.562 | 4.682x10-3 |
| NFric (#) | 3998 | 3783 |

It can be seen in Table 4 that despite the very small scale of the CCN compared to the full-scale Prototype, CCN has pretty good hydraulic similarity with respect to the total friction number (Nfric) a very important result that contributes to the complete thermal-hydraulic similarity.

Part 2 (Two-phase flow analysis)

The main objective is to estimate the hot leg and the cold leg void fractions 2 and 1 as a function of time until the difference reach a steady value .  From them:

- Estimate the overall friction coefficient of the circuit as explained in the Instructor’s Portion.

- Calculate the stationary mass flow rate as defined in the Instructor’s Portion.

- Given the overall friction coefficient of the full scale similar circuit of the advanced power reactor (prototype) the geometric data of the heating channels of both, the full size prototype and the reduced scale experiment, calculate the overall friction number, thus demonstrating the hydraulic similarity of the systems as defined in the Instructor’s Portion.

Example:

The geometric data of the CCN and of the full size prototype similar to the CCN are given in the Table 1.

Some results from a simulation model [7] of the two-phase operation of CCN are given i the Table 2b.  From the Table 2b the void fraction difference between the hot and the cold legs is 0.2937.  The average density is given in the Table 2b.  Thus, the numerical values from the Table 1 and Table 2b are substituted into the Eq. (13b) to calculate the total hydraulic resistance HR of this circuit for two-phase operation.

4.

HR = 2 814.8Kg/m3 957.9Kg/m3 9.81 m/s2 0.944 m 0.3 1396.5 W/ 5818.75 J/Kg 2

HR = 7.529×107 m−4

This HR value and the other needed data are substituted into the Eq. (11b) to obtain the natural circulation mass flow rate in the circuit

5.

m = 2 814.8Kg/m3 957.9Kg/m3 9.81 m/s2 0.944 m 0.37.529×107 m−4

m = 0.24 Kg/s

This value of the mass flow rate coincides with the value obtained with the simulation model [7] thus illustrating the usefulness of the present steady state formulation.

TABLE 2b.  Thermodynamic and Operation Data of the CCN for Two-Phase Flow.

|  | CCN |
| --- | --- |
| Pressure (MPa) | 0.10132 |
| Saturated Liquid Density, f: (Kg/m3) | 958.5 |
| Saturated Steam Density, g: (Kg/m3) | 0.59762 |
| Density Difference, : (Kg/m3) | 957.9 |
| Saturated Liquid Enthalpy, hf: (KJ/Kg) | 410.64 |
| Saturated Steam Enthalpy, hg: (KJ/Kg) | 2675.2 |
| Heat of Vaporization, hfg: (KJ/Kg) | 2264.5 |
| Applied Power (Watts) | 1425 |
| Percent Power Loss (%) | 2 |
| Liquid Power, Q (Watts) | 1396.5 |
| Hot Leg Void Fraction, 2 (#) | 0.3 |
| Cold Leg Void Fraction, 1 (#) | 0.0 |
| Hot Leg Steam Quality, x2 (#) | 2.57x10-3 |
| Cold Leg Steam Quality, x1 (#) | 0.0 |
| Hot Leg Density, 2: (Kg/m3) | 671.1 |
| Cold Leg Density, 1: (Kg/m3) | 958.5 |
| Average Density, ρ: (Kg/m3) | 814.8 |
| Hot Leg Enthalpy, h2: (J/Kg) | 416458.75 |
| Cold Leg Enthalpy, h1: (J/Kg) | 410640.00 |
| Enthalpy Variation, h: (J/Kg) | 5818.75 |
| Hot Leg Internal Energy, e2: (J/Kg) | 416307.77 |
| Cold Leg Internal Energy, e1: (J/Kg) | 410534.29 |

The hydraulic similarity for two-phase operation is demonstrated next.  The hydraulic resistances of the full-scale prototype similar to CCN, for two-phase flow operation, are given in the Table 3b.

TABLE 3b.  Thermodynamic and Operation Data of the Prototype for Two-Phase Flow.

|  | Prototype |
| --- | --- |
| Pressure (MPa) | 7.446 |
| Saturated Liquid Density, f: (Kg/m3) | 732.09 |
| Saturated Steam Density, g: (Kg/m3) | 39.16 |
| Density Difference, : (Kg/m3) | 692.93 |
| Hot Leg Void Fraction, 2 (#) | 0.4537 |
| Cold Leg Void Fraction, 1 (#) | 0.0 |
| Two-Phase Multiplier = 1 − Δ α Δ ρ2 ρf−1 | 1.2734 |
| Two-Phase Multiplier = 1 − Δ α Δ ρρf−1 | 1.7526 |
| Cold Leg Hydraulic Resistance (m-4) | 130.1 |
| Downcomer Hydraulic Resistance (m-4) | 1.3 |
| Lower Plenum Hydraulic Resistance (m-4) | 0.39 |
| Heater Hydraulic Resistance (m-4) | 1.96 1 − Δ α Δ ρ2 ρf−1 = 2.50 |
| Upper Plenum Hydraulic Resistance (m-4) | 0.95 1 − Δ α Δ ρρf−1 = 1.66 |
| Hot Leg Hydraulic Resistance (m-4) | 178.32 1 − Δ α Δ ρρf−1 = 312.52 |
| Cooler Hydraulic Resistance (m-4) | 2.12 1 − Δ α Δ ρ2 ρf−1 = 2.70 |
| Total Hydraulic Resistance (m-4) | 451.17 |

From the Table 1 data, the cross-sectional area of the heater of each system, AH and the Friction Number NFric are

6.

NFric = 451 3.562 2 = 5722

For the CCN:

NFric = 2.9836×108 4.682×10−32 = 6540

The hydraulic similarity data is summarized in the Table 4b.

TABLE 4b.  Hydraulic Similarity of the Systems for Two-phase flow

|  | Prototype | CCN |
| --- | --- | --- |
| HR (m-4) | 451.17 | 2.9836x108 |
| AH (m2) | 3.562 | 4.682x10-3 |
| NFric (#) | 5722 | 6540 |

It can be seen that despite the very small scale of the CCN compared to the full-scale Prototype, CCN has a good hydraulic similarity with respect to the total friction number for two-phase flow operation, a very important result for the complete thermal-hydraulic similarity.

### Lab Report

We would expect a formal Lab Report where the students explain the theory, including a review of the similarity concept, and their opinion on the importance of analytic methods validation in scaled similar experiments.  They will be ask to perform some study enabling them to clearly explain of the reason for being identical two analytic problems, of a full-scale and a reduced scale similar experiments.  Finally, a critical analysis of the experimental accuracy and of the thermodynamic data, the source of the errors, and which should be the adjusted or tuning variables in the experiment.

#### General Requirements

You must prepare your own report. The report will be given to lab’s instructor as printed paper. You must use a text processor.

You must include in your lab report a data treatment program. This program should be able to read the temperature data input from LabVIEW as a function of time, and to generate a temperature data sheet which will be used to create the thermocouples T4 and T8 plots shown in Fig. 1 to Fig. 3.

Guidelines

- Cover sheet with student name’s report information.

- Introduction – brief general discussion of the experiment including a theoretical highlights.

- Experimental Apparatus Description – experimental facilities description including drawings and specifications, equipment used and experimental procedures.

- Experimental Results – should be presented as tables, graphs, and other information used in the analyses.

- Discussion – an interpretational discussion should be included on the temperature – time plots and experimental results uncertainties.

- Concluding Remarks – conclusions and improvements of the work in your own opinions.

- References.

### Data Sheet

For both parts of the experiment, you need to record at least five ambient air temperature value during the runs (as shown by digital thermometer), and heat exchanger secondary side flowrate (ask the instructor for more details). Pay attention: these data are not saved in the automatic data acquisition system!