# VISA and Serial Communication

Matthew Otis

Texas A&M University

## Instructor’s Portion

### Summary

This lab will show students why there is a need for instrument control, the software architecture to control it, and how to do basic serial communication.  The main focus of this lab is to teach students how to communicate with instruments and how to use NI-VISA to communicate with instruments using different communication protocols (Serial, GPIB, etc.).  The students will also gain LabVIEW experience by making several VIs.

### Uses

This experiment can be used throughout all engineering and science disciplines as long as there is a need for serial communication with an external device.

### Equipment List

- A computer with Windows, Macintosh, Linux, Sun, or HP-UX (individual system requirements can be found at: http://www.ni.com/labview/lv_sysreq.htm).

- LabVIEW 6i or better (ie LabVIEW 6.1 PDS part number 776678-03).

- RS-232 Serial Port on the computer being used.

- RS-232 Cable (part number 182238-01).

- NI Instrument Simulator (part number 777369-01) or other Serial Device.

- Paperclip or a piece of wire.

### Setup

- This lab is written under the presumption that there will be one NI Instrument Simulator for each workstation.  However you may be able to work around this and have less than one NI Instrument Simulator per station.  The students could build the VIs on one computer, transport the VI via disk, and then attempt to run it on another computer that is connected to the NI Instrument Simulator.

- While the lab procedure could be fairly easily adapted for use with another instrument, it is designed for the NI Instrument Simulator.  Most of the basic procedures and concepts will remain the same regardless of the instrument, however many of the specifics will vary with use of a different instrument

- Before the Day of Lab

- Install LabVIEW (see the LabVIEW Release Notes for your version of LabVIEW).

- Ensure that the NI Instrument Simulator has power and that the power light is lit when turned on.

- Conduct a run-through of the lab procedure the students will perform.

- On the day of Lab

1. Make sure everything is powered on before commencing the lab.

### References

- Bishop, Robert H. (2001), LabVIEW Student Edition 6i, Prentice-Hall Inc. Upper Saddle River, NJ., Section 10

## Student’s Portion

### Introduction

A typical laboratory setup will use multiple instruments to conduct an experiment.  Some common examples of instruments are oscilloscopes, digital multimeters, and function generators, but an instrument can be anything used to record, measure, or control.  While the functionality of each instrument is different, most instruments have two common components:

User Interface - The user interface is commonly made up of dials, buttons, or knobs for the user to provide input to the instrument, and display screens or digital readouts for the user to see the output from the instrument.

Communication Protocol - In this experiment we will look at one of the most common and simplest of communication protocols known as serial communication.  Serial is very useful for smaller amounts of data, and is conceptually quite easy to understand.

Varied protocols for communication such as serial, General Purpose Interface Bus (GPIB), VXI, etc. are available to engineers and scientists.  However, they are all inherently different, rendering each protocol incompatible with any other. Industry has been working to make a standardized system that can operate all of these devices.  This standard has become known as Virtual Instrument Software Architecture (VISA).  We will use VISA within LabVIEW to communicate via serial.  VISA can serve to greatly simplify the making of many programs due to its versatility across communication protocols.

### Objective

- Understand the benefits of VISA and when to use it.

- Learn how serial communication works.

- Learn when to use serial communication.

- Learn how to use LabVIEW to communicate with serial instruments.

### Theory

Serial Communication

RS-232 Instrument

RS-232 Cable

PC Serial Port

Figure 1: An RS-232 Instrument is connected to the PC Serial Port with an RS-232 Cable.

Serial communication is a popular means of transmitting data between a computer and a peripheral device such as a programmable instrument or even another computer. Serial communication uses a transmitter to send data, one bit at a time, over a single communication line to a receiver. You can use this method when data transfer rates are low or you must transfer data over long distances. Serial communication is popular because most computers have one or more serial ports, so no extra hardware is needed other than a cable to connect the instrument to the computer or two computers together.

Serial communication requires that you specify the following four parameters: the baud rate of the transmission, the number of data bits encoding a character, the sense of the optional parity bit, and the number of stop bits. Each transmitted character is packaged in a character frame that consists of a single start bit followed by the data bits, the optional parity bit, and the stop bit or bits. The Figure 5 shows a typical character frame encoding the letter m.

Figure 2: A byte is sent as a package including a start bit, data bits, sometimes a parity bit, and stop bits.

Baud rate is a measure of how fast data are moving between instruments that use serial communication. RS-232 uses only two voltage states, called MARK and SPACE. In such a two-state coding scheme, the baud rate is identical to the maximum number of bits of information, including control bits that are transmitted per second.

MARK is a negative voltage, and SPACE is positive. The previous illustration shows how the idealized signal looks on an oscilloscope.

The following is the truth table for RS-232:

Signal > +3 V = 0

Signal < –3 V = 1

The output signal level usually swings between +12 V and –12 V. The dead area between +3 V and –3 V is designed to absorb line noise.

A start bit signals the beginning of each character frame. It is a transition from negative (MARK) to positive (SPACE) voltage. Its duration in seconds is the reciprocal of the baud rate. If the instrument is transmitting at 9600 baud, the duration of the start bit and each subsequent bit is about

0.104 ms. The entire character frame of eleven bits would be transmitted in about 1.146 ms.

Data bits are transmitted upside down and backwards. That is, inverted logic is used, and the order of transmission is from least significant bit (LSB) to most significant bit (MSB). To interpret the data bits in a character frame (Figure 2), you must read from right to left and read 1 for negative voltage and 0 for positive voltage. This yields 1101101 (binary) or 6D (hex). An ASCII conversion table (Figure 3) shows that this is the letter m.

Figure 3: Table with Hex values for ASCII characters.

An optional parity bit follows the data bits in the character frame. The parity bit, if present, also follows inverted logic, 1 for negative voltage and 0 for positive voltage. This bit is included as a simple means of error handling. You specify ahead of time whether the parity of the transmission is to be even or odd. If the parity is chosen to be odd, the transmitter then sets the parity bit in such a way as to make an odd number of ones among the data bits and the parity bit. This transmission uses odd parity. There are five ones among the data bits, already an odd number, so the parity bit is set to 0.

The last part of a character frame consists of 1, 1.5, or 2 stop bits. These bits are always represented by a negative voltage. If no further characters are transmitted, the line stays in the negative (MARK) condition.

The transmission of the next character frame, if any, is heralded by a start bit of positive (SPACE) voltage.

Knowing the structure of a character frame and the meaning of baud rate as it applies to serial communication, you can calculate the maximum transmission rate, in characters per second, for a given communication setting. This rate is just the baud rate divided by the bits per frame. In the previous example, there are a total of eleven bits per character frame. If the transmission rate is set at 9600 baud, you get 9600/11 = 872 characters per second. Note that this is the maximum character transmission rate. The hardware on one end or the other of the serial link might not be able to reach these rates, for whatever reason.

There are many different recommended standards of serial port communication, including the following most common types.

RS-232

This standard has been adopted for use in serial communications between computers and printers, modems, and other equipment. The serial ports on standard IBM-compatible personal computers follow RS-232.  Figure 4 shows what an RS-232 connector looks like.  Figure 5 entails details about what each pin does.

Figure 4: RS-232 Serial Ports contain 9 different pins each of which has its own separate function.

| Line # | Line Name | Line Type | Function |
| --- | --- | --- | --- |
| 1 | Carrier Detect | Handshake | Determines if the port is connected to a working device. |
| 2 | Receive Data | Data | Receives information from the device. |
| 3 | Transmit Data | Data | Computer sends information to the device. |
| 4 | Data Terminal Ready | Handshake | Computer tells the device that it is ready to talk. |
| 5 | Signal Ground | Common | Pin is grounded. |
| 6 | Data Set Ready | Handshake | Device tells the computer that it is ready to communicate. |
| 7 | Request To Send | Handshake | Computer asks the device if it can send information. |
| 8 | Clear To Send | Handshake | Device tells the computer that it can send information. |
| 9 | Signal Indicator | Other | Computer acknowledges that a signal was detected. |

Figure 5: Table lists what each individual line’s name is, whether it is used for data transmission or handshaking purposes, and what the exact function of that line is.

RS-449, RS-422, RS-423

The RS-449, RS-422, and RS-423 are additional EIA serial communication standards related to RS-232. RS-449 was issued in 1975 and was supposed to supersede RS-232, but few manufacturers have embraced the newer standard.

VISA

Figure 6: VISA is able to communicate with all of these different forms of communication by calling on their individual low-level drivers.  This allows for faster software development time.

Virtual Instrument Software Architecture (VISA) is the lower layer of

functions in the LabVIEW instrument driver VIs that communicates with the driver software.

VISA was developed and designed to have the following attributes:

• Maximize ease of use and performance.

• Maintain long-term compatibility with the installed base.

• Maintain multivendor open architectures.

• Maximize multiplatform capability.

• Maximize expandability and modularity in frameworks.

• Maximize software reuse.

• Standardize the use of system software elements.

• Treat instrument drivers as part of the instrument.

• Accommodate established standards.

• Maximize cooperative support of users.

VISA by itself does not provide instrumentation programming capability.

It is a high-level Application Programmer Interface (API) that calls in low-level drivers. VISA can control VXI, GPIB, serial, or computer-based instruments and makes the appropriate driver calls depending on the type of instrument used. When debugging VISA problems, remember this hierarchy. An apparent VISA problem could be an installation problem with one of the drivers that VISA calls.

In LabVIEW, VISA is a single library of functions you use to communicate with GPIB, serial, VXI, and computer-based instruments. For example, some instruments give you a choice for the type of interface. If the LabVIEW instrument driver were written with functions on the Functions» Instrument I/O»GPIB palette, those instrument driver VIs would not work for the instrument with the serial port interface. VISA solves this problem by providing a single set of functions that work for any type of interface.

VISA Programming Terminology

The functions you can use with a resource are operations. The resource also has variables, or attributes, that contain information related to the resource. The following terminology is similar to that used for instrument driver VIs:

• Resource—Any instrument in the system, including serial and parallel ports.

• Session—You must open a VISA session to a resource to communicate with it, similar to a communication channel. When you open a session to a resource, LabVIEW returns a VISA session number, which is a unique refnum to that instrument. You must use the session number in all subsequent VISA functions.

• Instrument Descriptor—Exact name of a resource. The descriptor specifies the interface type (GPIB, VXI, ASRL), the address of the device (logical address or primary address), and the VISA session type (INSTR or Event).

The instrument descriptor is similar to a telephone number, the resource is similar to the person with whom you want to speak, and the session is similar to the telephone line. Each call uses its own line, and crossing these lines results in an error.

Figure 7: VISA Write writes data to a specified device.

The VISA Write function writes the write buffer string to the device specified by the VISA resource name. dup VISA resource name returns the same handle to that session. On UNIX platforms, data are written synchronously. On all other platforms, it is written asynchronously. Return count contains the number of bytes actually transferred across the GPIB. The error in and error out clusters contain the error information.

Figure 8: VISA Read reads the specified number of bytes from specified device or interface.

The VISA Read function (Figure 8) reads data from the device specified by the VISA resource name. byte count indicates the number of bytes to be read into the returned read buffer string. dup VISA resource name returns the same handle to that session. On UNIX platforms, data are read synchronously. On all other platforms, it is read asynchronously. return count contains the number of bytes actually transferred across the GPIB. The error in and error out clusters contain the error information.

How to use

Use the VIs and functions located on the Functions>Instrument I/O>Serial palette for serial port communication.

The VISA Write and VISA Read functions work with any type of instrument communication and are the same whether you are doing GPIB or serial communication. However, because serial communication requires you to configure extra parameters, you must start the serial port communication with the VISA Configure Serial Port VI.

The VISA Configure Serial Port VI initializes the port identified by VISA resource name to the specified settings. timeout sets the timeout value for the serial communication. baud rate, data bits, parity, and flow control specify those specific serial port parameters. The error in and error out clusters maintain the error conditions for this VI.

### Pre-Lab Preparation

- Read the theory and lab procedure portion of this experiment.

- Bring with you:

- Pen or Pencil.

- A formated 3.5-inch floppy disk.

- This experiment.

### Workstation Details

- Computer with LabVIEW Software

- A regular RS-232 serial cable

- NI Instrument Simulator or another Serial Device

- Piece of wire or paperclip

### Lab Procedure

- In order to understand how serial is working, you must first understand what serial is working with.  A serial byte is sent as a packet of information and that information includes start, data, and stop bits.  

a. 	Search online, and try to find a table with the binary values for the individual ASCII characters (or you can convert the Hex values for ASCII characters from Figure 3 into the needed binary values).

- b.	On your data sheet draw out what your initials would look like if they were sent in a serial transmission with one start bit, eight data bits, no parity bit, and one stop bit
(i.e. N -> Hex: 4E = Bin:01001110 &  I -> Hex: 49 = Bin:01001001 
N ->
           

I ->

- ).

- For you to gain a basic understanding of the way serial communication occurs you are going to construct a simple test called a loop-back test.  A loop-back test causes the signal received to simply be the same signal that was sent.

You are going to send a simple signal out, and then receive the same signal back and read it.  A loop-back test sends the outgoing signal back into the line for the incoming signal by making an external connection on the serial cable. This can be useful for seeing if a serial connection or cable is in proper working order.

First we will build a VI that uses Virtual Instrument Software Architecture (VISA) to communicate with the serial port.  

a. 	Open LabVIEW, create a new VI and go to the block diagram (Ctrl-E).  Then right click on the block diagram and go to Functions > Instrument I/O > Serial and place VISA Configure Serial Port, VISA Write, VISA Read, VISA Close on the block diagram.  
   

b. 	Place Time & Dialog > Simple Error Handler VI to indicate if any errors occurred.



Figure 9: Possible front panel for Serial Loop-Back Test.vi

- Figure 10: Block Diagram for Serial Loop-Back Test.vi

c.	VISA Resource Name, Write Buffer, Read Buffer, and Return Count can all be created by using the wiring tool and right clicking on their respective nodes and either creating an indicator or a control depending upon what is needed.

- d. 	Save the VI on your 3.5-inch floppy disk as Serial Loop-Back Test.vi. 

The structure of this VI does the following:

- Configures the serial port and prepares for data transmission.

- Writes the desired string, converts the string to the proper bit structure required for sending including the proper start, parity and stop bits.

- Reads the same number of bytes as sent, decodes the data from VISA Write from a binary string back into ASCII then to text as seen in the indicator

- Closes the VISA session

- Reports if any errors have occurred and where they occurred.

- To do the actual loop-back test we will need to connect the sending and the receiving lines together.  Refer to the above pin-out in Figure 7 and the information from Figure 8 to see which lines to will need to be connected.  If you properly do the loop-back test, you should receive the same information you sent.

e.	Now run the VI and use whatever string you wish in the write buffer.  Once the VI has run, make a screenshot of both the completed front panel and block diagram.

- Now that you know how to make a simple VI to communicate using the serial port, you will use those skills and to construct a VI that can communicate with an actual instrument. 


Figure 11: These switch settings configure the instrument as a serial device with the following settings:
     Baud rate = 9600
     Data bits = 8
     Parity = no parity
     Stop bits = 1
     Flow control parameters = hardware handshaking

- a.	Turn the power off on the NI Instrument Simulator and configure the switches according to Figure 11.  Once the switches are set connect the serial cable to the NI Instrument Simulator and turn the power on for the NI Instrument Simulator.  (To avoid errors, screw the cable in place so it does not become unplugged.)

- Figure 12: Possible Front Panel for Instrument Simulator Write & Read.vi.


Figure 13: Block Diagram for Instrument Simulator Write & Read.vi.

- b.	Now resave the previous VI as Instrument Simulator Write & Read.vi.  You will modify the block diagram to look like that of Figure 13.

- i.	All of the VISA commands can be found in Functions > Instrument I/O > Serial.

- ii.	Functions > String is where Line Feed Constant, Concatenate Strings, and Scan From String are located

- iii.	VISA Resource Name, Write Buffer, and Read Buffer, were all created by right clicking on their respective nodes and either creating an indicator or a control depending upon what is needed.

- The basic structure of this VI is the same as the previous with a few pointed differences:

- The NI Instrument Simulator requires a linefeed as the end of string character.

- The first VISA Read reads a string that tells the number of characters that will be returned by the device for the response to the query.

- Scan From String converts the number shown in the first read string into an actual number so that the next VISA Read knows how many characters to expect.

- VISA Read then receives the response with the proper # of characters.

- In order for the handshaking to properly work, you will need to configure your VISA port such that it is using RTS/CTS protocol (this is the protocol used by the NI Instrument Simulator). To ensure this protocol is being used you will need to create the proper constant on VISA Configure Serial Port.  RTS and CTS refer to lines 7 and 8 respectively on a serial port with RS-232 configuration.

- On the front panel put *IDN? in the write buffer (“”). This is a standard command that asks an instrument to identify itself.  The device should answer with National Instruments GPIB and Serial Device Simulator Rev B.1 (or the name of the device in use).

- Save and Run the VI. Once the VI has run, make a screenshot of both the completed front panel and block diagram.

- 3. 	The next VI is going to take data from the NI Instrument Simulator, convert the returned string data into numerical data, and display the numerical data on a graph so that the data is more easily recognized.  The same VI that we just used can be modified for this part of the lab.

- Figure 14: Possible front panel with data for Instrument Simulator with Graph.vi.

- Figure 15: Block diagram for Instrument Simulator with Graph.vi.

  - Open Instrument Simulator Write & Read.vi and save as Instrument Simulator with Graph.vi.

  - Spreadsheet String To Array can be found under Functions > String.

    - On Spreadsheet String To Array you will want to create constants on three of the nodes:

 - On the delimiter (Tab) put a  ,  in the constant (Delimiter tells the VI what character to look for so as to break the string into an array).

 - On Array Type (2D Format) simply create a constant and do not populate the array 
 
(This tells the VI what type of array you wish to make from the string data).

- On the format string put  %f  in the constant.

  - Format Strings can be found in the LabVIEW Help, under Format Specifier Syntax and Format Specifier Syntax Elements

  - Save the VI as Instrument Simulator with Graph.vi.

- This VI will do the same functions as the previous VI, except that the final data will be converted from string data to numerical data, so that the data can be displayed.

  - Type FORM:DATA ASC; SOUR:FUNC  SIN; SENS:DATA? in the write buffer (“”), and run the VI.

  - Once the VI has run, make a screenshot of both the front panel with the waveform and block diagram.

  - Type FORM:DATA ASC; SOUR:FUNC  SQU; SENS:DATA? In the write buffer (“”), and run the VI.

  - Once the VI has run, make a screenshot of both the front panel with the new waveform.

  - Look at the front of the NI Instrument Simulator to find some of the other strings that will result in waveforms.

  - Finish filling in the information on your Data Sheet.

  - Close LabVIEW.

  - Have your instructor verify that you have completed the lab.

  - Logoff of your computer.

### Lab Report

This lab report should be an informal report. The lab report will consist of the normal contents of an informal report: title, introduction, results, and discussion. The report must be typed. You should include the following in your lab report:

- Staple your data sheet to the back of your report.

- All screenshots should be included in your report.

- If there were any problems in the lab, discuss them.  Why do you think they occurred?

- Write about how you think that this communication can be improved upon, and what merits you think it currently has.

Sheet for students to fill out during lab with the data they have taken  (blank tables, questions, and sections for the instructor to initial, etc.).

What do you expect in the lab report? Will the report be formal or informal?

### Data Sheet

- Serial Transmission of ASCII characters.

  - What are your initials? __ __

  - Draw the serial representation of your initials:

First Initial =

Second Initial =

- Creating a Loop-Back Test

  - What VISA Resource Name do you have to use? ___________

    - Why do you think it is named as such?

    - What other VISA Resource Names were listed?

  - Which pins did you connect with the wire?

    - Will anything happen if you connect the wrong pins?

  - Why is VISA being used in this operation?

  - What string did you send?

- Instrument Simulator Write & Read

  - Which of the LEDs on the NI Instrument Simulator upon power the instrument on were lit?

  - Which LEDs changed while the VI was running?

  - What characters does the Line Feed Constant represent?

  - What do RTS and CTS stand for?

  - Why is handshaking necessary?

- Instrument Simulator with Graph

  - If you wanted to change a string into an array, what VI would you use?

  - What does a delimiter do?

  - What does %f do?

    - Will any other format string(s) work and yield the same results, if so which ones? (Hint: There are at least 2 others that will work)

  - In part 4-e, what are the first five data points returned?

    - What is the maximum value for any data point?

  - In part 4-f, what are the first five data points returned?

    - What is the maximum value for any data point?

  - _______________________________________________ and 

_______________________________________________ are two of the character strings used to generate other waveforms with the instrument simulator.

  - The NI Instrument Simulator only has a few sets of stored data points that it then randomly sends back as a set.  How many possible different sine waves are sent back?