EE133 – Lab 1

Amplitude Modulation and Demodulation

# Lab Notes

- A Word about Power Supplies: Remember that the SA612 is not rated for 9V.  We recommend that you power it up with 4.5V (and remember to use bypass capacitors).  Note that this is on the very low end of the recommended voltage range.  You will be building a 4.5V reference voltage supply that will be used by a few other blocks in the receiver (the IF stage and phase-locked loop), but because we will ultimately power our circuits with a 9V battery, this voltage can droop and vary as your circuit operates over time (hence your 4.5V supply could sag below 4.5V).  You can either plan to use that supply for the SA612 as well, or you can use a different voltage to power it.  We have LM7805 5V regulators available in the lab.  Note that this would require you to have three different voltage references (9V, 5V, and 4.5V), but it would give a more stable supply to the multiplier.

# Characterizing the Four-Quadrant Multiplier

The four-quad (AKA cross-coupled multiplier) is a much more versatile circuit than the basic transconductance (two-quad) multiplier.  One of the most important differences is the ability to allow carrier suppression.  It should be clear that all the information being transmitted in a radio signal is contained in the sidebands.  Thus, transmitting the carrier is a waste of power.  One of the key parameters of your cross-coupled multiplier is how much suppression of the carrier can be achieved.  This is measured in dB below the sidebands.  Thus, if carrier suppression is 50 dB, that means that the carrier was 50 dB below the amplitude of the sidebands.

The cross-coupled multiplier is not perfect, however.  One of the problems is the distortion that takes place because the carrier signal is not a small signal.  When you eventually get your multiplier working, you should notice that the output signal is not a modulated sine wave, but a distorted one.  This will result in copies of the carrier and sidebands at higher frequencies.

Figure 1: The SA612 Four-Quadrant Multiplier

We will now begin testing the cross-coupled multiplier built on the transmitter side.  You’ll notice that the measurements here can be somewhat repetitive.  You can reduce tedium in the lab by writing a LabView VI to automate this process.  While optional, writing the VI is highly encouraged as it will give you practice for later labs where the VI’s will be mandatory.  The TA’s will have pre-wired multipliers in the lab for you to test your code during office hours.

- Gain Conversion and Compression:

  - Start with a carrier frequency of 24 MHZ at +1.5 dBm power into the multiplier.  Note that there is an impedance mismatch, and that you will want to measure the correct voltage at the input with an active probe.  You might recall from the datasheet that using this magnitude of voltage at the input is not recommended for general use.  We will now examine why.

  - Use a modulating sine wave of frequency 2 MHz with amplitude 50 mV peak-to-peak.  Any frequency sinusoid should do, but 2 MHz gives clear spacing on the Agilent 8591E spectrum analyzer.

  - Observe the output waveform on the oscilloscope and spectrum analyzer.  Are these results consistent with your prelab?

  - What is the conversion gain?  Does it agree with the datasheet?

  - Using the oscilloscope to measure your signal, reduce the carrier level in steps until the multiplier ceases to act as a multiplier (we’re looking for a qualitative assessment, so just estimate this.)  Switch over to the 8591E spectrum analyzer and note that the signal is still discernable in the frequency domain (and still discernable by your circuit.)  What is the smallest level you can discern?  You may want to decrease the resolution bandwidth of the analyzer to lower the noise floor.  Keep this in mind as you measure your signals in the future.

Now carefully repeat this experiment with different carrier powers.  What changes do you observe in the frequency spectrum of the output?  Record any change in conversion gain with carrier power, and postulate a reason for these changes.  You can be qualitative for most of your records for this experiment, but be sure to record the output spectrum at 1.5 dBm and -45 dBm for comparisons to later measurements.

- Power Supply Sensitivity: Vary your power supply voltage between 3V and 7V.  Do you see any changes in conversion gain or the output?  Is this what you expected?  Plot these changes vs. voltage for your lab report.

- THD: Now, apply two new inputs, fc1 = 1.0 MHz with +1.5 dBm power to the LO port, fc2 = 1.003 MHz with 50 mV amplitude to the RF port.  You should get a 3 kHz output that you can view on the low-frequency signal analyzer to find THD.  Repeat for a RF generator setting of -45 dBm for fc1.  This information will be useful later on during system integration.

- Optimum Inputs: From your experiments, what should be the carrier input level to your SA612?  Is there a limit on the voltage of the modulating signal?

- Other figures of merit:  Think of other figures of merit you should be concerned about.  Consider especially that you will eventually use this in a mobile system.  Devise experimental setups for measure them and perform the measurements.  As always, record results in your lab notebook.

It’s a good idea to think about relevant figures of merit before each lab.  In the future, we will not be prompting you for every figure of merit; it will be up to you to decide what you need to measure.

- The Receiver SA612:  Make a few measurements of your choice on your receiver-side SA612 to characterize it and verify that it is working properly.  Be careful about the frequencies you choose to measure – the receiver side has AC coupling caps that only work at high frequencies.

# AM Demodulation using Synchronous Detection

To investigate demodulation, use the RF signal generator to produce a 400 Hz signal amplitude modulating a 24 MHz carrier.  Connect this output with the lower input of the SA612.  The outer input should be another 24 MHz sinusoid (usually called a local oscillator, or LO) from another signal generator.  Be sure that the LO power is high enough to give good conversion gain and carrier suppression.

- Distortion Measurements:  What is the THD on the received 400 Hz signal?  How does this change with varying signal and carrier amplitudes?  How does this THD compare with your envelope detector of lab 1?

- Your First Demodulated Audio Signal:  Now turn off the 400Hz sine wave and modulate the signal with an external input.  Use the output from the radio (using the headphone jack available in lab) as the modulating signal.  Connect the output of the circuit to the amplifier/speaker and enjoy the music!

- Your First Radio Transmission (optional):  If you finish early and have some extra time, you can play with transmitting and receiving signals over the air.

  - Transmitter: Connect an audio-band signal (from a stereo or microphone) to the input port of the SA612 on your transmitter.  Connect a 24 MHz signal from a signal generator to the carrier (LO) port.  Connect a long wire to the output to act as your antenna.

  - Receiver: At the input (RF) port of the receiver SA612, connect a wire antenna, similar to the one on the output of the transmitter.  Connect a 24 MHz signal to the LO port.  (You may need to make this a branch off the carrier from the transmit board.)  Finally, connect the output to a speaker.

Power up both boards and start your transmission.  You will probably not get much range out of this system, so keep the boards close.  You should now be able to hear your audio signal coming out of the receiver.  Congratulations on making your first radio transmission!

Alternate Receiver: If you have trouble, the 8591E spectrum analyzer has a demodulation feature.  To access this feature, press AUX CONTROL, then the Demod softkey (the keys on the edge of the screen), and select AM.  Adjust the volume using the small knob on the front panel of the 8591E.  You may need to increase the Dwell Time to get good results.