EE133 – Prelab 1

Amplitude Modulation and Demodulation

# Introduction

This week we will be taking a look at the amplitude modulation (AM) scheme.  In this process, the amplitude of the signal carrier is varied so that it is proportional to the instantaneous amplitude of the modulating signal.  With the invention of oscillators, this scheme quickly supplanted the first spark-gap transmitters.  They did so because AM supported the easy transmission of voice while allowing more people to transmit without interfering with each other.

To transmit an oscillating electrical signal though the air, it must first be converted into electromagnetic radiation.  To maximize the efficiency of transmission, the physical dimensions of the transmitting and receiving antennas must be on the same order of magnitude as the wavelength of the transmitted signal (often ¼ wavelength).  For audio signals, whose frequencies range from 10 Hz to 20 kHz, this corresponds to wavelengths from 15 km to 15000 km.  As you can probably guess, an antenna of this length would be absurd for general applications.

AM solves this problem by using a ‘slow’ moving audio signal to modulate a ‘fast’ moving radio signal.  The fast signal is called the carrier signal, and the audio frequency signal is typically called the modulating or baseband signal.  Typical carrier frequencies range from 30 kHz to upwards of 30 GHz.

By using a carrier, users can ‘tune’ in to a particular transmitter.  With the old spark-gap transmitters, every user heard all transmitters that were within range.  With interference problems, greatly reduced, the radio became something for the masses.

In this prelab, we will use LabView examine the mathematical basis for AM signals, both in the time domain and the frequency domain.  Using that knowledge we’ll design a simple peak detector to decode AM signals.  At the same time, we’ll be using HSPICE to investigate the difference between the two-quad and four-quad multipliers.  These multiplier chips are the modern day equivalent of the bulky mechanical contraptions that radio pioneers used to create AM signals.  What’s more amazing is that we’ll use those same chips to demodulate our signal, too!

At this point, all this might sound very disconnected from the FM circuit we’re supposed to be working on.  Lest you think that you’re falling behind some master schedule, we’ve actually chose a system architecture where the FM modulation and demodulation also requires the use of the same multiplier chips required for this lab (See Figure 1).

Figure 1: Lab 1 Roadmap

# Warm-up Exercises

## Units for Power Calculations

The spectrum and network analyzers we have in lab can display values in a number of different units – mV, mW, dBm, dBV, etc. – so it is important to know what these mean.

The decibel scale is convenient, because it allows us to express a quantity that varies over many orders of magnitude in a concise way.  Because the argument of a logarithm must be a unitless ratio, however, we must measure quantities like power and voltage relative to some reference value.  (Think – how is this different from computing a gain in dB?)  In communication systems, power is usually referenced to 1 mW and is measure in dBmW, or dBm.  1dBm is equal to 10logPower in mW.  For reference, 0dBm = 1mW, 10dBm = 10mW, 20dBm = 100mW, etc.  Similarly, you can define 1 dBV as 20logVolts.  When deriving voltage from power, be careful to take into account the impedance of the measured component.  The input or output impedance of most high-frequency equipment is 50Ω, so when you read a power value in dBm on the spectrum analyzer, this is the power into a 50Ω load.  The RMS voltage across this load will be Vrms=50⋅Power.  Note that if the node you measure has an impedance different from the load of your measurement equipment, you must equate the power read on the spectrum analyzer with the power (Vrms2/Z) into your load in order to find the actual voltage across your load.  Let’s do some exercises to make sure we’ve got things straight.

exercises:

- Calcualte the power in mW into a 50Ω load if the measured value is -15 dBm.  What voltage value does this correspond to?

- Calculate the power in dBm dissipated by a 75Ω resistor if 0.2V is measured across it.

- Suppose we have an amplifier with Zin = 70Ω and Zload = 40Ω.  If 1 mVrms is measured at the input, and 2 mVrms is measured across the load, what is the input power and the power delivered to the load in dBm?  What is the power gain of this circuit?  (Note that you can subtract the input power in dBm from the output power in dBm to obtain a gain in dB.)  This is known as the ‘operating’ gain of the circuit.  There are a number of other definitions of gain for a given circuit, which we will get into later.

- Assume you have an amplifier with a 500Ω load impedance.  Using a probe that does not load the output, you measure a power of -5 dBm at the output.  What is the voltage at this point?

- You have an amplifier with Zin = Zout = 50Ω and a power gain of 20dB when connected to a 50Ω load.  Your spectrum analyzer has Zin = 75Ω.  You apply 1 Vrms to the input.  What power will the spectrum analyzer read?

## High Frequency Components

We will be working with high-frequency signals this quarter, and though it won’t be crucial for this lab, it is important to be aware of the types of components that can be used at different frequencies.  Read the handout ‘Capacitors Demystified’ for a detailed description of different kinds of capacitors and their uses.  When in doubt, you can always check component values with the LCR meter in the lab.

# A First Look at Amplitude Modulation

If we use an RF signal of the form

vc=Vccosωct+φ	(  )

we can vary Vc, ωc, or  to create three distinct modulation schemes:

- Varying Vc gives us amplitude modulation (AM)

- Varying ωc gives us frequency modulation (FM)

- Varying  gives us phase modulation (PM)

Let us work through the equations that describe an AM signal.  To start, we take a carrier of the form:

vc=Vccosωct	(  )

with a baseband signal of the form:

vm=Vmcosωmt	(  )

When we multiply the two signals the result is:

vAM=VmVccosωmtcosωct	(  )

- Your first LabView VI: AM signals.  To visualize the AM signal, we’re going to create our first VI.  Start LabView and create a front panel similar to the following:

Figure 2: AM Modulation VI front panel

Next, create a block diagram that will display the time- and frequency- domain of the AM signal specified by the front panel controls, similar to that shown below.  The Simulate Signal express VI is found in the Input menu of the functions palette, and the Spectral Measurements express VI is under the Analysis menu.

Figure 3: AM modulation VI block diagram

When you are finished with the VI, turn the knobs to get a feel for how AM signals change with carrier and baseband frequencies and amplitudes.  What is happening in the time domain?  In the frequency domain?  Include a plot of one example in your lab notebook.

- Adding DC Offset: The VI you just made is an incomplete mathematical model.  We know that in amplitude modulation, a term at the carrier frequency is often present.  To model this, we introduce a DC offset in the modulating signal.

vm=VDC+Vmcosωmt	(  )

vAM=VDCVCcosωct+VmcosωmtVCcosωct	(  )

We now define a new parameter, the modulation index, m=VmVDC, which is a simple figure of merit used to describe quickly what the waveform looks like.  As long as the modulation index <=1, the modulating signal will never drop below zero, which means that the envelope of the modulated signal will be a faithful reproduction of the original.

Modify your AM modulation VI to include VDC and m.  Investigate the relationship between VDC and m.  What happens as you decrease VDC?  For m=1, what is the relationship between the energy in the sidebands and the energy in the carrier.  Include plots in your lab notebook to illustrate your observations.

# The Peak Detector

Figure 4: Peak Detector

In this section, you will be investigating the peak detector (see Figure 4) as an AM demodulator.  You will design a peak detector to demodulate an AM signal with fc = 30 kHz, fm = 1 kHz, and m = 0.5.  The input voltage looks as follows:

Vint=cos2π30 kHzt+14cos2π29 kHzt−π2+14cos2π31 kHzt+π2

- Designing our Low Pass Filter: Choose R and C values for your low-pass filter which pass the desired audio signal but filter out any higher frequencies.  Make sure your filter passes the entire audio band (up to 20 kHz), not just the 1 kHz tone!  You should have from class an expression for the optimal value of C as a function of Vin and R, which can assist you in your choice of C.  Please use standard component values for R and C, which are available on the web page at http://www.stanford.edu/class/ee133.

- HSPICE AC Analysis: Using HSPICE, obtain plots for the input and output voltages in the time domain as well as the frequency domain.  (A sample HSPICE deck, with critical values omitted, is available on the web page.)  Does the time domain waveform look as you expected?

- HSPICE Distortion Analysis: What is the Total Harmonic Distortion (THD) for your demodulator?  Replace R by a resistor that is 10 times too large and resimulate.  Indicate what type of distortion is present as well as the THD.  Now replace R by a resistor 10 times too small.  Again, indicate the type of distortion and THD you find.  For this case and for the rest of the quarter, include the input HSPICE deck and the output plots in your lab notebook.

Note: you can tell HSPICE to calculate THD by using the output command:

.four <fundamental freq.> <node>

These results can then be found in the *.lis file.

For your information, HSPICE calculates THD using the following equation:

THD%=100V22+V32+V42+…+Vn2Vfundamental	(  )

Where Vi is the ith harmonic which is an integer multiple of the fundamental frequency.  V1 is the signal of interest.  Keep this in mind, because some machines have quirks where they don’t always calculate it properly.

# The Two-Quadrant Multiplier

After all this modeling, you would probably guess that there is a circuit that performs this multiplication.  And you would be correct.  Figure 5 is one of the simplest multipliers: a variable transconductance, emitter-coupled pair multiplier.

Figure 5: Two-quadrant multiplier

This basic multiplier has a few limitations.  The circuit functions as a multiplier only under the assumptions that Vc is small compared to VT, and that Vm is greater and VBE,on.  This latter restriction means that the multiplier functions in only two quadrants of the Vc – Vm plane, and hence the circuit is called a two-quadrant multiplier.

- HSPICE Analysis: Simulate the current circuit in Figure 3 using the Spice deck provided on the EE133 web page.  Use Rc = 2.2 kΩ, Rb = 510Ω, RQ1a = RQ2a = 27 kΩ, and RQ1b = RQ2b = 15 kΩ.

- Additional HSPICE Simulation: Repeat your analysis for (a) Vc = 10 mV;  Vm = 1 V and (b) Vc = 2 V;  Vm = 1 V.  Include the output waveforms in your lab notebook.  Explain any differences between the results for (a) and (b).

- Figures of Merit: One figure of merit that is often used is conversion gain, which is the ratio of the usable output signal voltage to the input signal voltage.  Using the small-signal approximation, what is the conversion gain for this circuit?  Gconv=VsidebandVm.  We’ll go over additional figures of merit in the next section.

# The Four-Quadrant Multiplier

From your analysis of the two-quad multiplier, you might have noticed that the output voltage of the two-quadrant multiplier has a term which is a function of only the carrier frequency.  Arguably, the information you want is stored in the sidebands, and this carrier term is just taking up extra bandwidth.  In the search for ever more efficient transmitters, engineers have designed all sorts of circuits to eliminate the carrier (suppressed carrier, or SC modulation), and even one of the sidebands (single-sideband, suppressed carrier, or SSB-SC modulation).

In this class, we’ll just be exploring the elimination of the carrier.  (Eliminating a sideband requires a lot of additional circuitry.)  To accomplish this, we use a second emitter coupled pair cross-coupled with the first.  Figure 6 shows a schematic of a four-quadrant multiplier, the MC1496, with biasing circuitry.  It’s a relatively simple circuit, so we’ll use it for SPICE simulation.  In your real circuit, you’ll be using the SA612, a more complex, but definitely more effective chip.

First, think about suppressed carrier modulation.  If we eliminate the carrier, what does the signal look like in the time domain?

Now think about the circuit.  Note that the output is still taken just below the two Rc resistors.  Think of the bottom pair as a variable current source that slowly rocks current back and forth between the left and right upper pairs.  Carefully trace how Vc affects the outer and inner pairs of the top part of the circuit.  Think of this part as performing an inversion on the incoming current signal from the lower pair for half of every cycle of Vc.  These current changes are then converted into voltages by the resistors labeled Rc.  This circuit is known as the Gilbert Cell, after analog guru Barrie Gilbert.

Figure 6: Four-quadrant multiplier

Hopefully, this will make more sense with the following HSPICE simulations:

- Transient HSPICE Analysis: Using th MC1496 Spice deck, obtain the transient response for Vc = 2 V; Vm = 1 V.  Include the results in your lab notebook.  What is the conversion gain of this multiplier?  How does this compare to the conversion gain for the two-quad multiplier?

- Advantages of the Four-Quad Multiplier:  How does the output waveform differ from that of the two-quadrant multiplier?  Can you use a peak detector with this type of waveform?  Why or why not?

- Configuring the SA612: This multiplier has its own internal biasing circuitry, and all you will need to add is a few DC blocking and supply bypass capacitors.  Read the SA612 datasheet, which is available on the EE133 web site.  Find out how to configure it for single-ended operation where we will use only one side of the output.  Draw a schematic in your lab notebook.  We have transformers in the lab which can theoretically improve the conversion gain of your circuit by 3 dB, so you may consider using one of those in your circuit.

- Figures of Merit:  As you review the data sheet, record the following “Figures of Merit.”  These will prove useful to refer to as you experiment with the multiplier in lab, and when you later integrate it into your radio:

- Power Supply Voltage Range: Unfortunately, all chips are not designed to run off the same supply voltage.  What is the range for the SA612?  How do its characteristics vary over this range?

- Input Resistance (RIN): This is important for impedance matching and maximum power or signal transfer.

- Input Capacitance (CIN):  This defines the imaginary part of the input impedance that you might wish to take into account when designing your impedance matching network between the LNA and mixer.

- Output Resistance (ROUT):  This tells you what the source impedance is for your next stage.

- Conversion Gain (Gconv):  As we stated before, this is the useful output voltage compared to the input.

- Noise Figure (NF): This is the Signal-to-Noise Ratio (SNR) at the input port divided by the SNR at the output port.  This is a measure of how much noise has been added to your signal due to this circuit.  Each block of your receiver will add more noise to the system, but as we shall see later, this noise of successive blocks has less effect on the overall performance than the noise added by blocks closer to the input.

- Third Order Intercept Point (IIP3):  We will go into more detail about this later in the quarter, but in a nutshell, this is the input power level at which the third order harmonics (i.e. distortions) at the output are equal to the desired signal at the fundamental frequency.  You will want to operate at input powers a good distance below this point.

Now we an explore how the same multiplier chips can perform AM demodulation.

# AM Demodulation using Synchronous Detection

Though the simple envelope detector provides a decent demodulation of an AM signal, there are three major problems with the diode peak detector.

- It only works for signals that are less than 100% modulated.

- The non-linear components distort the output

- The system attenuates the output signal rather than providing gain.

The synchronous AM detector is amore complex circuit that can provide a larger, less distorted output, and will also allow you to demodulate a signal having an arbitrary modulation index.

The theory behind the synchronous detector assumes that we have an AM signal of the form:

vt=VDCcosωct+m2cosωc−ωmt+m2cosωc+ωmt	(  )

If we multiply this signal by the original carrier, we will generate the following:

vtcosωct=VDC2+m2cosωmt+12cos2ωct+m4cos2ωc−ωmt+m4cos2ωc+ωmt	(  )

The resulting signal contains a DC term, a baseband term, a term at twice the carrier, and terms at twice the carrier, plus and minus the baseband frequency.  A coupling capacitor can remove the DC term, and a low-pass filter eliminates the high frequency components, recover the modulated signal regardless of the incoming AM signal carrier amplitude (within limits, of course!)  The principal restriction on the demodulaing signal is that it must be of exactly the same frequency as the carrier signal that originally generated the AM.

- Mathematical Analysis of Synchronous Detection:  Why is our last restriction true?  Describe what happens when the demodulating signal is of a different frequency than the carrier.

- The Effect of Phase on a Signal:  Why does the demodulating signal need to be phase matched?  Give a mathematical explanation to justify your answer.

Since synchronous detection is essentially the multiplication of an AM waveform with a carrier, the circuit necessary for a synchronous detector is (surprise!) a four-quadrant multiplier.  To test the circuit in lab, you will use the same multiplier from the previous section, the SA612.

- LabView Analysis:  Write a VI that takes an AM signal and demodulates it by multiplying the signal with another sinusoid.  In your VI, the user should be able to set the frequency, phase, and DC offset of the demodulating signal.  You may (and should!) use your VI from Section 3 to produce the AM signal.  Investigate the effect of changing the demodulating frequency and phase.  Are there any restrictions on VDC?  Summarize your conclusions and include time and frequency domain plots of the input and output signals in your lab notebook.  Comment on the differences between this demodulator and the peak detector (distortion, expense, complexity, etc.)

# A Few Final Comments

We strongly recommend coming to office hours to solder up your multipliers.  That will leave you more time to perform your characterization.

Solder an SA612 to the transmitter side of the board before coming to lab.  On the transmitter side, the Voltage Controlled Oscillator (VCO) block will feed the RF input port with your slow moving modulating signal.  Since this is a low-frequency signal, use a large (22 μF) capacitor.  Electrolytics should be fine here.  Remember to use a high-frequency AC coupling cap for the LO port (the upper pair).  This one, as you can guess, should NOT be an electrolytic capacitor.

Once you’ve finished that, solder a second SA612 to the receiver side of your board.  Keep in mind that the amplifier in Lab 2 will eventually feed the RF port (the lower pair), a crystal oscillator (XO) (AKA the local oscillator or LO) will feed the upper pair.  Orient your SA612 accordingly.  Don’t forget your AC coupling and supply bypass capacitors!

Finally, read Lab 1 before coming to lab.  You will be testing your SA612 circuits, and the more familiar you are with the tasks that need to be performed, the more efficient you will be.  Furthermore, characterization often involves a fair amount of tedium.  We also highly recommend writing a LabView VI to automate the testing process.