EE133 – Lab 3

Frequency Modulation and the Colpitts Oscillator

# Introduction

In this lab, you will be characterizing the Colpitts oscillator you designed in the prelab, and using it to transmit.  At the end of this lab, you will compare the performance of this architecture with that from Lab 2, and decide which you will use for your final design.

# The LC Resonator

Figure 1: LC Resonator

After carefully reviewing Figure 1, solder your LC circuit to the transmit board.  Instead of connecting your LC tank to ground, connect it to Vdd, using bypass capacitors on the power supply lines.  Note that the power supply is an AC ground for high-frequency signals.

- Estimating Q with the 8591E and LabView:  In HSPICE, we were able to apply a current source input to out circuit in order to measure the impedance vs. frequency of our tank circuit.  We don’t have access to an AC current source in the lab, so we will have to improvise.

We will want to hook up the tap between inductors to the 8591E spectrum analyzer.  At this point, ask your TA for a solder-on BNC connector.  Solder the BNC connector to the board and connect it across the smaller of the two inductors.

Make an antenna out of a small straight wire and attach it to the BNC output of the high-frequency signal generator.  We will use this antenna to couple energy into the tank and measure Q.  Bring the antenna near the capacitor in your circuit (trying to keep it away from your output) and fix this setup so nothing moves.  Using the 8591E spectrum analyzer and the VI you wrote for the prelab, make a table of the output power (in dBm) vs. frequency.

- Estimate the Q of your circuit and compare this to your prelab results.

- Verify that the resonant frequency of your tank is approximately 24 MHz.

# The Colpitts Oscillator

Build the colpitts oscillator designed in the prelab.  Use a 30 pF capacitor in parallel with a variable capacitor (0.5 pF to 30 pF) to realize C1.  Remember, the combination of C1 and C2 along with the varactor will act as your tank capacitor.  You will be using the Toshiba 2SC3302 bipolar transistor which has and ft = 7 GHz.  Remember, the collector and base leads are opposite each other with the collector being the longer lead.  The other two leads are both connected to the emitter.

Note that the schematic in Figure 2 includes a potentiometer at the input to the varactor.  If you choose to use this Colpitts design as your transmitter, this input will come from your audio amplifier, which already includes the potentiometer and coupling capacitor.  Therefore, you should connect a wire from your varactor input to the output of your audio amplifier.  If you have not yet built the audio amplifier, you can just wire up a potentiometer in a position near where you will build it.  You will want to keep the audio amp output flexible (keep a moveable wire attached to it) so that you can use it to test your chip-based transmitter as well.  (Since the input to the varactor is in the audio range, you will need a large capacitor – around 22 μF.  An electrolytic cap wil work here; just be sure to get the polarity right!)

Figure 2: Colpitts Oscillator

## Characterizing the Colpitts

Now we will look at some of the functional characteristics of the Colpitts oscillator.

- Power Up:  For now, do not give an input to your circuit.  Simply connect the potentiometer in the correct place and bias the varactor to have 2.25 VDC across it.  (i.e. bias the varactor such that its DC voltage is about 6.75 V.)  Power up your oscillator and view the ouput on the 8591E.  If your oscillator doesn’t behave correctly, check to make sure that you have bypassed your power supplies very close to the circuit.  Also make sure your varactor (the reverse-biased diode) is still operational.

- Basic functionality:  Power up and tune your oscillator to some frequency around 24.3 MHz by adjusting the variable capacitor (C1).  When using the 8591E, make sure you set the frequency span to 1 MHz in order to get an accurate frequency measurement.  You can also use the bandwidth key (BW) to adjusts the resolution to smaller frequency increments.  Play around with changing the resolution and video bandwidths.

- Output Measurements:  What is the power output of your oscillator into 50Ω?  What is the output in volts?  What do you calculate to be the voltage at the top of the tank?  How does this compare with your prelab values?

- Power Output vs. Current:  Decrease the bias resistor you have chosen to be around 1 kΩ.  You can do this by placing resistors in parallel, if you don’t want to desolder.  What is the new output power into 50Ω?  What is the new tank voltage?  Does this agree with the expression given in the prelab?

- Frequency Characterization:  Tune the oscillator by adjusting the potentiometer.  Use the VI you wrote for Lab 2 to plot the f-V curve.  What are the upper and lower frequency bounds?  If the curve looks at all linear, what is the tuning constant, k, for your oscillator (in Hz/V)?  We eventually want a 50 kHz bandwidth on either side of the center frequency (total bandwidth of 100 kHz); what ΔV (relative to the varactor bias voltage) gives this ±50 kHz deviation?

## Modulation with the Colpitts

Now we will investigate how we can produce frequency modulation with the Colpitts.  We will be applying the modulating signal to the center of the potentiometer which biases the varactor.  In order not to load the tank with a low impedance, be sure to place a 10 kΩ resistor and a large coupling capacitor (about 22 μF) in series with the input.

- Modulation Bandwidth:  Apply a 1 V, 400 Hz modulating signal to your VCO input.  View the output on the spectrum analyzer.  Adjust the input voltage until you get a 100 kHz bandwidth.  What is this voltage?  (You will want to measure the voltage directly at the varactor, since the signal may be attenuated through the 10 kΩ resistor.)

- Demodulation on the 8591E:  Now you can use the demodulation feature of the 8591E spectrum analyzer to demodulate and listen to your signal.

  - Use the AUX CTRL key and turn the demodulation on.  Make sure that the demodulation type is FM, that your marker is on your center frequency, the frequency span is 1 MHz, and the speaker is on.  Do you hear the 400 Hz tone?  The volume knob is on the front, below the screen.

  - Using the same 400 Hz input to the VCO, view the output on the spectrum analyzer.  How does the output power compare with your prelab result?  What do you think is causing the discrepancy?  (Don’t worry if the difference is large.)

- Now for some fun:  To transmit with the Colpitts, we’re going to follow a procedure similar to that in lab 2.  Disconnect the probe that connects the 8591E and replace it with an 11’ wire.  This will act as your makeshift antenna.  Connect the other antenna to the spectrum analyzer and make sure your labmates’ oscillators are off at the moment.  Repeat the above step and listen to the 400 Hz tone.  Turn off the modulation for a moment and note the power into the spectrum analyzer.

- Now, take the antenna off your spectrum analyzer and attach it to one on the other side of the room.  With the modulation still off, what is the received power now?  (Record the distance and orientation of the antennae.)  Turn the modulation on and listen to the tone, and once again, revel in your RF coolness.

- Connect the output of the microphone circuit (if you built one) to the input of the Colpitts varactor.  Reset the center frequency of the spectrum analyzer to 24.3 MHz.  Speak into the microphone and listen to yourself on the analyzer.  How does the sound quality compare to the LM566/Multiplier topology?