EE133 – Prelab 3

Frequency Modulation and the Colpitts Oscillator

# Introduction

Last week, we studied the theory behind frequency modulation and explored one circuit to perform that modulation, the LM566 VCO.  This week, we’ll look at another type of oscillator, the Colpitts Oscillator.  This circuit is an LC oscillator that employs capacitive feedback to produce oscillation.  In EE133, we will be using a single bipolar transistor as our gain stage, but in general, any amplifier will do.

Figure 1: Roadmap for Labs 2 and 3

# The LC Resonator

Like most oscillators (one exception being the relaxation oscillators we studied last week,) the Colpitts oscillator requires a narrow band resonator.  Our resonator consists of a parallel LC filter, also known as an LC tank.  In this section, you will be designing an LC tank to resonate at 24 MHz.  Figure 2 shows the ideal and actual implementations for this circuit.  In the actual circuit, notice that L is replaced by a combination of L1 and L2, as well as the parasitic series resistance, Rs.

We would like our oscillator to drive a 50Ω load, but to do this, we need to tap the inductor so as not to load the output too much.  You can think of this as an impedance transformation of the load resistance to some higher equivalent resistance that appears at the top of the tank (for high Q, we wish the parallel resistance of the tank circuit to be high.)

Figure 2: LC Resonators: Ideal and actual implementations.

Ignoring the parasitic resistance Rs:

Req≈50ΩL1+L2L22	(  )

- Find C for 24.3 MHz Resonance: Using L1 = 1 μH and L2 = 0.1 μH, find the correct value for C to resonate at around 24.3 MHz.  Compute Rs from the Q values given for the inductors (see the web page for a list).  (Remember that Qseries = ωL/Rs for an inductor.)  To simplify the calculations, you may ignore the series resistance of the 0.1 μH inductor.

- Equivalent Tank Impedance: Using the impedance transformation methods discussed in class, what is the equivalent series resistance of the 50Ω impedance across L2?  You can now add this to the series resistance of L1 and transform the new series R to a parallel Rt.  What is this Rt, the resistance seen by the LC tank?  What is the Quality Factor of the tank (Qt) based on this value for Rt?  (Notice that we are assuming that the capacitors are ideal and do not severely degrade our value for Q.  This is not a perfect assumption, but will suffice for our use.  Try measuring Q of a silver-mica capacitor to be sure.)

- HSPICE AC Analysis:  Plot the AC frequency response with a current source input and a voltage output (both at the top of the LC Tank).  Remember to only use capacitive values that are available on the web page.  What value for C gets you closest to 24.3 MHz?  What is the 3 dB bandwidth of your LC tank?  What is the corresponding Qt?  How well do your hand calculations match the simulations (give percent error)? (The HSPICE input file is available on the web page.)

- LabView VI:  Write a LabView VI that will use the 8647A RF signal generator and the 8591E spectrum analyzer to measure the frequency response of the tank and calculate Q.  You will use this VI in the lab to characterize your LC tank.

# Colpitts Oscillator Theory

The Colpitts oscillator is the most famous of a family of LC oscillator circuits.  A useful block diagram representation of a general oscillator is shown in Figure 3.

Figure 3: Oscillator block diagram

This block diagram allows us to see how oscillation could possible occur.  Suppose we break the loop at point (x) and input a sinusoid at any amplitude.  This signal, Vin, would be multiplied by the gain (A) and the feedback factor (f) before coming back to point (x).  If this loop gain, Af, was exactly equal to unity, with its phase exactly zero, the sinusoid would repeat forever and we would sustain an oscillation.  This condition that Af = 1 with angle(Af) = 0 is the only possible condition for oscillation.  If Af > 1,  the amplitude of the sinusoid would grow exponentially with time.  If Af < 1, the amplitude would decay exponentially to zero.

The Colpitts oscillator translates to this block diagram very well.  Looking at Error: Reference source not found, we see a simplified version of the Colpitts that you’ll be building.  You might notice that the transistor’s base is grounded, hinting that we probably have a common-base amplifier.  Recalling that the small-signal gain for this amplifier is Av = gmRl (note that the gain is positive), it is easy to see how this could make up the gain block.  The feedback in this case is the capacitive divider formed by C1 and C2.  Recalling a bit about capacitive dividers, we remember that f = C1/(C1+C2).  In order to satisfy our oscillation condition, we must set A= 1/f, or gmRl = (C1 + C2)/C1.  Also, since the phase must be exactly zero, the oscillation can occur only at the resonant frequency of the LC tank, ω=1LC.

Figure 4: Basic Colpitts Oscillator

A few issues were overlooked in the preceding analysis, and we should take a moment to consider them.  First, you may have been wondering if the capacitive divider shifts the resonant frequency of the LC tank.  The answer is yes.  To account for this, we simply calculate the new tank capacitance to beCtotal=C+C1C2C1+C2.  Secondly, recalling that the impedance looking into the emitter is 1/gm, it will appear across the capacitive divider.  We can transform this impedance up so that it appears across the tank, just as we did with the tapped inductors.  After some math crunching, we find the equivalent resistance across the tank to be Req=1gmC1+C2C12.  We substitute this into our oscillation condition and obtain:

gmRt‖1gmC1+C2C12C1C1+C2=1	(  )

This can be rewritten in terms of conductance using n=C1C1+C2 andGtank=1Rtank.

gmnGtank+gmn2=1	(  )

And we now have a simple expression for the loop gain, Af.

One last major issue needs to be considered.  It is difficult to control this loop gain to be precisely unity.  Therefore, it would seem difficult to sustain a stable oscillation.  In this case, we don’t have to be this precise, because the oscillator corrects for our mistakes.  If we make Af > 1, we are sure that the amplitude of any sinusoid will rise exponentially with time.  Once the amplitude begins to rise to values on the order of 100 mV or greater, our assumption of small-signal operation is no longer valid.  What happens next is that the small-signal gm begins to get smaller with increasing amplitude.  Eventually, the new transconductance, called Gm, will decrease until the loop gain is exactly unity.  So we take advantage of this fact to have the oscillator amplitude limit itself.  (This phenomenon is referred to as the limit cycle.)  As Prof. Lee would say, Murphy never gives us a free lunch, but sometimes he does give us a light, nutritious snack to munch on.

We will finally try to estimate the amplitude of oscillation for this Colpitts Oscillator.  Because many people have looked at this problem before, we have a good approximation for what this large signal Gm may be.  As long as Vtank >> Vt,, then to a good approximation,Gm=2gmVtnVtank.  If we substitute this expression for gm in equation  (  ), we find the following expression for the amplitude of oscillation, Vtank:

Vtank=2gm1−nVtGtank	(  )

Since gm = IC/Vt, this expression reduces to:

Vtank=2Ic1−nRtank	(  )

# Analyzing the Discrete Component Colpitts Oscillator

Figure 5: Colpitts Oscillator (with biasing and varactor)

So now we’ll apply the theory we’ve just learned to design and simulate a discrete-component Colpitts Oscillator.  We will use the LC tank designed above for the resonator.  Note that the impedance of the next stange (the power amplifier) will be roughly 50Ω, so we will want to use the tapped inductor configuration to take our output.  Start with the following values for the components in Figure 5: C1 = 60 pF, C2 = 120 pF, R1 = R2 = 2.2 kΩ, and Rbias = 6.8 kΩ. L1 = 1 μH and L2 = 0.1 μH.  Use a bias current of IC = 0.5 mA, Vcc = 9 V, n = 1/3, and fosc = 24.3 MHz.  We will use Cbig = 1000 pF.

NOTE: Since any new additional capacitance will push our center frequency below 24.3 MHz, we will replace the C capacitor from Figure 4 with the series equivalent of C1 and C2 from Figure 5.  In order to tune the circuit, you may realized C1 as a fixed capacitor in parallel with a tunable 7.5-30 pF variable capacitor.

You might notice the funny looking symbol that appears to be a cross between a diode and a capacitor.  This is called a varactor, short for “variable capacitor.” Unlike physically tunable capacitors, the varactor is tuned by voltage.  Varactors are made from reverse-biased diodes, where the reverse bias modulates the depletion region of the PN junction and hence the junction capacitance.  This voltage-controlled capacitance allows us to use the Colpitts oscillator as a VCO.

Now we will characterize some features of the Colpitts oscillator:

- Tuning Range:  Look at the datasheet for the 2N3904 NPN transistor. What is the range of input capacitance, Cibo, for a  voltage range of 0-3V?  This is the capacitance value of interest, since we will be connecting the transistor as a diode.

- LC Tank Resonant Frequency: Ignoring the varactor capacitance for now, what is the resonant frequency of this circuit?  Include Cπ= 2 pF in your calculation.  How much frequency deviation would the varactor allow you to achieve?

- Small-Signal Gain:  What is the small-signal loop gain for this circuit?  Will the oscillations grow with time?

- Output Voltage:  Eventually, the amplitude will stop growing as Gm decreases.  For IC= 0.5 mA, what is the resulting Vtank?  If we tap the output inductors, what is the output amplitude as seen by the spectrum analyzer?  What is the output power into the 50Ω input of the spectrum analyzer (in Watts and dBm?)  (Hint: the power delivered to the effective resistance of the tank circuit is the same as the power delivered to the 50Ω spectrum analyzer input, since ideally the reactive components don’t dissipate any power!)

- HSPICE Analysis: Simulate this circuit using the HSPICE deck provided and the Toshiba 2SC3302 bipolar transistor.  Use the midrange value for the varactor capacitance.

  - How do your calculations compare with your HSPICE results?  Look at the output file to find the collector current.  Is this what you expect?  Why?  What is the expected Vtank for this current?

  - What is the total DC power consumption of this oscillator?

  - Separately plot the output voltage across the tank and the voltage across the 50Ω impedance.  Do the results make sense to you?

  - Now plot the collector current vs. time.  Why does the current have the shape it does?

  - Change the varactor capacitance to the maximum and minimum capacitance values – what variation in frequency can you obtain?

  - Now change the bias resistor to 1 kΩ and look at the voltage and current waveforms again.  Is the result what you expect?

In order to isolate the bias voltage on the varactor from the bias voltage on the collector of the transistor, we will need to put the varactor in series with a large silver-mica capacitor.  You will need to pick a capacitor large enough that it does not appreciably affect the value seen by the tank.

# Analyzing the Colpitts Oscillator on the SA612

Read the sections of the data sheet and application notes for the SA612 pertaining to the Colpitts Oscillator.  Using what you now know about oscillator circuits answer the following:

- Can you identify the different elements of the feedback system?  Which is the gain element?  What part(s) make up the feedback path?  Which component plays the role of the LC tank?

- Now look at how the crystal is situated in the circuit.  Drawing on your knowledge of crystal resonators, are you using the crystal in its series or parallel resonant mode?  A schematic model of a quartz crystal appears in Figure 6.

Figure 6: Equivalent Circuit for a Quartz Crystal

# Soldering Recommendations

- This is probably the most touchy circuit you will build all quarter.  Proper soldering technique and good bypass can mean the difference between smooth sailing and maddening frustration.  Use only as much solder as you need to make a proper solder joint, and make sure your joints are not cold.  (A proper joint should be shiny and not require much solder.  A cold joint, will appear dull and may require a lot of solder.)

- Bypass capacitors are also important.  Since this is an oscillator, the oscillating current through the circuit is particularly strong, and will cause noise on the supply line.  This can interfere with the circuit’s operation.  To keep the supply stable, be sure to bypass the supply close to the circuit.

# The Future and Some Design Choices

Now that you have multipliers and VCO’s under your belt, you have all the blocks necessary to make an FM transmission.  The only thing missing is extra amplification to improve range.  Before we do that, you have a decision to make.  We’re going to let you choose from two transmitter architectures.  Feel free to choose whichever suits your time and taste.

- The first is the chain that’s been outlined in the labs.  You will use the LM566 to produce the FM signal at 300 kHz, and the SA612 to upconvert the signal to 24.3 MHz.  This signal will then enter the power amp before transmission.  Most transmitters using this architecture will have 2-4 dBm of output power, but have high stability in the transmission frequency.  As you saw in lab 1, the mixer has an upper limit to the power of signal it can take before creating distortion.  In addition, the LM566 VCO has a square wave output, not a sinusoid, so there are some additional filtering issues with this approach.

- The second option is to use the Colpitts oscillator to modulate your audio signal directly up to 24.3 MHz.  As you can see from the prelab, one could apply a voltage directly across the varactor and create an FM signal around 24.3 MHz.  Unfortunately, since this signal is not based on a crystal oscillator, the carrier frequency can drift significantly, even within a single use of the transmitter.  The added complexity makes it a headache to solder and debug.  On the other hand, students have obtained 10-15 dBm of output with this approach – nearly a factor of 10 better than the alternative!

You may choose either method for your project.  Once you make your choice, remember to remove power from unnecessary circuits on your board.  (You don’t need to dismantle circuits you’ve already built.)