### Pre-Layout Checklist

- General Checks

- 2,4 or 6 layer quickturn designs with silkscreen

- Board height of 0.062"

- 23/24 style pre-defined holes

- 35 holes/square inch

- Copper weight: 1oz

PCB Properties

To set the following values:

- Select Options > PCB Properties

Now with the checklist, ensure that you have the following settings:

| Checklist | Tab | Section | Setting |
| --- | --- | --- | --- |
|  | Grids & Units | Grid Step Values | - Ultiboard Default: 15 or 20 mils. - Recommended: User Preference (1 or 5 mils) - For more precise placement of copper, parts and vias - consider setting each Grid Step type to 1 mil. |
|  | Copper Layers | Board Outline Clearance | - Min: 20 mils - Recommended: 25 mils (not UB default) |
|  | Copper Layers | Board Thickness | - 0.062” |
|  | Pads/Vias (Vias Section) | Via Drill / Pad Diameter | - Min Drill: 16 mils / Pad: 31 mils - Recommended: Use Min Settings (not UB default) - Max Drill: 20 mils / 40 mils |
|  | Design Rules (Trace Width Setting) | Trace Width | - Recommended: 8 or 10 mils (10 is UB default) - Max: 100 mils |
|  | Design Rules (Trace Width Setting) | Minimum Trace Width | - Min: 6 mils (not UB default) |

| Checklist | Tab | Section | Setting |
| --- | --- | --- | --- |
|  | Design Rules (Clearances Settings) | Clearance to Traces | - Min: 6 mils - Recommended: 10 mils (UB default) |
|  | Design Rules (Clearances Settings) | Clearance to Pads | - Min: 6 mils - Recommended: 10 mils (UB default) |
|  | Design Rules (Clearances Settings) | Clearance to Vias | - Min: 6 mils - Recommended: 10 mils (UB default) - See special notes about Inner Layer drill spacing below* |
|  | Design Rules (Clearances Settings) | Clearance to Copper Areas | - Min: 6 mils - Recommended: 12 mils (not UB default) - See special notes about Inner Layer drill spacing* |

| Other Setting Considerations |  |
| --- | --- |
| Hole Sizes | - Ultiboard: No default - Sunstone uses 23 hole styles from 0.16” to 0.251” for manufacturing (0.08” holes are available on 4, 6 layer services). Any holes will be mapped to the available default hole sizes other than specified to one of the available fixed sizes. Most through- holes will be ok, however if a precise hole is required consult the website and choose a specific hole size (or next size up) for the desired hole. |
| Inner Layer Clearances* | - Ultiboard: 10 mils default (12 mils recommend for inner planes) - Increase clearances on any ground or power planes on the inner layers if large holes over 0.100” are present. Individual planes can be set with a specific clearance setting. |

### Post Layout Checklist

- General Checks

Perform the following standard checks on your file (generally done prior to the export process):

| Checklist | Setting | Check |
| --- | --- | --- |
|  | Netlist and DRC | - Correct any issues found |
|  | Connectivity | - Complete any traces |
|  | Visual Checks | - Check net names - Check power (Verify IC connects to power source and ground) - Use net preview (in spreadsheet view) or print out individual layers |
|  | Trace Widths / Clearances | - Verify traced widths: 6 mil minimum - Verify trace clearances: 6 mil minimum - Verify plane clearances: 12 mil minimum or more for inner layers |
|  | Hole Checks | - Confirm holes min/max: 0.016” to 0.251” (0.008” on 4/6 layers) - Hole spacing should be 10 mils min. edge to edge - Hole density: 35 / square inch maximum |

Gerber File Generation

In Ultiboard you can export a number of different industry standard formats including Gerber files. Sunstone Circuits accepts the Gerber RS-274x standard. To export in Ultiboard; Select File > Export, Select RS-274x and click on the Properties button

Ensure that your export has the following properties:

| Checklist | Export | Setting |
| --- | --- | --- |
|  | Gerber Style | - Gerber RS-274X |
|  | Gerber Layers | - Select Board Outline, Copper Top, Copper Bottom and all Copper Inner layers. - Select Solder Mask Top and Solder Mask Bottom - Select Silkscreen Top (and Silkscreen Bottom if any parts on bottom) |
|  | Board Outline | - Merge Board Outline Layer and Copper Top Layer |
|  | Soldermask | - Check the Oversize Soldermask options for both Solder Mask Top and Solder Mask Bottom. Set each to 8 mils. |
|  | Export | - Click on Ok to set settings, and then choose Export to save all layers to multiple files |

NC Drill File Generation

To export in Ultiboard:

- Select File > Export

- Select NC Drill

- Click on the Properties button

| Checklist | Export | Setting |
| --- | --- | --- |
|  | NC Drill | - Verify imperical units as default, choose Export to save .drl and .rep files |

- Prepare Exported Data

| Checklist | Action |  |
| --- | --- | --- |
|  | Rename Files | Remove project name from the Gerber files. For example, “AmpProf – Copper Bottom.gbr” should be renamed to just “Copper Bottom.gbr” |
|  | Create zip File | Zip all .gbr, .drl and .rep files and rename zip file to the project name (or part number/revision number) |