ELEC 522, Advanced VLSI

Rice University

# Spring 2003

Homework 1

18 February 2003 - Due 27 February 2003

The purpose of this lab is to learn the use and design flow for the Xilinx ISE tools and also for the LabVIEW FPGA tools and hardware. Please print out the appropriate files, and due err on the side of printing more information than less…

Please use the same part as in the LabVIEW FPGA system:

xc2v1000-4-fg456   to build all projects.

1). Design a 1 bit adder using the Schematic capture capability in Xilinx ISE tools.

Please design this similar to the handout give on 1 bit adder.

For this Schematic project, please print out the appropriate Schematic, along with the Post Layout Timing Report in the Implementation Reports Folder. Use the FPGA Editor

under Xilinx->Accessories to view the blocks that are used. Please zoom in, locate the

logic blocks that are used, and select the LUT tables used. Please verify the equations

reflect the logic intended, and comment on any differences.

2). Design a 1 bit adder using VHDL. A sample VHDL file will be provided in the handout. Please implement with Xilinx ISE and use FPGA Editor to view, print, and verify. (In next homework, we will test and simulate with Modelsim, but not now…)

For the VHDL project, please print out the VHD along with the Post Layout Timing Report.

3) Using LabVIEW FPGA, verify that you can operate the Digital(Host) demo in the Getting Started/Programmatic example area under RVI. Please wire the external connector block, and print out the VI’s and a snapshot of the front panel. (This is basic.)

VLSI15 is functional, the I/O FPGA boards are labeled on the box.

4) Design a simple adder circuit for 16 bit numbers for the FPGA. Add a stop button. Make this controllable from the host in LabVIEW. Please build, run, and verify.

5) Add 16 bit digital output to problem 4. Observe the values on the external DIO

pins and record using a multimeter. (Next time we’ll use the logic analyzer.)