ELEC 522, Advanced VLSI

Rice University

Spring 2003

Homework 2

4 March 2003- Due 18 March 2003

The purpose of this lab is to learn the use and design flow for the State Machine  Editor and to use it to build an interface controller for a simple calculator. Refer to the “State CAD Tutorial” for Xilinx and the LabVIEW state machine tutorial.

Again, Please print out the appropriate State Editor, Schematic, and/or VHDL files. Also do a screen capture of the LabVIEW results, and print out the Xilinx ModelSim results.

1). Complete the Xilinx State Editor tutorial and run the LabVIEW “coke” state machine tutorial.

2). Build a serial to parallel converter and a parallel to serial converter. Use the State CAD editor to respond to LoadA, LoadB, Add, and UnLoad signals. Also, have a DataIn and DataOut signal. Design the loading of two 8-bit numbers, the addition of these numbers, and the unloading of the 9-bit result. You will also need a reset signal.

Synthesize and simulate with ModelSim.

3). Build a similar structure to problem 2 using LabVIEW and LabVIEW FPGA.

First provide the numbers from the host side for the serial data source. Design the state machine in LabVIEW. Simulate and verify. Pick 10 pairs of numbers to verify the

operation.