ELEC 522, Advanced VLSI

Rice University

Spring 2003

Homework 3

20 March 2003- Due 31 March 2003

The purpose of this lab is to learn use and integration of pattern generation and data acquisition to build a Logic Analyzer functionality with the the LabVIEW FPGA system. You may work in groups of two students to complete this project. I will also ask you to demo your projects in the lab on 1 April.

Refer to the attached notes on pattern generation for the National Instruments DIO-32 digital pattern generation board and translate to the DIO capabilities of the LabVIEW FPGA system.

1). Attached to board 15 and the MIO connector is a basic TTL 7400 series NAND gate. DIO9 is connected to pin 1, DIO10 to pin 2, and DIO11 to pin 3. Power and ground is already connected. Manually generate the truth table for the NAND gate and output the signals to the device under test. Acquire the results and display on the host screen in an analyzer window. In this problem, host and FPGA vi’s will need to be created. However, the data pattern can be fixed and the timing parameters can be fixed. The output screen

can also be fixed, but there should be a trigger type signal to start new data acquisition.

2). Add a variable time base feature to sweep from low frequencies in KHz to frequencies

in the MHz. Design in 8 frequency steps of your choosing.

3). Modify this to test the 74LS138 3 to 8 decoder chips in the VLSI lab. Make a simple assignment of the pins. More info to follow. Process an input file in IRSIM .cmd syntax.

Display the results based on labels derived from IRSIM.

4). Also implement the time-varying frequency capability.