Exercise 4.1 Creating a Custom Triggering Application

## Exercise Objective

This exercise will introduce you to using I/O in LabVIEW FPGA. This application will also show the flexibility of a LabVIEW FPGA system, creating analog input and output nodes and performing parallel processing for analog triggering.

## Exercise Summary

- Create an FPGA Application to read and write to Analog I/O and implement a custom trigger

Hardware Setup

Wire analog output channel 0 to analog input channel 0 of the R Series.

- Launch LabVIEW 8 and open an Empty Project.

- Click the Save All button  and save your project as Custom Trigger to the “Intro Demo” folder located on the desktop.  Your project should look like this:

- Right-click on My Computer and select New>>Targets and Devices. This will allow us to add an FPGA target to our project. Expand the FPGA Target folder, highlight the PCI-7831R device and select OK.

- [Optional] In the LabVIEW Project Window, right-click on the FPGA target (PCI-7831R) and select New>>R Series Expansion Chassis to add the expansion chassis to t he project.  A dialog window will pop up, choose Connector 1 as the connection for the expansion chassis and Click OK.

- [Optional] To add the C Series Modules, right click on the Expansion Chassis and select New>>C Series Modules.

- [Optional] After the dialog pops up, select to add an “Existing target or device” and highlight the C Series module.  You will get a warning dialog that states autodetection will stop the execution of the FPGA.  Click Continue.

By selecting “Existing target or device” LabVIEW will automatically scan the CompactRIO system and detect all modules present in the chassis. If you are working without hardware installed, you can still develop your LabVIEW FPGA code off-line by selecting “New target or device” and adding the I/O modules.

- [Optional] After you devices are detected, select all of them by clicking the module in slot one, then hold down the shift-key and click on the module in slot 4.  Click OK to add them to the project.

- Developing the LabVIEW FPGA Application
In this section you will:

- Design an application that uses parallel loops for allowing different rates for Analog Input (AI) and Analog Output (AO).

- Access I/O using FPGA I/O nodes.

- Use Single-cycle Timed Loops to optimize a triggering routine

- Trigger Digital Output (DO) off of Analog Input.

The completed application should look like this:

| Figure 10. Block Diagram | Figure 11. Front Panel |
| --- | --- |

- In the LabVIEW Project Window, right-click on the FPGA target (PCI-7831R) and select New>>VI to start a new LabVIEW FPGA VI (LabVIEW programs are called Virtual Instruments or VI’s).  Save this VI as Simple AIAO (FPGA) in the Filename text box to signify that this program will be run on the FPGA.

A common use for LabVIEW FPGA and RIO hardware is custom triggering.  Here we will read in data from an analog channel, compare it to a threshold, and write the appropriate value to a digital channel.  If the analog input value exceeds the threshold then an LED on the top of the Digital Output module (cRIO-9474) will turn on.  If the value is below the threshold then the LED will turn off.  We will begin building the application by reading in the analog channel at a timed interval.

- First, place a While Loop (Functions >> Structures) on the block diagram.

- From the Functions>> FPGA I/O palette place a FPGA I/O Node function inside the While Loop.

- Right-click the FPGA I/O node and select “Add New FPGA I/O.”

- Within the new FPGA I/O window, expand “Analog Input>>Connector0”, “Analog Output>>Connector0”, and “Digital Line Input and Output>>Connector0” to access all the necessary I/O for our application. Add AI0, DO0, and AO0 to the I/O node as shown below. Click OK to add the I/O to the project.

- Now resize the FPGA I/O Node by clicking and dragging the bottom of the node so that only AI0 is shown.

- Place a Loop Timer function (Functions >> Time & Dialog) inside the loop. Select μsec as the counter units and 32 Bit as the size of the internal counter.

- Right-click on the input terminal to the Loop Timer Function and create a control.  Label the control AI Scan Rate (uS).  This control will set the wait time which controls speed of the analog input.

- From the Functions>>Structures palette enclose the timing functions inside the loop in a Flat Sequence Structure.

- Right click on the sequence structure and add a frame after.  Expand this frame and place the Analog Input inside.

- Right click on the output terminal of the I/O node and create an indicator.  Label the indicator AI  Ch 0.  We will use the indicator to pass data back to the host.

- Wire a FALSE Boolean Constant to the While Loop Condition terminal. The VI should look like this:

Figure 15.  Analog Input Loop

Note:  On the first execution of the loop timer function, the internal timing value is set but no delay is added to the loop.  By placing the loop timing function in the first frame of the flat sequence structure, we ensure that the correct loop timing occurs on the first iteration of the loop.  If no sequence structure were used, the second acquisition would occur immediately after the first since no delay would be added.

- Place another While Loop (Functions >> Structures) on the block diagram.

- Place a Loop Timer function (Functions >> Time & Dialog) inside the loop. Select μsec as the counter units and 32 Bit as the size of the internal counter.

- Right-click on the input terminal to the Loop Timer Function and create a Control.  Label the control AO Scan Rate (uS).  This control will set the wait time which controls speed of the analog output.

- Drop down a FPGA I/O Node function (Functions >> FPGA I/O).  Left-click the I/O node and select Analog Output>>NI 9263>>AO0 to access the analog output channel 0.

- Right click on the input terminal of the I/O node and create a control.  Label the control “AO Ch 0”.

- Next, enclose the loop timer and the IO node in a flat sequence structure just as you did with the analog input loop to ensure that the loop timing function executes before the rest of the code.

- Wire a FALSE Boolean Constant to the Loop Condition terminal. Now the block diagram should look like this:

Figure 16.  Analog Output and Analog Input Loops

- Drop down another While Loop that will run parallel to the analog out and analog in loops.  This loop will be where we perform our custom triggering.

- Drag out a Timed Loop (Functions >> Structures>>Timed Structures) inside the while loop.  This is a single-cycle loop, which means that the code inside it will run in a single cycle of the onboard clock of the FPGA (40 MHz).  Basically we want to check every clock cycle to see if the input is above or below the threshold.  If so, we will set the Digital Output to the appropriate value.

- Create a Local Variable for the “AI Ch 0” indicator, place it inside the Timed Loop, and change it to read by right clicking on it.

- Drop a Greater? function (Functions >> Comparison) on the block diagram, wire the AI Ch 0 local variable to the top input and create a control labeled Threshold on the bottom input.

- Right click the border of the Timed Loop and select Add Shift Register.  This register will store the previous value of the threshold.  If the analog in value has risen above or dropped below the threshold value then we will stop the timed loop and update the digital output.  Connect the output of the Greater? function to the input of the shift register on the right border of the timed loop.

- Drop a Not Equal? function (Functions >> Comparison) on the block diagram inside the Timed Loop, wire the output of the Greater? function to the top terminal and the output of the shift register on the left hand side of the timed loop to the bottom terminal.  Wire the output of the Not Equal? function to the timed loop condition terminal.

- Right click the output of the shift register on the right border of the timed loop and create an indicator.  Name this indicator Over Threshold.

- Drop down a FPGA I/O Node (Functions >> FPGA Device I/O) inside the while loop (outside of the timed loop).  Left-click the I/O node and select Digital Line Output>>NI 9474>>DO0 to access the digital output channel 0.  (This is the output to the LED so that when the threshold is reached, the LED will turn on.)  Wire the output from the right shift register to the Digital Output node.

- Wire a FALSE Boolean Constant to the Loop Condition terminal of the outer while loop.  Because of the single cycle loop, it only takes 25 ns for the trigger to be recognized.  Now, the VI should look like below:

Figure 17.  Completed Block Diagram

- Since the user interface in an FPGA VI is only used for passing data, you may leave this front panel as is.  You should explain to the customer that in an FPGA application, an interface VI will be used to display all data.  Make sure, on the front panel, you enter in the values shown below and then select Edit >> Make current values default.  This will set it so the analog output loop runs at 100 kHz and the analog input loop runs at 1 kHz.  Also with the threshold set to 0 and the analog output set to 20,000 will cause the digital output to be high when the application starts.

Figure 18.  Completed Front Panel

- Save the VI and project.  Click the Run button to start the compile process.  Follow the steps through the compile process as was mentioned in the previous exercise.

- Click OK to close the build report.  The FPGA VI will begin to run automatically.  Adjust the analog output (AO 0) voltage and observe the reaction of the system.

- Click Stop Server to close the compile server.  After you have run and tested the code close the application and the Project Manager.

- When you are finished testing your FPGA application, close the application and close the Embedded Project Manager.