Exercise 4.2: Data Flow in Parallel Loops Using a FIFO VI

Objective: To create parallel loops on the FPGA that pass data using the FPGA FIFO.

Create a new project and a new VI and save. Name the project as FPGA FIFOs and the VI as Two Loops FIFOs.vi.  Follow the configuration steps as mentioned in previous exercises.

Block Diagram.

Your final block diagram should look like this:

- Create a For Loop with a constant 10 wired to the count terminal.

- Place 2 FIFO Write nodes from the Functions>>Advanced>>Synchronization>>FPGA FIFO palette inside the For Loop

- Left Click on the FIFO and select Add New FIFO.

- Fill in the Edit FPGA FIFO window that appears with the following:

  - Set Name to “Value”

  - Memory Type to Block Memory

  - Depth to 1028

  - Datatype to U32

  - Click OK

- Left click on the other FIFO and select  Add New FIFO…  Fill in the Edit FPGA FIFO window with the following:

  - Set Name to “FIFO Iterations”

  - Memory Type to Block Memory

  - Depth to 1024

  - Datatype to U32

  - Click OK

- Connect the For loop iteration terminal to the “FIFO Iterations” terminal of the FIFO Write node.

- Create a control on the “Value” terminal of the FIFO Write node.

- Create a constant value of -1 to the timeout of both FIFO Write nodes.

- Place a While Loop to the right of the For Loop

- Place a Wait Function inside the while loop.  Set the units to mSec.  Create a constant of 500 for the wait.

- Place a Case Structure inside the While Loop.

- Create a Boolean Control for the Case selector terminal.  Label the control “Write”

- Copy the 2 FIFO Write nodes and place the copies inside the Case structure.

- Create a control on the Value terminal of the FIFO Write node. Label the control “Value to Write”. Set the timeout to 0.

- Add a shift register to the While Loop.  Initialize the shift register with a value of 0. Right-click on the constant 0 and set its representation to I32.

- Wire the “FIFO Iterations” terminal to the shift register. Make sure the timeout is set to 0.

- Create code that will increment the value in the shift register by 1 when the “Write” Boolean is true. If the “write” Boolean is false the value in the shift register is just passed through unaltered.

- Create a control for the stop condition of the while loop. Change the mechanical action of the Boolean to a switching action.

- Create another while loop and place it below the previous while loop.

- Place a Wait Function inside the while loop.  Set the units to mSec.  Create a constant of 500 for the wait.

- Place a Read FIFO that will read from the FIFO Iterations FIFO. Define a 0 for timeout.  Pass that value to an indicator and label the indicator “FIFO Interations”.

- Create an indicator for the Empty terminal of the FIFO Iterations node.

- Create a case structure that when a Boolean Control labeled “Read” is true reads from the Value FIFO.  Pass this value to an indicator labeled Value Read. Set the timeout of the FIFO read to -1.

- Create a local variable for the stop Boolean and place it in the second loop to stop the second loop.

- Wire from the Full terminal of a FIFO node to the edge of the While loop to control the execution of the while loops.

27.  Save the project and VI.  Test the program in a Windows environment by switching the execution target to LabVIEW for Windows.  Once you have checked that it is working, go back to FPGA target and compile and run the VI.

End of Exercise 4.3