Exercise 5.1:  Create a FPGA Interface

## Hands-On Objective

To create a user interface for the VI created in exercise 4.1

Use the interface VI to communicate with the FPGA VI. You can run the host VI on an RT target or on a Windows PC. You will use the host to perform any data logging, integration with other hardware, or floating point calculations such as model-based simulation, control algorithms.  For a RT system, the host VI will be your time critical code.

- In the LabVIEW Project from exercise 4.1, create a new VI for the RT controller by right clicking on the controller and adding a new VI.

- Go to the block diagram and show the FPGA Interface palette.

- The FPGA Interface palette is a small set of six functions that will allow you to build a complete host interface to your custom device. You will use only four for this demo. Open FPGA VI Reference opens the reference to the VI targeted to the FPGA VI. Read/Write Control allows you to interact with the FPGA VI’s front panel controls and indicators, Invoke Method allows you to programmatically start and stop the FPGA execution among other things; you can demonstrate this functionality using other demos, and Close FPGA VI Reference closes out the reference. The last VI (we will not use it) is Call VI. This allows you to call the FPGA VI as if it were a subVI. Thus it would run once and return its outputs. For this demo, we would like both the FPGA VI and the host VI to run and interact continuously.

- Place the Open FPGA VI Reference function on the block diagram.  Use this function to open a reference to the VI you downloaded to the FPGA device. Every host VI must use the Open FPGA VI Reference function.

- Right-click the function and select Select Target VI from the shortcut menu. The Select Target VI dialog box appears.

- Select the FPGA VI you created in lesson 4.1 and click the OK button.

- Right-click the Open FPGA VI Reference function and select FPGA»device where device is the FPGA device you are using.

- Place a While Loop on the block diagram. Right-click the conditional terminal and select Create»Control from the shortcut menu to create a Stop button.

- Place the Close FPGA VI Reference function outside of the While Loop. Every host VI must use the Close FPGA VI Reference function.

- Place a Read/Write Control function inside the While Loop.

- Wire the HW Exec Ref Out output of the Open FPGA VI Reference function to the HW Exec Ref input of the Read/Write Control function.

- Wire the HW Exec Ref Out output of the Read/Write Control function to the HW Exec Ref input of the Close FPGA VI Reference function.

- Wire the error out output of the Open FPGA VI Reference function to the error in input of the Read/Write Control function.

- Wire the error out output of the Read/Write Control function to the error in input of the Close FPGA VI Reference function.

- Right-click on the Read/Write Control function and choose Add Control.  Repeat this process thrice more, so that there are 5 Unselected entries listed in the Read/Write Control function.

- Right-click on the top Unselected control and choose Scan Rate (uSec).  Repeat this for the remaining Unselected controls and change the names to Threshold, X, Y, and Threshold Reached, respectively.

- Wire each of the controls and indicators to their respective read/write terminals as shown below.

- Add software timing to the loop by placing a Wait Until Next ms Multiple function from the Time & Dialog pallete.  Create a constant value of 1 millisecond to time this real-time loop at a 1 kHz loop rate.

- If desired, navigate to the File >> VI Properties menu and set the Execution priority to time critical (highest).

Running the Complete Application

- Run the completed RT application. Adjust the analog output (AO 0) voltage and observe the reaction of the system. Notice that the update rate for the user interface is much faster than it was when running in LabVIEW FPGA interactive mode.

- If desired, navigate to the Embedded Project Manager and navigate to the Tools > Download VI or Attributes to Flash Memory menu. In an instant boot configuration, the FPGA application bitstream is automatically loaded from non-volatile memory on the chassis. You can configure the bitstream loader attributes so that the bitstream is loaded only at power-up (Autoload on Power-On Reset), or it can be set to load/reload upon any device reset (Autoload on Any Device Reset).