Montreal-based AV&R Vision and Robotics has integrated vision into manufacturing in a different way. Rather than developing a set of standard drivers, as GM is doing, its engineers used a common programming environment to integrate vision-based inspection into a robotic deburring cell. The systems integrator developed the process for an aircraft-engine manufacturer as part of a lean manufacturing project to automate the refinishing of jet-engine turbine airfoils during routine service.

After a grinding operation refinishes an airfoil, an operator loads it into the deburring cell. A six-axis robot manipulates it over a tool to remove any burrs from the root of the blade, break the edges, and regenerate their radii. Without putting the airfoil down, the robot then presents it to a Smart Camera vision system from National Instruments Corp. of Austin, Texas for inspection. As the camera looks for defects on the critical surfaces, the software classifies them according to their shape and records its findings with the airfoil’s serial number, which the camera also reads.

In the past, the process was a labor-intensive process because workers would deburr and inspect the radii and complex surfaces manually, using an assortment of deburring tools and hand gages. “We developed a cell that can automatically perform these two processes, ensuring that every part leaves the cell with the desired quality,” says Michael Muldoon at AV&R Vision and Robotics.

To automate those tasks, the engineering staff used National Instruments LabVIEW programming environment to develop the human-machine interface (HMI) and program the robot, the camera, and software conducting the classifications and reporting. “If you program each of those subsystems in a different programming environment, you have to spend a lot of time on communications and synchronizing the systems,” says Carlton at National Instruments. “Having everything in one environment just makes the integration a lot easier.” he says.

“It also makes it easier to inspect parts while you’re moving it or to use the camera as a feedback device to control the motion of the robot,” he adds. In some cases, the camera can replace the encoder and provide the feedback for motion control.