High Speed Control of Hydraulic Diecasting Machine Using NI CompactRIO and LabVIEW FPGA

### Authors: Paolo Catterina,  EUROelectronics srl – ITALY

Industry: Industrial Controls/Devices/Systems

Application Area: Control and Simulation

Campaign:

Product: NI CompactRIO, NI DAQ, NI NI LabVIEW FPGA, NI NI LabVIEW Real-Time, NI LabWindows/CVI

Challenge: Developing a closed loop control system for a hydraulic cylinder that is both cost-effective and highly reliable.

Solution: Using the high speed performance of the CompacRIO FPGA device to develop a robust system architecture that meets the rigorous requirements of the control system.

EUROelectronics is a machine builder asked to design the closed loop hydraulic cylinder control system for diecasting press machine. The high speed press moves at speeds varying from 0 to 10 meters per second, and so requires a high speed control system. To meet this challenge, we relied on National Instruments LabVIEW FPGA software and CompactRIO programmable automation controller (PAC) hardware. The FPGA enabled us to develop a system capable of low-level customization using a commercially available system. To meet the unique requirements of the application, we were able to implement a highly-optimized encoder interface in the FPGA to measure the cylinder position while programming the system entirely in LabVIEW .

Essentially, the system controls the cylinder position to track velocity and acceleration trajectory values entered by the user. The cylinder movement is controlled through a PID algorithm. The pressure sensor feedback is in the order of a few milliseconds which was fully matched by the processing speed of CompactRIO.

The motion profile and other parameters are shared via Ethernet with a PC running a complete supervisory software for the machine. This operator interface was implemented with NI LabWindows™/CVI software. This application also provides diagnostic and machine monitoring features by measuring motion quality and repeatability, performing industrial data acquisition measurements of the process variables (positions, speeds, pressures, temperatures) and reporting statistics for quality certification.

Position and pressure control of a hydraulic cylinder is a common application in the field of industrial automation,  but the precision control of such systems has traditionally presented significant challenges due to their high speeds and pressures.

In our application, the cylinder moves over a software defined trajectory with specific velocity and acceleration profiles which must guarantee accuracy and repeatability up to a maximum speed of 10 meters per second. For reliable control of cylinder braking and acceleration, the closing of the loop must be performed at 1 kHz processing rates.

The first thing to consider when choosing an acquisition systems is the quality of sensors required for pressure measurement and position. In this case, we used linear magnetic stripe sensors for position measurement. Sensors must guarantee precision and reliability measurements, and the signal processing interface for the sensor signals must be fast and robust.

The analog input modules available with CompactRIO have excellent accuracy and precision while the encoder function for cylinder position control has been fully implemented using just two high speed digital inputs. Due to the FPGA, the signals for encoding positions have been treated directly from the sensors so that no intermediate processing or amplification device is required. An evident reduction of noise was achieved and a correspondent increase in processing speed.

The cylinder movement must precisely follow the profile of position, speed and acceleration profiles predetermined by the supervisory software. In a process cycle faster than one millisecond the valve position is measured, the speed is calculated, both are compared to the setpoint, and the  movement is corrected using a PID algorithm acting. To keep the hydraulic circuit balanced, pressure values in the front and at the back of the cylinder are simultaneously controlled in order to avoid instantaneous peaks. The complete interface with the machine is constantly and perfectly held by CompactRIO real-time program, acting in this case as a perfect PLC.

The ability in this application to effectively close the loop over the hydraulic servo valve is possible only if the processing cycle time is absolutely deterministic. Likewise, the hydraulic circuit must respond quickly, precisely and repetitively. In this case, the  hydraulic servo valve was controlled through an analog output signal.

The precise tuning of the PID algorithm was obtained through the calculation of a linearization “table” of the response values for the valve which has unique non-linear behaviour. This PID gain scheduling method allowed us to obtain very accurate responses both at low velocities (from 0.05 to 0.30 meters per second during the start movement of the cylinder) and at high velocities (the real maximum speed reached is 7.5 meters per second).

Using “feed forward” and “smoothing” techniques on the command signal, we were also able to calibrate the PID algorithm so that in rapid commutation points (the positions at which cylinder must increase or decrease as quickly as possible) the risk for movement instability was solved.

Using the CompactRIO Ethernet port, the embedded LabVIEW system is able to communicate with the supervising application developed using LabWindows CVI. The supervisory software enables the operator to define the injection profile for the cylinder in two different ways: 1. by inputting numerical values or, 2.by drawing the profiles interactively using a graphical procedure. The interface also enables the operator to set various parameters required to perform the machine  cycle concerning position, velocity, pressure and time.

Using an NI PCI-6025E data acquisition card, we also added a number of diagnostic signals to the supervisory software such as the position, pressure and temperature profiles for each injection. The machine monitoring software provides plots of the machine operation and calculates various control values such as position of speed commutation, mean and peak velocities, times, pressures and temperatures.

Programming the CompactRIO with LabVIEW FPGA made it easy to implement the most critical parts of the motion control system with very high speed processing. This enabled us to implement command and feedback control without resorting to low level development tools, custom circuitry, or any other languages different from the graphical programming of LabVIEW. The performance of CompactRIO gave us the ability to encapsulate the system into a small, rugged device that is complete and robust. We were able to achieve significant improvements in process yield thanks to the advanced control capabilities that system enabled us to implement.

Project and development time required for this application was significantly reduced thanks to this integrated approach for homogeneous development using National Instruments software and hardware. With this project, we were able to go from the prototyping phase to the final setup of the machine in only three weeks.

For more information, contact:

Paolo Catterina, Software Development Engineer

EUROelectronics srl – ITALY

Tel: 39 030 2534521

Fax: 39 030 2533705

E-mail:   p.catterina@euroelectronics.it