# PXI-Based Audio Matrix Switch with Touch-Screen Control

By

Ken Lawton

Electronics Technician 2

Government of Alberta

Canada

Category:

Electronics/Electricity

NI Products Used:

LabVIEW 7.1

PXI-1010 combo-chassis

PXI-2501 FET Switch

PXI-6052E Multifunction DAQ

PXI-8186

SCXI-1127

The Challenge:

Finding a backup sound solution to the 12-year-old custom-built operator console for the digital audio reinforcement system for the Fall session of the Alberta Legislature

The Solution:

Implementing a complete backup solution using a PXI combo-chassis, a 21-inch LCD touch-screen, and five NI PXI-2501 FET modules to form an audio matrix switch.

Abstract:

Senates and Legislatures are a broadcast audio application with a large number of microphones controlled by an operator at a console. The state-of-the-art solution today typically involves digital audio technology incorporating digital signal processing. This paper describes an application using National Instruments hardware and software to produce an analog audio matrix switch controlled by a 21-inch LCD touch-screen console. The end result is a system for the lowest cost and considerably less complexity.

Introduction:

In 1992 the Government of Alberta invested $500,000 a state-of-the-art digital sound system for the Legislature. The Universal Digital Audio Processing System, or UDAPS, was the digital signal processor array that performed the sound system function. The installation included a custom-built operator’s console for selecting the microphones. 12 years later, this custom one-of-a-kind console failed. Though the console was repaired, the reality of a repeat failure substantiated a backup plan. TelAv was hired to setup and operate on standby a distributed conferencing system that cost $16,000 to rent for the remaining week of session.

Our project team was in the process of drafting the executive summary for the project to replace of the UDAPS at the time the failure occurred. Attempting to shorten the project by so many months would force the new design toward a modified conferencing system. Instead, a backup solution was proposed that would buy more project time and protect against potential UDAPS failure for the next fall and spring sessions. The final sound function for the UDAPS and this backup system would be the Alberta centennial celebration with the Royal visit of Queen Elizabeth II speaking to the Legislature in May 2005.  The National Instruments hot standby backup was built for less than the $50,000 budgeted, saving $100,000 overall, and potential Royal embarrassment.

Hardware:

The existing UDAPS audio system allows up to any four microphones active at one time.  Using the stock PXI-2501 FET boards configured as 2-wire quad 6 x 1 multiplexer to route audio worked perfectly providing only one input is active per bank. With two or more channels active at a time the output level was attenuated proportionally. This attenuation effect could be compensated for using an automatic gain control or other means, but there was another alternative.

PXI-2501 Block Diagram			Typical Summing Amplifier Circuit

If some simple changes are made, each PXI-2501 can switch 24 channels of line level audio without an attenuation of signal. An operation amplifier with a feedback resistor per side completes the mixer. Although the PXI-2501 has an op amp on board, for the best sound performance an Analog Devices’ OP275 was used in an external summing amplifier chassis.

Hot Standby Block Diagram

The complete computer control package includes a PXI combo chassis with the five FET boards. A PXI-6052E measurement card for fault and override inputs; an analog input and output are used for the volume control in the external summing amplifier.  In the SCXI portion of the chassis there are four SCXI-1127 modules each with 32 individual relays that are used for indicator LED’s on each desk. The processor is the PXI-8186 running XP; the 21-inch LCD touch-screen is by Stealth, SV-2100 with RS232 mouse emulation.  The LCD was located at the console operator position using an NTI Xtendex KVM with RS232 over CAT5.

Sound Reinforcement:

The existing Legislature member's desks each contain a microphone, speaker and LED. Obviously connecting to 100 plus speakers for hot swapping was unlikely. Instead four Frazier 1482 speakers would be set up in the public galleries for reinforcement in the event of a main system failure. Using only one speaker in each corner generated acceptable reinforcement without sound localization issues. The design worked perfectly for several functions when the UDAPS became unreliable.

LabVIEW Program:

The first step was to decode the protocol between the console and the UDAPS, then build a LabVIEW program to emulate the original console.  Once the protocol was determined then the final VI was created to emulate the UDAPS, the console, or both depending on the mode of failure of the existing system.  The concept allowed both systems to run in parallel providing hot standby for television broadcast and secondarily audio reinforcement after a short recess.

The virtual console buttons are read into an array of button values; on a change in console switch data the sequencer steps through each bit and carries out the correct response to a change in switch value. While the switch pattern is not changing the software tests the current FET and relay switches to confirm that only the correct points are active. Special functions such as Mr. Speaker's override, manual and automatic modes, etc. increased the complexity of the system considerably beyond the basic switching function.

Hot Standby Console Program - Sequencer Portion

Virtual Console Layout for Stealth 21-inch LCD Touch-Screen