Rugged Line Replacement Unit Field Testers based on NI CompactRIO

Category:

Aerospace/Defense

Author:

Russell Blake, G Systems, LP

NI Products Used:

cRIO 9004

cRIO 9104

NI 9401

NI 9205

LabVIEW Real-Time Module

LabVIEW FPGA Module

NI-RIO Version 2.0

The Challenge:

Building a rugged, yet portable, automated tester that soldiers can easily transport and deploy in harsh environmental conditions to ensure their line replaceable units (LRUs) are combat ready.

The Solution:

Create a compact and portable custom tester utilizing CompactRIO and the LabVIEW FPGA Module for high channel count, analog input, and digital I/O to control I2C and serial communications.

Abstract:

The field testing of military units is essential for success in combat. Automating this process reduces errors and time. This paper discusses the automation of a portable military tester.

Introduction

A defense contractor decided to redesign and automate its current test system, choosing G Systems, a National Instruments Alliance Partner, to work on the project because of a positive past experience.

The original system consisted of a software application and a separate hardware box with cables. The software application, or user interface, ran on a laptop instructing the user to perform the test procedure. The hardware box contained loads and relays, which could be connected to the units under test through the cables. During a typical test, the user interface instructed the soldier to manually switch relays and measure analog readings with a separate handheld digital multimeter, or perform a self test on the hardware box using shorting plugs. The soldier then had to document the test results.

The new system automates the hardware steps and automatically logs test results, reducing test time and improving test accuracy by eliminating error-prone manual tasks. The portable tester, designed by G Systems replaces and automates the previous tester. The hardware can be remotely accessed and controlled from the customer test software application running on the laptop by using a defined command set, which allows the tester to function like a test and measurement instrument.

Industrial and Rugged Hardware

When selecting the hardware an important component was the tester controller. National Instruments CompactRIO was chosen because of the ease and flexibility of programming along with environmental certifications.

G Systems’ automated tester operates in harsh and extreme conditions. The tester can be used worldwide with an AC power or a DC power source and complies with MIL-STD-1275B. The tester is also certified to handle shock and vibration defined by MIL-STD-810E.

Due to the complexity and number of cables, the tester includes full self-test capabilities with embedded self-test connectors for all cables. This reduces the need for shorting plugs that were previously used for the self test of a cable and provides better self-test coverage.

Software Overview and Architecture

The architecture is designed to be flexible and allow for remote communication and monitoring of the portable tester from the customer’s test application residing on the laptop.

The NI CompactRIO has an embedded LabVIEW Real-Time application, which runs automatically using the built-in LabVIEW Real-Time executable startup feature. It handles all of the tester logic, self-test procedures, logging of results, network communications to the laptop, RS232 serial communications, and interfacing with the LabVIEW FPGA application. The LabVIEW FPGA application controls all the analog input and digital I/O functionality.

To allow the customer’s test application to communicate to the portable tester, G Systems provided a DLL with a defined command set. The DLL handles all the network communications between the laptop and the CompactRIO.

Debugging and monitoring of the software and hardware was simplified using a reusable and efficient software component called the G Systems Event Tracker. This software component time stamps event messages to a millisecond resolution, logs event messages to disk, and displays live event messages. These messages can be viewed from G Systems Event Viewer application or incorporated into a custom application using G Systems Event Viewer control.

Custom Hardware Control

As described earlier, the previous system required the user to manually switch relays. Using National Instruments reconfigurable LabVIEW FPGA Module and a CompactRIO digital I/O module, G Systems developed a scalable, cost-effective solution for automating the process. The G Systems’ approach uses two digital inputs on the NI 9401 to generate the data and clock lines to control an I2C compatible 2-wire serial interface digital I/O chip. Since the data line from the chip is also connected to a digital input on the NI 9401, the software can poll the state of the digital I/O chip.

G Systems created a reusable driver using the LabVIEW FPGA Module to implement the I2C communications protocol. A modular chip-specific driver was developed using the LabVIEW Real-Time module and the LabVIEW FPGA Read/Write Control function with the LabVIEW FPGA application. The main LabVIEW Real-Time application uses the modular chip-specific driver to communicate with a custom PCB board, designed in-house, to control and monitor up to 56 relays.  However, by adding additional I2C chips, the hardware can be expanded to control up to 448 relays through the same 2-wire serial interface.

Conclusion

G Systems designed and built a rugged, portable automated tester with an embedded controller, resulting in improved test accuracy, repeatability, and time savings.

Figure 1: The rugged portable tester.

Figure 2: A high-level software diagram.