Distributed Event Monitoring System

Category:

Aerospace/Defense

Author:

Geeta Hegd and Justin Thomas

Captronic Systems Pvt. Ltd, Bangalore, India.

NI Products Used:

NI PXI-1000B 8 Slot DC Chassis

NI PXI-8187 Pentium 4-M Embedded Controller

NI PXI-6254 M Series DAQ Card

LabVIEW 8.0 Professional Development System

LabVIEW Real-Time Module

LabVIEW Internet Toolkit

LabVIEW Report Generation Toolkit

The Challenge:

Developing a reliable, cost-effective, portable data acquisition and event monitoring system that can distribute the computational load between multiple networked systems.

The Solution:

Using the National Instruments PXI platform, NI DAQmx, and the National Instruments LabVIEW Real-Time Module, we provided a high-performance, completely synchronized, and distributed network of data acquisition and measurement systems.

Introduction

The distributed measurement system consists of three data acquisition units that are separated from each other by more than 10 meters. These units are connected through an Ethernet link and the acquired data is processed and stored locally during tests. There are four display units to which data can be transmitted, both during tests and offline.

The main requirement for the system was to perform complete functional tests for distributed electronic assemblies. These functional tests include detecting and identifying commands produced by various command-generation systems in these electronic assemblies.

The measurement system was required to perform the following tasks:

- Detect and identify events through optically isolated digital input lines with a resolution of 100 milliseconds

- Detect and identify analog events through analog inputs with a resolution of 100 microseconds

- Maintain separate tables of detected analog and digital events for each acquisition system

- Transmit test data through a network to remote display units during tests and offline

- Merge test data from various acquisition systems and generate reports

We decided to build the test system based on the open industry standard PXI architecture. the range of National Instruments PXI products available provided us with the hardware for all of our signal types. NI LabVIEW is the language of choice for programming the system because of its excellent graphical capabilities, distributed system design features, and tight integration with NI PXI products using NI DAQmx.

System Configuration

The distributed event monitoring system consists of the following two sections:

- The remote data acquisition systems (RDAS) based on the PXI platform

- Data display systems based on laptop computers running Microsoft Windows

The RDAS communicates with display systems through an Ethernet connection. Each PXI chassis consists of two NI PXI-6254 12-bit resolution M Series DAQ cards. The 10 MHz clock routed through the PXI back plane is used to synchronize both of the cards. Each RDAS can monitor up to 96 isolated digital inputs and 64 analog inputs. The system provides a maximum sampling rate of 10 kHz per analog channel.

Depending on the complexity and size of the electronic assemblies to be tested, our measurement systems can work in one of the following modes:

- Stand-alone mode for testing a small subsystem with a single RDAS and display unit

- As part of a network to transfer data to any of the displays

Just as in a single-computer system, the components of a distributed system need to share data with each other in an easy and transparent way. The challenge was to implement a communication scheme that was best suited to link distributed components together.

System Implementation

New features and functions in LabVIEW 8.0, especially those related to data communications, made it very easy to develop the distributed measurement system. The application software developed is mainly divided into three modules – the display module, the real-time module, and the network module. The display module runs on all laptops and takes care of all user interface actions. The real-time module runs on all remote PXI systems. The network module runs on both and interacts with the other two modules.

The GUI in the display module consists of user-friendly software panels. The user has the option to configure the following:

- Determining digital and analog channels for event and pulse acquisition

- Giving legend names and order for displaying the events in the test panel

- Deciding pass/fail criteria by selecting event time windows, threshold, and range for pulse currents

- Setting sensor parameters such as sensitivity and offset

The tests are initiated by the display units. Tests can be started on remote systems simultaneously or one after the other. Synchronization is provided using a common hardware trigger line for all remote systems. The real-time systems continually monitor events for a duration of three to four hours. During the tests the systems perform the following functions:

- Analog event monitoring with 100 µs resolution

- Digital event monitoring with 100 ms resolution

- Analog and digital event comparison details with user-configured lookup tables

- Pass/fail criteria determination and generation of result tables

- Software updating mimic panel corresponding to 96 digital channels

- Event-based logging

- Battery voltage monitoring to alarm the user of any drastic voltage variations.

Some of the other features provided by the software include:

- Transfering configuration and test data files from display to real-time systems and vice versa

- Analyzing previous test results offline

- Merging files from different real-time systems.

- Generating quick reports both online and offline

- Setting security for administrator and user login

The distributed system developed is flexible since each individual component can be upgraded or modified without affecting the other parts of the system. Computing capacity and functionality can be added by upgrading hardware components or by adding computer nodes.

Conclusion

We were capable of designing and developing a test system that enabled our client to maximize the throughput and capabilities of the time-critical acquisition system. The LabVIEW development environment and the modularity extended by the PXI architecture and networking features equipped us to provide a flexible and reliable solution.

Figure 1:.An overview of the distributed measurement system.

Figure 2: A block diagram representing software implementation.