High-Speed In-Vehicle Data Acquisition System for ABS Testing

Category:

Automotive

Authors:

M. Arun Kumar, Sriram Iyer, J. Karthikeyan, and Senthil Raj Desappan, Apna Technologies & Solutions Pvt. Ltd.

NI Products Used:

LabVIEW 8.0

LabVIEW Real-Time Module

LabVIEW FPGA Module

DIAdem 10.0

cRIO-9004 – CompactRIO Controller

cRIO-9104 – CompactRIO Chassis

cRIO-9201 Analog Input Module

cRIO-9411 Digital Input Module

cRIO-9211 Thermocouple Input Module

cRIO-9237 Strain Gauge Input Module

The Challenge:

To develop a rugged in-vehicle data acquisition system that is suitable for antilock break system (ABS) testing.

The Solution:

To acquire signals from different sensors at a high sampling rate and to perform data acquisition, processing, logging, trending, and analysis by using the National Instruments CompactRIO platform and National Instruments LabVIEW software.

Introduction

Due to new government regulations, all vehicles in India should be fitted with ABS in a few years. This makes ABS testing all the more important, as it must ensure that the testing procedure complies with the standards and is capable of meeting the requirements of the new regulations. The vehicle fitted with an ABS will be tested under various conditions of load and various test tracks with different surface friction and braking at various speeds. The signals to be acquired during the testing include pressure, speed, load, steering angle, yaw, and temperature. Apart from the physical parameters measured, the system needs to calculate the distance traveled, speed, and deceleration of each wheel and vehicle by measuring either distance or speed.

During the test the vehicle is likely to undergo high deceleration and rapid change in the yaw. This requires the data acquisition system used for ABS testing to be rugged enough to sustain high vibration and shock; compact enough to be placed inside the vehicle; able to provide easy connectivity and have a locking mechanism for connecting sensors; dust, oil, and moisture proof; and to have appropriate sensor signal conditioning and sensor excitation built in.

ABS Testing

The basic functionality of the ABS unit in a brake system is to prevent the locking of wheels, allowing the driver greater control when braking. When the vehicle is stopped from a high speed on any road, the wheels tend to lock due to the transfer of vehicle load (inertia of motion) which causes skidding and poor braking. The ABS unit controls the pressure distributed to each of the wheels, and when one or more wheels locks, pressure to this wheel is cut out and distributed equally to the other wheels, avoiding locking of the wheels, and hence, skidding.

Another important parameter is the angle of steering. The IS 11852 standard has stringent rules about the maximum angle of steering during braking. This regulation ensures that even at high speeds, the vehicle does not swing, but maintains its line of motion until it comes to a standstill.

System Requirements:

The system requirements include:

- High-speed data logging of speed, load, pressure, and temperature during testing of the vehicle in different conditions of load and surface friction

- Ability to interface with the sensors and transmitters providing the necessary 15 VDC or 5 VDC excitation to the sensors

- Easy connectivity to the sensors through LEMO or D-type connectors.

- Ability to be powered and operated by a 12 VDC car battery

- Ability to interface to Corrsys-Datron speed sensor to measure vehicle speed, acceleration, deceleration, and stopping distance

- Ability to measure the vehicle speed from a noncontact optical speed sensor or a fifth wheel speed sensor

- Ability to measure the vehicle speed from a GPS device

- Ability to measure the speed of the vehicle as a function of analog frequency

- Ability to calculate the stopping distance, deceleration, and mean fully developed deceleration based on the wheel speed signal

- Configurable channels for calibration and for deriving additional pseudo channels from existing hardware and pseudo channels.

- Configurable display in the software for viewing the parameters during acquisition that can be saved and loaded when required

- User-friendly configuration options like drag-and-drop.

- Ability to see the test results and test summary in user-defined units that can be changed anytime

- Ability to have an online X-Y trending for any parameter on either axis

- Test configuration for configuring data logging speed; pre-log and post-log time; pre-trigger, start trigger, and stop triggers; and selecting which channels to log

- Ability to run more than one test at a time independently

- Modular hardware options to be adaptable to different test methods and future expansion for more channels or for adding vibration/sound

- High-end analysis module in the software to perform extensive research on test data taken during trials

- Features to export test data to National Instruments DIAdem and Microsoft Excel formats for analysis and to generate custom defined test reports

apnaVDAS Description

The National Instruments CompactRIO platform, National Instruments LabVIEW, and the National Instruments LabVIEW Real-Time and LabVIEW FPGA modules were used to create the apnaVDAS, vehicle data acquisition system to address the requirements of the ABS data logger application. The cRIO-9004 CompactRIO controller was used with the cRIO-9104, 8-slot chassis. The cRIO-9201 Analog Input Module, the cRIO-9411 Digital Input Module, the cRIO-9211 Thermocouple Input Module, and the cRIO-9237 Strain Input Module were used for data acquisition. The data is transferred to the laptop or hand-held display through a TCP/IP communication.

apnaVDAS Software:

The apnaVDAS software is a versatile data acquisition system software that caters to the vehicle testing applications and the stringent usage conditions of ABS . The apnaVDAS software consists of three parts:

- The FPGA software that runs on the chassis which is used for data acquisition, basic analysis, and filtering

- The real-time software that is used for streaming the data to storage, to set calibration factors, configuration, and for various analysis of data and data streaming to a laptop or hand-held display

- The application software that provides the user interface on the laptop

All the configuration modules in apnaVDAS can be accessed from one common screen called the control palette. The user can activate as many tests as required, each test independent of the others. Thus, a user can configure a test to continuously monitor the temperature at a low log rate and have another test to log the pressure and speed values at a higher log rate only when the brakes are applied.

ABS Testing:

The apnaVDAS software was configured to acquire the four wheel speeds, vehicle speeds, brake line pressures, brake chamber temperatures, pedal efforts, steering angles, and brake trigger inputs at a 1KS/s acquisition rate in order to evaluate the dynamic performance of the vehicle under different braking conditions. Data is continuously acquired on all channels which the user can view even when a test is not running. As apnaVDAS displays data, the configuration is dynamic and flexible, the user can open and close as many indicators as required and can use the drag-and-drop feature in the display configuration to visualize the parameter he or she wants by dropping onto the selected indicator. The configured display settings are saved and loaded when required.

Conclusion

With the help of the NI CompactRIO platform and LabVIEW 8.0, the apnaVDAS ABS data logger was built, meeting all the requirements of the vehicle test engineer. The ruggedness of the CompactRIO along with the rugged enclosure built by ATS, which offers easy connectivity, makes the logger a perfect fit for all vehicle applications. The apnaVDAS software developed in LabVIEW 8.0 provides all the features needed for vehicle testing in a simplified flow, which allows dynamic display configurations and multiple test handling, bringing together the world of measurement and an easy-to-use, flexible user interface.

Figure 1:  Main user interface screen of the apnaVDAS software

. Figure 6:  Scale configuration in the apnaVDAS software.