## Racecar Tyre Testing Apparatus

Category:

Automotive

### Author(s):

Russell Aldridge and B. Taylor Newill, Brigham Young University

### NI Products Used:

CompactRIO,

LabVIEW

DIAdem

### The Challenge:

Knowledge of tyre parameters is vital to racing success and purchasing tyre data from third-party vendors is extremely expensive. The Brigham Young University Formula Racing Team needed to build a robust, versatile, low cost data acquisition system for testing key tyre parameters under harsh testing conditions.

### The Solution:

BYU employed National Instruments CompactRIO hardware and National Instruments LabVIEW software to build a tyre testing machine that saved the team thousands of dollars and gave them a competitive edge at the competition.

Abstract:

How does a university SAE team build a tyre testing machine which can operate in real world conditions such as extreme temperatures, constant vibrations, moisture, and dust, and do it all for a price that makes a million dollar tyre testing machine blush? They do this by correctly applying National Instruments equipment. The tyre testing machine designed and built by the BYU team is towed behind a truck or SUV using the vehicle’s receiver hitch and trailer receptacle. This puts the challenge of powering the tyre in the hands of automotive engineers. The tyre is turned from left to right over a 30 degree range as dictated by a predefined program while strain gauges and infrared thermocouples measure imparted forces and resultant temperature gradients. The heart of the system is the NI Compact RIO, brought to life with NI LabVIEW software. This winning combination provides a robust, cost effective solution to a challenging problem.

Introduction

Tyre data is used to generate a comprehensive, mathematical tyre model. This model simulates the performance of a racecar tyre and is used to define vehicle geometry and predict performance characteristics. Design factors such as, suspension and steering geometry spring and damping ratios, location of the center of gravity, and tyre settings such as camber and caster are set using information from the tyre model. When track times for the top drivers are within 0.2 percent of each other, the importance of an accurate tyre tester and tyre model are readily apparent.

System Benefits

A key benefit of the tow-behind tyre tester is the ability to test tyres in real world conditions. Expensive tyre testing machines are permanently located inside a building and use a continuous belt to simulate road travel with a coefficient of friction that is extremely high. Ambient conditions such as temperature, solar radiation, and surface properties cannot be accounted for with this type of testing. The tow-behind tester is robust enough to gather data from real world conditions.

The tester was designed to be simple and robust enough to be used by the average college freshman and versatile enough to be towed by whatever vehicle he or she may own. The Web hosting feature of the CompactRIO Real-Time target makes this possible. Anyone with a laptop and a Web browser can run the tester by connecting via a wireless router to the CompactRIO. The wide input voltage range of the CompactRIO (11 to 30 V) allows the tester to be powered by any vehicle using a standard 7-pin trailer plug.

Each time the tester is connected to a new power source, it automatically recalibrates its measurement system. The configurability of the system allows the user to run a variety of testing programs or to generate their own testing cycle, thus enabling the system to make custom tyre models.

System Design

Automation, measurement, and data acquisition are controlled by the CompactRIO in the following manner. As the vehicle travels in a straight line, the slip angle is varied using a linear actuator controlled by a four channel relay module. Feedback for the closed loop system is achieved using a linear potentiometer.  Power for the potentiometer is sent via an analog output card (+/-10 VDC), with the voltage difference being measured using one channel of a four channel analog input card (+/-80mV). The output voltage from the potentiometer is calibrated by a timer which runs the tyre through its range of motion between two limit switches. Voltage at each limit is recorded and tyre angle is calculated using a LabVIEW MathScript Node which converts this voltage signal into a degree measurement.

Strain gauges measure the forces imparted by the tyre at different slip angles in lateral and longitudinal directions.  Excitation voltage is provided by an analog output card (+/-10 V) to two full Wheatstone bridges and one half bridge. Voltage differentials across the bridges are measured using a four channel analog input card (+/- 80 mV).

Also measured by a low-voltage analog input card are the voltage differentials created by the infrared thermocouples. These thermocouples actively record the temperature across the tread face and side wall. A GPS system transmits speed, distance, and direction to the CompactRIO via an RS-232 serial port communicating at 19,200 kbps.

The FPGA target was programmed with four parallel synchronous loops and 32 I/Os. The real-time target was similarly programmed and was responsible for the signal conditioning. On the real-time level the only aspect which had to be controlled deterministically was the data logging loop.  Deterministic data logging was necessary to visualize delta effects on key parameters. A timed while loop operating at 50 Hz was incorporated to achieve this objective. The generated data strings are concatenated with a GPS and a system time stamp and then written to flash memory, to be retrieved later via the CompactRIO FTP server. The data files are saved in CSV format and the data is analyzed and reported using DIAdem software.

A small 700 by 400 pixel tabbed GUI gives the user quick access to key parameters such as tyre temperature, force measurements, speed, and position. The user is also able to control which test programs are run, when the data logging begins, as well as the name of the data file. All of these systems work together to achieve a reliable tyre testing platform which accurately measures racecar tyre parameters.

Building a tyre tester similar to machines used by professional tyre testing companies on a Formula SAE budget would have been impossible. With the help of NI software and equipment, the BYU Formula Team created an automated tyre tester with enough money left over to build a racecar.

Figure 1: Completed Race Tyre Testing Machine.

Figure 2: Tyre Testing Machine Front Panel.

Figure 3: CompactRIO Integration.