Starter Generator Test System

Category:

Electrical/Electronics

Author:

Vijay Jayabalan and Anu Kalidas, Captronic Systems Pvt. Ltd.

NI Products Used:

cRIO-9002 Real-Time Controller

cRIO-9102, 8-Slot 1 M Gate Reconfigurable Chassis for CompactRIO

cRIO-9215, 4-Channel, 100kS/s 16-Bit Analog Input Module

cRIO-9263, 4-Channel ±10 V, 100 kS/s/Ch, 16-bit, Analog Output Module

cRIO-9425, 32-Channel 24 V, 10 µs, Sinking Digital Input Module

cRIO-9476, 32-Channel 24 V, 500 µs, Sourcing Digital Output Module

LabVIEW 8.0

LabVIEW Real-Time Module

LabVIEW FPGA Module

NI-RIO Version 2.0

NI Toolkit Software

The Challenge:

Developing a flexible, rugged, and cost-effective automated test system for testing the performance of the starter-generator at various load conditions.

The Solution:

Developing a flexible, rugged, cost-effective automated “starter-generator test system” using the National Instruments CompactRIO platform and National Instruments LabVIEW.

Introduction

Briefly, a starter-generator rotates the turbine engine fast enough so the fuel can be ignited and takes over as a generator providing DC power to the electrical system after the start of an aircraft.

There are two types of starts; shunt start and series start. The shunt type can usually be identified by having two large terminals and two small terminals located on the terminal block marked A, B, D, and E. For starting, terminals B and E supply current to the armature and stator windings. At the same time, voltage is applied to the shunt field terminals A and D from the generator control unit (GCU).

Three large terminals on the terminal block marked as C+, B and E- can identify series start type units. Shunt field terminals A and D are located on either side of the larger terminals. Terminals C+ and E- supply current to the armature and stator windings during the start cycle and the starter generator works the same as a DC electric motor. The shunt windings are not used in this mode. Following the starter operation, the GCU regulates shunt field current with voltage supplied from generator residual voltage and maintains a nominal system voltage of 28 V DC.

The starter-generator is tested in two different modes, namely generator mode and starter mode, to check its performance at various load conditions. Both the modes have a common control desk and data acquisition system (DAS) to control, monitor, and store the required parameters.

Generator Mode

In this mode the equipment used includes, an inverter duty AC motor, an AC drive, a step up gear box, a torque sensor, and a spline shaft and cooling blower. The AC motor and step up gear box were mounted on a mild steel-base plate. This entire setup was grouted onto a foundation/stress relieved mild steel structure. The gearbox output spline shaft is coupled to the generator.

In generator mode of operation, the generator is driven by an AC motor connected to a 1:10 step up gear box. The generator was flange mounted on to a hollow shaft torque sensor. The torque sensor was mounted on the step up gearbox. The AC Motor speed is varied through a closed-loop flux vector drive. The generator was loaded at various speeds in steps using the resistive load bank in steps and a momentary load for 0.3 seconds at the beginning of each step of loading.

Starter Mode

In this mode the equipment used includes, a powder dynamo meter, an adaptor plate, a mounting pad, and a spline shaft. The powder dynamo meter is mounted on a mild steel base plate. The starter motor is mounted on the reaction type hallow shaft torque sensor. The power supply to the starter motor is fed from a DC power supply of a maximum of 4000 A and 24 V. There is a separate field supply of 30 V, 20 A for the field coil excitation.

The starter-generator can be tested in both auto mode and manual mode. In auto mode, the CompactRIO data acquisition and control system takes over control of the entire test. In manual mode, the user can perform the entire test by performing appropriate operations in sequence through the inputs available in the control desk. The control desk is used for getting all the operational instructions. Test inputs such as generator speed, load, starter torque, etc, are fed through the PC. The control disk consists of potentiometers, selector switches, illuminated push buttons, and emergency stop and reset buttons for getting the operator instructions.

Auto Mode of Operation

Generator Mode

Auto mode of generator testing has user-programmable entries available for functions such as the speed of the generator, load-and-load duration, and generated voltage. The system establishes the test conditions including switching on the drive, blower motor, lube oil pump, etc. Once the initial condition is established, generator speed is set through flux vector drive by generating appropriate AO voltage. The system regulates the generator voltage through the field excitation and uses the digital PI controller for regulating the generator voltage.

The proportional integral controller is implemented in the FPGA. Nonlinear behavior of load Vs generated voltage enforces the need of piece-wise linearization of the system around the operating points. The system uses the gain scheduling at appropriate loads for generator voltage control. Once the generator attains the rated voltage, the generator is loaded through auxiliary contactors. The generator is loaded dynamically for a duration of 60 seconds with six dynamic loads; each consists of overload applications at the beginning of  each step for the duration of 300ms. The NI CompactRIO Real-Time Controller allows us to provide the time deterministic loading duration.

It is worth sharing how LabVIEW and CompactRIO helped us solve our problem in a timely and cost-effective manner. The generator mode of testing uses a 250kW regenerative vector drive for driving the AC motor, which generates EMC noise to few meters. Even properly shielded and grounded signal cables do not prevent the noise from getting coupled with signal cables. Electromagnetic Shielding of the entire drive is quite expensive; more over, it takes a lot of time for purchase and implementation.

Thanks to LabVIEW, the EMC noise problem has been solved by using the “median filter” in the software. The median filter provides the great advantage that it does not change the end-to-end transfer function of the system, which is very important for any dynamic testing.

Starter Mode

Starter mode – auto mode of testing performs the five starting operations. Starter is tested for a series of motor start operations. The system sets the required torque through a series communication to the power type dynamometer, and it sets the appropriate voltage to the remote power supply through the CompactRIO AO Module. The system establishes the required initial conditions such as water supply to the dynamometer and cooling air to the starter, before starting the actual test.

The user has the provision of aborting the test at any point of time by pushing the emergency stop button. The user can bring the systems to initial condition by pushing the reset button. The entire system handles three digital PID controllers, one for controlling the generator voltage, another for motor speed in the generator rig, and another for controlling the torque in the starter rig. Controllers are tuned for slightly lower than the critical damped response.

Manual Mode of Operation

In generator mode - manual mode, the operator performs all the operations through the selector switches and push buttons and through the potentiometers. Users can load the generator continuously in manual mode. The software reads all the analog and digital inputs and based on the user inputs (push button and selector switch actions) and interlocks, the software updates the digital and analog outputs. The sequence, as well as the operational interlocks, are implemented in the software. This implementation eliminates the need for external relay logics. Starter mode – manual mode is same as that of generator mode - manual mode.

Conclusion

We have developed a flexible, rugged, cost-effective automated “starter-generator test system” using CompactRIO and LabVIEW. The system based on LabVIEW provides advantages over the PLC based system in terms of analog signal processing as well as the graphical user interface.

Figure 1: Generator mode test rig.

Figure 2: Generator mode schematic diagram.

Figure 4: Starter mode schematic diagram.