PXI-Based Automated Wafer Probe Tester

Category:

Electrical

Author:

Andrew H. Kahn, G Systems, L.P.

NI Products Used:

PXI 1042

PXI 8185

PXI 4070

PXI 2532

LabVIEW™

NI Switch Executive

The Challenge:

Increase wafer probe test throughput and improve tester flexibility for use with various semiconductor sensors.

The Solution:

Design and build an automated production test system with high-speed switching that is configurable for multiple specification limits using National Instruments LabVIEW software and National Instruments Switch Executive.

Abstract:

G Systems developed an automatic test system to test pressure transducers at the wafer level. The tester is fully expandable and features complete DC parametric analysis including functional tests. The new system replaces an expensive 20-year-old Teradyne tester and has reduced test time by 50 percent.

The system tests voltage, current, resistance, and pressure for individual sensor die on a silicon substrate. There are currently five separate test points per sensor, and the tester is configured to allow up to 20 test points per sensor. Currently, 15 parameters are being tested per die.

Introduction

Increasing production throughput is a common challenge for any wafer probe operation. Using old equipment that is not readily expandable and configurable only complicates the problem.

In order to expand probe capacity and have a dynamic and configurable test system, a silicon sensor manufacturer contacted G Systems. G Systems designed an automated wafer probe tester that not only increased test flexibility but also reduced total test time by more than 50 percent.

NI-based Hardware

The G Systems wafer probe tester uses NI hardware to interface between a vintage Electroglas™ prober and four  high-speed, high-sensitivity source/monitor units (SMUs). The signals are routed through the high density NI PXI 2532 512-Crosspoint Matrix Switch Module configured in an 8 by 32 2-wire array. The PXI 4070 FlexDMM measures voltage and sensor leakage.

NI LabVIEW Software for Control and Operator Interface

The system is controlled with LabVIEW software to sequence parametric tests and update results to a database. Each sensor type has an executable test program and a unique configuration file. This software architecture made it simple to modify test parameters and adjust test limits without having to make changes in the code. The test software and files are downloaded on demand from a remote, secure file server. NI Switch Executive allowed for easy routing of all signals between the SMUs and the wafer through the prober.

The test software was developed with an operating mode for production testing as well as an engineering mode. In engineering mode, the software provides access to additional controls and test data. Furthermore, in engineering mode a technician can step through the individual tests one at a time and pause on request.

Conclusion

A highly configurable, high-speed PXI-based automated wafer probe tester was developed, which reduced test time by more than 50 percent. A LabVIEW software program provided a simple, operator friendly test solution that is configurable for multiple test limits and multiple sensor types. Overall, test times were reduced at significant cost savings.

Figure 1: G Systems’ automated wafer probe tester.