Distributed Manufacturing Test System for Vacuum Tubes using Programmable Automation Controllers and Modular Instruments

Category:

Manufacturing

Authors:

Lakshmana S. Kumar and Baskar Ceri, Apna Technologies & Solutions Pvt. Ltd.

NI Products Used:

PCI-6221 M Series DAQ Card,

PCI-5112 -100MHz Digitizer

FP 2000 LabVIEW Real-Time Controller

FP-AI-110 Analog Input Module for FieldPoint

FP-DI Dual Channel Discrete Digital Input Module,

FP-AI-V10B Dual Channel Analog Input Module

FP-DO-410 Discrete Digital Output Module

LEM sensors for High Voltage & Current

LabVIEW

LabVIEW Real-Time Module

LabVIEW Report Generation Toolkit

The Challenge:

Testing custom passive vacuum tubes across nine distributed individual test stations and sending the test results and data to the centralized database server for data analysis and future storage.

The Solution:

A totally flexible, user-friendly, and distributed test system was developed using National Instruments programmable automation controllers (PACs), modular instruments and data acquisition devices along with National Instruments LabVIEW software and the NI LabVIEW Real-Time Module. On each and every individual station the required tests are conducted using PAC or modular instruments. The obtained data from each and every station are also transferred to the centralized server through TCP/IP and FTP.

Introduction

Bharat Electronics, Ltd (BEL) is an electronics manufacturer who produces several pieces of electronic and telecommunication equipment. The Passive Vacuum Devices Division at BEL hass been manufacturing Vacuum Interrupter (V.I) tubes since 1985, seeking continual improvement on tube testing, data collection and analysis, and corrective and preventive action by automating the process. Current testing processes are manual and the project was envisaged to automate the tests distributed at nine stations by integrating the existing testing equipment with appropriate data acquisition and application software.

The distributed test system developed was based on NI LabVIEW, which involves data logging and analysis from nine distributed acquisition stations, out of which five PAC stations are independent real-time FieldPoint acquisition stations, and two are PC-Based data acquisition stations with modular instruments. At each station the unit under test (UUT) is tracked by barcode and the test parameters are logged and updated to the server database where each test is consolidated based on the stem number.

System Requirements

The basic requirements of the system should be:

- Interfacing with the current test equipment

- Performing accurate testing on the individual stations distributed on the manufacturing floor

- Acquiring test parameters and analyzing the data of each individual station

- Controlling the working operation of each and every station through a centralized server

- Consolidating all station test parameters based on stem numbers on the centralized server

- Producing reports based on user requirements

- Updating data on the server by just a “one click” option

Monitoring of production in a centralized manor\

System Description

There are four types of tests conducted on each V.I. on the nine stations distributed on the manufacturing floor. Some of the tests that consume more time have multiple stations. The test stations are:

- Contact Resistance (CR) – one station

- Aging Test – five stations – Aging MWB1, MWB2, MWB4, MWB3, JRC

- Impulse Test – two  stations – Impulse MWB3, JRC

- Vacuum Test – one station

Each V.I. is scanned for the barcode at each test station before the testing process and goes through all four tests in the stations based on the V.I. type.

Contact Resistance Station:

The contact resistance station is a data acquisition station based on FieldPoint that determines the resistance of the vacuum tubes by measuring the voltage. Each tube is identified by the stem number which is in the barcode format. The obtained voltage is converted into resistance by measuring currents up to 100 A. Data logging takes place after the user acknowledges by pressing the button on the panel. The contact resistance should be within specified limits for each V.I, in order to limit the heating of V.I tubes through ohmic current heat loss.

AC Aging and Power Frequency Withstand Test (PFWT):

AC Aging is based on both PC and FieldPoint data acquisition stations which determine the voltage and current of the vacuum tube. There are three types of tests conducted - chamber aging, contact aging and PFWT. The voltage and current obtained from the test station is very high and appropriate LEM current and voltage sensors are used before interfacing to FieldPoint and data acquisition hardware. The tube name and type is set based on the PFWT voltage comparison with the voltage limits on the setup file.

Impulse Test

The impulse test system is based on a modular instrument - the NI 5112 Digitizer - which captures the impulse waveform of the tube under test. The system captures both positive and negative impulse waveforms and the captured waveforms are analyzed and stored. The positive and negative peak of the impulse determines the tube characteristic. The application gives the flexibility to the user in the selection of impulse waves for analysis. The waveform is stored in an image file format, which helps for future analysis, and the measured parameters, such as positive peak and negative peak, are also stored in the file and later transferred to the server for consolidation.

Vacuum Test

Measurement of vacuum by ionization current method at various stages of the test process ensures the vacuum integrity of the V.I tube.

Server

The data from individual stations that are stored as files are collected at the centralized point based on the user configuration. The communication between clients and server is through TCP/IP and file transfer that happens through FTP. The server also performs other operations including, setup entry, individual station file view, manual data entry, and report generation.

Application Software and Modules

The application software was developed in LabVIEW and the LabVIEW Real-Time Module for PAC. The client and server run different application modules.

Test Station Application Modules

Authentication Module – This checks for the authorized user entry and gives user-wise features.

Bar code Module – This module reads the barcode (consisting of the stem numbers) from the bar code reader and is used before starting the test at each station.

Data acquisition Module – each and every individual test station has its own data acquisition module depending on the type of module used and has its own logging system.

User interface Module – each PC-based acquisition system has an interface to start the acquisition, control the process, and measure the required parameters.

Communication Module – establishes and maintains the communication between clients (both FieldPoint and PC) and the server.

File Transfer Module - this module transfers the individual files from the clients to the server based on user instructions.

Server Station Application Modules

Communication Module – This module establishes and maintains the communication between clients (both FiledPoint and PC) and the server.

Setup Module -- The setup module is the centralized user interface module that allows users to configure the test parameters according to the test needs. All test station parameters are entered under one tube type and saved for the future. The configured parameters for each test and tube type will be uploaded to the individual stations using TCP/IP and FTP communication.

Client Station View Module – Data which are transferred from each and every individual station can be viewed on the server.

Manual Data Entry Module – Manual data entry gives the flexible option for the user to enter the individual stations data or the measured parameters of the tube under test can be manually updated to the server through this option. Only administrator and super users can access this option.

Report Module - Data which are collected from all the stations are consolidated in the server. Reports can be generated based on stem number, date,  tube type or name, or PFWT voltage. The saved reports can also be previewed before printing. All the reports are generated using the LabVIEW Report Generation Toolkit and are saved in an Excel format.

System Features:

- User-friendly distributed test applications

- Appropriate interlocks on both hardware and  software control to avoid human errors

- Accurate measurement of voltage and current parameters

- User-defined test procedures and a secured environment over the test access

- One-click report generation with adequate details for future analysis and appropriate security

Conclusion:

The fact that National Instruments hardware works so easily in a networked environment and integrates very well with networked application software created in LabVIEW was a huge advantage while developing this distributed test system. Using LabVIEW, the LabVIEW Real-Time Module, and the NI data acquisition platform, we were able to implement a high-performance, flexible, and distributed test system that was otherwise fully manual.

Figure 1: Vacuum tubes waiting in line for testing.

Figure 2: Barcode scanning of a vacuum interrupter.

Figure 3: AC aging test station of a vacuum interrupter.

Figure 4: AC aging test of a instrumentation panel.

Figure 5: Test stations and server architecture.