Development of a Fully Autonomous Humanoid Robot for Novel Locomotion Research and as the First US Humanoid Entry to Robocup

Category:

Mechatronics

Authors:

Karl Muecke and Dr. Dennis Hong, RoMeLa (Robotics and Mechanism Lab) at Virginia Tech

NI Products Used:

LabVIEW  Real-Time Module

LabVIEW Vision Development Module

3D-Picture Control VIs

The Challenge:

Developing a robotic system to act as a research platform for studies on novel locomotion and to serve as the first United States entry into Robocup, a soccer competition among autonomous robots.

The Solution:

Using National Instruments LabVIEW to interface with third-party hardware to speed up development and testing for novel locomotion studies, and to create an artificial intelligence using National Instruments LabVIEW Vision and Real-Time modules, that enables the robot to perform high-level functions, such as playing soccer.

Abstract:

The Dynamic Anthropomorphic Robot with Intelligence (DARwIn), a humanoid robot, is a hardware platform used for studying bipedal gaits. Current technologies for robot programming and control are usually created in C code, have a steep learning curve, and even more challenging to interface with changing hardware. Technology from NI has been used to accelerate development and research in novel robotic locomotion. The result is a fully autonomous humanoid robot that plays soccer and acts as a research platform for studying novel locomotion.

Introduction

Not only does NI LabVIEW create expandable adaptable software that eases and accelerates research on novel locomotion, but it also serves as a brain to enable a robot to perform higher level tasks, such as playing competitive soccer. Bipedal locomotion and other forms of locomotion are major research topics at the Robotics and Mechanisms Lab (RoMeLa) at Virginia Tech.

Ideas and theories for locomotion research are tested on hardware.  Robocup, a soccer competition among autonomous robots, is a competitive and realistic arena to test and demonstrate the robustness of one of RoMeLa’s robots, DARwIn..  LabVIEW not only acts as an interface with the robots and as the brain of the robots, but also aids in verifying locomotion research.

LabVIEW as an Expandable Hardware Interface

Since there are many different types of robotic platforms at RoMeLa, a system was needed that could be easily configured for different hardware setups. Most research using small scale robots uses personal digital assistants (PDAs) to autonomously control the robots.  By using the LabVIEW Real-Time Module on a PC104+ computer, there is virtually no overhead and an expandable computer architecture that accommodates a range of different sensors that a PDA cannot; IEEE 1394 cameras, RS-485 communication, multiple wireless networks, and more, are all used simultaneously. Adding a new camera or an 802.11 port, adapting drivers and writing code in C or C++ would take much longer than dropping in a LabVIEW virtual instrument (VI) that takes care of everything.

Currently, LabVIEW controls the robot’s motion over RS-485 and can read joint positions on the same serial network from the servo motors’ built-in potentiometers. While the robot is walking or moving, a rate gyro with acceleration and orientation information communicates with LabVIEW over an RS-232 serial connection so that the program modifies the walking gait to effectively balance the robot in real time.

Initially, interfacing with the servo motors and rate gyro was all that was needed for the robotic research platform. However, taking part in the July 2007 Robocup competition required more sophisticated hardware and programming. Using the initial software developed, which enabled the robot to walk and balance itself, additional software was added to act as eyes, a brain, and communication. Since the robot must be completely autonomous and untethered during the Robocup competition, a Web host controls the stop/start signals of the robot.  All of the software on the robot runs under the LabVIEW Real-Time Module, which reduces overhead and frees up CPU time.

Verifying Locomotion Research

When generating mathematical formulations of robot locomotion, it can be difficult to visualize the results. Not only does LabVIEW allow for easy deployment of gaits generated in other computing software packages such as Mathematica or Microsoft Excel, but it also aids locomotion research by creating a graphic visualization of the motion of the robot. Using 3-D Picture Control, LabVIEW can simulate what the robot will look like when performing a generated gait.  This saves in development and research time since it can be laborious to setup and test gaits with physical hardware.

When ready to test robot locomotion on the physical hardware, the robots artificial intelligence can be bypassed and emulated by a user-controlled joystick. The user acts as the robot’s eyes and brain, while the joystick acts as an interface to control the robots motion by sending commands like walk, kick, dive, etc. With the LabVIEW joystick VI, it was very simple to implement this concept, which allows for easy debugging of gaits and motion without simultaneously debugging the robot’s behavior.

Creating Artificial Intelligence

With no prior exposure to any kind of vision processing and only two hours, one graduate student configured two IEEE 1394 cameras and wrote a VI that identified and physically located the relative position of the orange soccer ball used in Robocup. It is this kind of development time that allowed for DARwIn’s soccer playing behavior control to be created in just one week by one student, whereas other universities around the world accomplishing the same task spend years with many students working on the code. In just one week, one student wrote the VIs that qualified DARwIn as the first and only U.S. humanoid robot for the international soccer competition Robocup.

Figure 1: DARwIn kicking a Robocup soccer ball.

Figure 2: A simulation using a 3-D picture control.