2 by 2 MIMO Discrete Multitone Transceiver Testbed

Category:

Prototype/Test

Authors:

Alex Olson, Aditya Chopra, Yousof Mortazavi, Professor Brian Evans and Ian Wong

The University of Texas at Austin

NI Products Used:

LabVIEW

Modular Instruments

PXI

LabVIEW Real-Time Module

The Challenge:

Prototype the physical layer of a communications systems to rapidly evaluate system design tradeoffs.

The Solution:

Use the National Instruments PXI signal generation and digitizer hardware and the National Instruments LabVIEW Real-Time Module to provide hardware and software platforms for the physical layer of a real-time communications system.

Abstract:

The design & development of a communication systems is often facilitated by tests using a prototype that mimics the operation of the final product. Since development time on the final product is often lengthened due to resource constraints (such as processor or memory) ,we sought to create a flexible prototype that would allow rapid exploration of various design tradeoffs in a Discrete Multitone Transceive (DMT) modem.

Introduction

The physical layer of a communications system is responsible for converting data to an analog waveform that can be sent over a wire or other medium. To further our research, our goal was to construct a prototype of the physical layer of a 2 by 2 (2 transmitters, 2 receivers) high-speed "discrete multitone" transceiver (DMT) – for example, a discrete multitone modulation is used in DSL systems. This prototype allows us to quickly evaluate system design tradeoffs in different environments, while avoiding the time-consuming process of acquiring channel measurements or using rough theoretical models. NI hardware and software allowed us to circumvent the time-consuming and expensive process of designing and programming custom digital signal processing (DSP) boards.

System Design

DMTs process samples in blocks and make use of various signal processing algorithms, such as the Fast Fourier Transform (FFT). In terms of computation, each block of samples requires multiple FFTs and other significantly more complex operations – all using floating point arithmetic. Thus, the NI LabVIEW Real-Time Module proved an ideal solution. The LabVIEW Real-Time module provides high-speed, deterministic execution while natively supporting the NI analog I/O hardware.

Signal generation and high-speed digitizer hardware from NI also proved to be an excellent match. Our application requires the ability to generate and digitize analog waveforms at up to 2 million samples/s and 16-bits/sample, simultaneously on multiple channels. Furthermore, it requires that all channels maintain sample clock synchronization as well as all initiate sampling simultaneously.

The NI PXI Chassis provided the necessary high-speed backplane required to link the analog I/O boards with the high-speed real-time controller. In addition, the NI-TClk synchronization provided the means for all I/O boards to begin sampling at the same time. We used two NI PXI-5421 arbitrary waveform generators, two PXI-5122 Digitizers, and a PXI-8186 Real-Time Controller to accomplish our goal.

Although the NI PXI hardware and the LabVIEW Real-Time Module provided the hardware and software environment to support our modem, we also needed to visualize its performance during runtime. We found National Instruments LabVIEW to be an excellent match for our user-interface development needs. It allowed us to quickly construct a GUI for visualizing key performance data related to the operation of the modem. The shared variable feature of NI  LabVIEW allows us to dynamically configure the real-time system remotely from the GUI.

In short, we built a functional high-speed modem capable of transmitting more than 4 MB/s using a PXI system for real-time processing and a desktop computer for visualization. The LabVIEW Real-Time Module and the PXI hardware allowed us to avoid time-consuming embedded programming and analog circuit design. The only other hardware needed was an analog filter and a hybrid (two-wire-to-four–wire-interface) to complete the modem.